A. QUESTION BANK FOR THEORY BASIC MAINTENANCE CAT A

MODULE 1 - MATHEMATICS

Sub-Module 1.1: Arithmetic

- A car travels 24 miles in 45 minutes. What is its average speed?

- 32 mph.

- 36 mph.

- 18 mph.

Ref EASA Module 1 – Mathematics/ Arithmetic

- What is a scalene triangle?

- 2 sides unequal.

- All 3 sides unequal.

- 2 sides equal.

Ref EASA Module 1 – Mathematics/ Arithmetic

- Work out the following sum: 4 {2 (5-1) -3} + 8.

- 28

- 37

- 54

Ref EASA Module 1 – Mathematics/ Arithmetic

- A rectangle is 11cm x 120cm. What is its area in m2 ?

- 0.132.

- 1320.

- 13.2.

Ref EASA Module 1 – Mathematics/ Arithmetic

- 3/4 multiplied by 0.82 is equal to

- 1.23.

- 0.615.

- 2.46.

Ref EASA Module 1 – Mathematics/ Arithmetic

- The formula for calculating the area of a right angled triangle is

- ½ height + base.

- ½ (base * height).

- ½ base / height.

Ref EASA Module 1 – Mathematics/ Arithmetic

- To convert imperial gallons to liters, multiply by.

- 4.5.

- 5.4.

- 4.7.

Ref EASA Module 1 – Mathematics/ Arithmetic

- How many centimeters is in an inch?

A.  25.4.

B. 2.54.

C. 0.254.

Ref EASA Module 1 – Mathematics/ Arithmetic

- What torque loading would you apply to a nut if the force is 50 lbs., exerted 2 feet from its axis?

A. 600 lbs.ft.

B. 100 lbs.ft.

C. 251 lbs.ft.

Ref EASA Module 1 – Mathematics/ Arithmetic

- Convert 162 knots to MPH.

A. 186 mph.

B. 176 mph.

C. 196 mph.

Ref EASA Module 1 – Mathematics/ Arithmetic

- To convert inches to millimeters, it is necessary to.

A. Divide by 25.4.

B. Multiply by 25.4.

C. Multiply by 2.54.

Ref EASA Module 1 – Mathematics/ Arithmetic

Sub-Module 1.2: Algebra

- Solve the following equation: 5x = 3x + 2.

A. 3.

B. 5.

C. 1.

Ref EASA Module 1 – Mathematics/ Algebra

- Simplify the following (w + z)(x - y)(y - w) / (y - x)(w - y)(w + z).

A. -1.

B. 0.

C. 1.

Ref EASA Module 1 – Mathematics/ Algebra

- Given 43 - x = 21, find the value of x.

A. 43 - 21.

B. 43/21.

C. 43 + 21.

Ref EASA Module 1 – Mathematics/ Algebra

- Given that A = X + BY, what is Y equal to?

A. A - X add B.

B. A - X divided by B.

C. A - X minus B.

Ref EASA Module 1 – Mathematics/ Algebra

- (x - 3)(x + 5) =.

A. x2 + 2x.

B. x2 + 2x - 15.

C. x2 - 15.

Ref EASA Module 1 – Mathematics/ Algebra

- Evaluate. 2X2 Z2 (3X - Z2) =.

A. 6X2 Z2 + 3X - Z2.

B. 6X2 Z2 - 2X2 Z2.

C. 6X3 Z2 - 2X2 Z4.

Ref EASA Module 1 – Mathematics/ Algebra

- Factorize the following: 4x2 - 6x - 28 = 0.

A. (4x - 14) (x + 2).

B. (2x + 7) (x - 2).

C. (2x2 + 7) (x + 2).

Ref EASA Module 1 – Mathematics/ Algebra

- Make m the subject of the formula in y = mx + c. Find m.

A. m = y - x/c.

B. m = (y – c)/x.

C. m = (y + c)/x.

Ref EASA Module 1 – Mathematics/ Algebra

- If x2 – 3 = 6, then x = ?.

A. ±3.

B. 18.

C. ± 2.

Ref EASA Module 1 – Mathematics/ Algebra

Sub-Module 1.3: Geometry

- In the following equation what is the y-intercept? 4y = 2x + 8

A. 2.

B. 4.

C. 8.

Ref EASA Module 1 – Mathematics/ Geometry

- How many times does the x-axis get crossed when y = x2 -3.

A. 3.

B. 1.

C. 2.

Ref EASA Module 1 – Mathematics/ Geometry

- Two lines with equations y = 3x-6 and y = 3x + 4?

A. meet when y = 3.

B. are at right angles.

C. are parallel.

Ref EASA Module 1 – Mathematics/ Geometry

- A straight line graph has the equation 3y = 12x–3. What is the gradient?

A. 1/4.

B. 4/1.

C. 3/4.

Ref EASA Module 1 – Mathematics/ Geometry

- What type of equation is y = x2 + 9x + 14?

A. Quadratic.

B. Circular.

C. Exponential.

Ref EASA Module 1 – Mathematics/ Geometry

- What is commonly referred to as the law of a straight line?

A. y = x2 + 180.

B. The line must pass through the 180-degree datum.

C. y = mx + c.

Ref EASA Module 1 – Mathematics/ Geometry

- What is the equation of a straight line with gradient m and intercept on the y axis c?

A. y = mx + c.

B. x = y + mc.

C. y = cx + m.

Ref EASA Module 1 – Mathematics/ Geometry

- What is the equation of the line shown?

A. y = 2x + 2.

B. y = -2-x.

C. y = x-2.

Ref EASA Module 1 – Mathematics/ Geometry

- What is the equation of a straight line that passes through the two points (0,0) and (3,2)?

A. y = 3x + 2.

B. y = 2x + 3.

C. y =2x/3

Ref EASA Module 1 – Mathematics/ Geometry

- The line with equation x = 3 is.

A. at 45° to both axes.

B. parallel to the x axis.

C. parallel to the y axis.

Ref EASA Module 1 – Mathematics/ Geometry

MODULE 2 - PHYSICS

Sub-Module 2.1: Matter

1. What is the mass number of an element?

A. Number of electrons and protons.

B. Number of neutrons and protons.

C. Number of electrons and neutrons.

Ref EASA Module 2 – Physics/ Matter

2. What is the atomic number of an element?

A. The number of neutrons in the atom.

B. The number of electrons in the atom.

C. The number of protons in the atom.

Ref EASA Module 2 – Physics/ Matter

3. What determines an element's identity?

A. The number of neutrons.

B. The number of electrons.

C. The number of protons.

Ref EASA Module 2 – Physics/ Matter

4. If an atom has 2 protons, how many electrons will it have?

A. 1

B. 8

C.2

Ref EASA Module 2 – Physics/ Matter

5. The number of protons in an atom is equal to the number of.

A. electrons.

B. neutrons.

C. mass number

Ref EASA Module 2 – Physics/ Matter

6. An atom with 3 free electrons in its outer shell is said to be.

A. pentavalent.

B. covalent.

C. trivalent.

Ref EASA Module 2 – Physics/ Matter

7. The splitting of a chemical compound is called.

A. bombardment.

B. synthesis.

C. analysis.

Ref EASA Module 2 – Physics/ Matter

8. An atom consists of.

A. protons, neutron and electrons.

B. protons and electrons.

C. protons and neutrons.

Ref EASA Module 2 – Physics/ Matter

9. The maximum number of electrons in the inner shell of an atom is.

A. 8.

B. 2.

C. 4.

Ref EASA Module 2 – Physics/ Matter

10. The mass of a proton equals the mass of a.

A. neutron.

B. beta particle.

C. electron.

Ref EASA Module 2 – Physics/ Matter

11. Catalysts.

A. speed up reactions.

B. speed up and slow down reactions.

C. slow down reactions.

Ref EASA Module 2 – Physics/ Matter

12. If the outer shell of an atom is full the the element is.

A. unstable / reactive.

B. a good conductor.

C. practically inert.

Ref EASA Module 2 – Physics/ Matter

13. Oxygen has a valency of.

A. 2.

B. 6.

C. 8.

Ref EASA Module 2 – Physics/ Matter

14. An isotope of an element will have more/less _________ than usual.

A. electrons.

B. neutrons.

C. protons.

Ref EASA Module 2 – Physics/ Matter

15. The smallest part of an element is.

A. a compound.

B. an atom.

C. a molecule.

Ref EASA Module 2 – Physics/ Matter

Sub-Module 2.2: Mechanics

1. For a car weighing 1000 N, what force would be required to accelerate the car to 3 ms-2.

A. 3270 N.

B. 305.8 N.

C. 3000 N.

Ref EASA Module 2 – Physics/ Mechanics

2. The SI unit of acceleration is the.

A. metre per second squared (m/s2).

B. metre per second (m/s).

C. square metre (m2).

Ref EASA Module 2 – Physics/ Mechanics

3. If a body starts at rest and reaches 84 m/s in 3 seconds, its acceleration is.

A. 28 m/s2.

B. 14 m/s2.

C. 252 m/s2.

Ref EASA Module 2 – Physics/ Mechanics

4. The SI unit of velocity is the.

A. metre per second squared (m/s2).

B. metre per second (m/s).

C. metre (m).

Ref EASA Module 2 – Physics/ Mechanics

5. A spring extends 7 cm to the ground and back again 20 times a minute. What is the period?

A. 7 cm.

B. 3 seconds.

C. 1/3 cycle.

Ref EASA Module 2 – Physics/ Mechanics

6. What is the angular velocity of a shaft rotating at 300rpm in radians/second?

A. 10π radians/second.

B. 2π radians/second.

C. 5π radians/second.

Ref EASA Module 2 – Physics/ Mechanics

7. A mass on a spring nearly hits the ground 20 times a minute, its frequency is.

A. 3 cycles per second.

B. 1/3 cycle per second.

C. 20 cycles per second.

Ref EASA Module 2 – Physics/ Mechanics

8. An object travels 18 km in 1 minute 30 seconds. What is its average speed?

A. 200 m/s

B. 270 m/s

C. 100 ms.

Ref EASA Module 2 – Physics/ Mechanics

9. A spaceship travels a distance of 480,000 miles in 2 days. What is its speed?

A. 48,000 mph

B. 36,000 mph

C. 10,000 mph

Ref EASA Module 2 – Physics/ Mechanics

10. Newton’s laws of motion apply to.

A. solid substances only.

B. all substances irrespective of state.

C. gases and liquids only.

Ref EASA Module 2 – Physics/ Mechanics

Sub-Module 2.3: Thermodynamics

1. Convert 25 degrees centigrade to fahrenheit.

A. 57

B. 77

C. -7

Ref EASA Module 2 – Physics/ Thermodynamics

2. When gases change temperature/pressure, the scale used is.

A. Kelvin.

B. degrees Fahrenheit.

C. degrees Centigrade.

Ref EASA Module 2 – Physics/ Thermodynamics

3. One degree increment on the Centigrade scale is equal to what increment on the Fahrenheit scale?

A. 1.8 °F

B. 33 °F

C. 12 °F

Ref EASA Module 2 – Physics/ Thermodynamics

4. What is 1 BTU/lb in joules per kilogram?

A. 4128 joules per kilogram.

B. 9.18 joules per kilogram.

C. 2326 joules per kilogram.

Ref EASA Module 2 – Physics/ Thermodynamics

5. What is -28 °C on the Kelvin scale?

A. 245 K.

B. 18 K.

C. 291 K.

Ref EASA Module 2 – Physics/ Thermodynamics

6. Which type of circuit is used when the temperature of the source has insufficient heat for thermocouple application?

A. thermistor.

B. temperature bulb.

C. balanced bridge

Ref EASA Module 2 – Physics/ Thermodynamics

7. The absolute temperature scale that has the same increments as the Fahrenheit scale is the.

A. Rankin scale.

B. Kelvin scale.

C. Celsius scale.

Ref EASA Module 2 – Physics/ Thermodynamics

8. The absolute temperature scale that has the same increments as the Celsius scale is the.

A. Rankin scale.

B. Kelvin scale.

C. Fahrenheit scale.

Ref EASA Module 2 – Physics/ Thermodynamics

9. You are at the north pole where the temperature is minus 50 °C below freezing. What sort of thermometer would you use to measure it?.

A. Water.

B. Alcohol.

C. Mercury.

Ref EASA Module 2 – Physics/ Thermodynamics

10. Convert 15 °C to °F.

A. 37 °F.

B. 59 °F.

C. 62 °F.

Ref EASA Module 2 – Physics/ Thermodynamics

11. 1 calorie is equal to.

A. 4 J.

B. 40 J.

C. 400 J.

Ref EASA Module 2 – Physics/ Thermodynamics

12. 1 BTU equals.

A. 1055 Joules.

B. 1055 Calorie.

C. 1055 kJ.

Ref EASA Module 2 – Physics/ Thermodynamics

13. Temperature is a measure of.

A. the amount of friction produced by two mating surfaces.

B. the amount of vibration of molecules.

C. the heat energy of particles.

Ref EASA Module 2 – Physics/ Thermodynamics

14. Heat is described as.

A. a high temperature.

B. energy in transit.

C. the stored energy in a hot body.

Ref EASA Module 2 – Physics/ Thermodynamics

15. What temperature scale is used with the combined gas law?

A. Absolute.

B. Fahrenheit.

C. Celsius.

Ref EASA Module 2 – Physics/ Thermodynamics

MODULE 3 - ELECTRICAL FUNDAMENTALS

Sub-Module 3.1: Electron theory

1. A neutron has,

A. The opposite charge to an Electron but half the weight of a proton.

B. The same charge as an Electron but half the mass.

C. No electrical charge but will add weight to the nucleus.

Ref EASA Module 3 - Electrical Fundamentals/ Electron theory

2. The unit which consists of two or more different types of atoms is known as a.

A. Particle of an element.

B. Molecule of a compound.

C. Molecule of an element.

Ref EASA Module 3 - Electrical Fundamentals/ Electron theory

3. What charge does the nucleus of an atom possess?

A. Positive.

B. Neutral.

C. Negative.

Ref EASA Module 3 - Electrical Fundamentals/ Electron theory

4. If electrons are added to an atom it becomes.

A. A neutral ion.

B. A positive ion.

C. A negative ion.

Ref EASA Module 3 - Electrical Fundamentals/ Electron theory

5. If a free electron is given to another atom, that atom is a.

A. Valency atom.

B. Negative ion.

C. Positive ion.

Ref EASA Module 3 - Electrical Fundamentals/ Electron theory

6. The atomic number of an atom is determined by the number of

A. Protons.

B. Electrons.

C. Neutrons.

Ref EASA Module 3 - Electrical Fundamentals/ Electron theory

Sub-Module 3.2: Static Electricity and Conduction

1. On current is found in.

A. Conductors and semi-conductors.

B. Liquids and gasses.

C. Semi-conductors.

Ref EASA Module 3 - Electrical Fundamentals/ Static Electricity and Conduction

2. Electricity conducts through.

A. Vacuum.

B. Solids and liquids and gasses.

C. Solids only.

Ref EASA Module 3 - Electrical Fundamentals/ Static Electricity and Conduction

3. Electricity built up as an electrical charge on the surface of an object. It is considered

A. DC

B. AC

C. Static

Ref EASA Module 3 - Electrical Fundamentals/ Static Electricity and Conduction

4. If an insulated conductor is placed near to a negatively charged rod, the nearest end of the conductor becomes.

- No change will occur.

- Negatively charged.

- Positively charged.

Ref EASA Module 3 - Electrical Fundamentals/ Static Electricity and Conduction

5. The electromagnetic force between two charged bodies is.

  - Inversely proportional to the distance between them

  - Inversely proportional to the square of the distance between them.

  - Proportional to the distance between them.

Ref EASA Module 3 - Electrical Fundamentals/ Static Electricity and Conduction

6. What effect do the electrons of one atom have upon the electrons of another atom?

A. They have no effect on each other.

B. They repel each other.

C. They attract each other.

Ref EASA Module 3 - Electrical Fundamentals/ Static Electricity and Conduction

7. If a negatively charged conductor is placed next to an insulated rod.

A. The rod becomes negatively charged.

B. The rod has no charge.

C. The rod becomes positively charged.

Ref EASA Module 3 - Electrical Fundamentals/ Static Electricity and Conduction

Sub-Module 3.3: Electrical Terminology

1. Which of the following is 1 Amp?

A. 1000 mA.

B. 1000 kA.

C. 1000 μA.

Ref EASA Module 3 - Electrical Fundamentals/ Electrical Terminology

2. A volt can be considered to be a.

A. Unit of electrical power.

B. Quantity of electrical energy.

C. Unit of electrical pressure.

Ref EASA Module 3 - Electrical Fundamentals/ Electrical Terminology

3. What is 3.25 volts in millivolts?

A. 3,250 millivolts.

B. 325,000 millivolts.

C. 3.25 millivolts.

Ref EASA Module 3 - Electrical Fundamentals/ Electrical Terminology

4. Electromotive force is.

A. Watts.

B. Ohms.

C. Volts.

Ref EASA Module 3 - Electrical Fundamentals/ Electrical Terminology

5. 1µF capacitor is equivalent to.

A. 1,000,000 Farads.

B. 0.000,001 Farads.

C. 0.001 Farads.

Ref EASA Module 3 - Electrical Fundamentals/ Electrical Terminology

6. What is the SI unit of conductance?

  - Ohm.

  - Siemen.

  - Ohm-metre.

Ref EASA Module 3 - Electrical Fundamentals/ Electrical Terminology

7. Potential difference is another term for

  - Voltage

  - Charge

  - Energy

Ref EASA Module 3 - Electrical Fundamentals/ Electrical Terminology

Sub-Module 3.4: Generation of electricity

1. A piezoelectric device generates electricity through.

A. Friction.

B. Light.

C. Pressure.

Ref EASA Module 3 - Electrical Fundamentals/ Generation of electricity

2. A photozoidal cell produces electricity when subjected to.

A. Light.

B. Pressure.

C. Heat.

Ref EASA Module 3 - Electrical Fundamentals/ Generation of electricity

3. A component that produces an output voltage due to incident light is called a.

    - Liquid crystal.

    - Solar cell.

    - Light emitting diode.

Ref EASA Module 3 - Electrical Fundamentals/ Generation of electricity

4. Thermocouple harnesses are made from the same materials as the thermocouple so that.

A. Mini junctions are not formed.        B. The resistance is not increased beyond limits.C. They will not corrode.

Ref EASA Module 3 - Electrical Fundamentals/ Generation of electricity

5. A simple generator converts mechanical energy and magnetism into?

- Lines of flux per revolution

- Reverse polarity

- Electrical energy

Ref EASA Module 3 - Electrical Fundamentals/ Generation of electricity

6. A solar cell or a photovoltaic cell is a device that:

A. Converts sun energy into electricity.

B. Converts electricity energy into light energy

C. Converts light energy into electricity.

Ref EASA Module 3 - Electrical Fundamentals/ Generation of electricity

7. Thermocouples generate:

A. A voltage between the two wires that is proportional to the temperature at the junction.

B. A current between the two wires that is proportional to the temperature at the junction

C. A variable resistance between the two wires that is proportional to the temperature at the junction

Ref EASA Module 3 - Electrical Fundamentals/ Generation of electricity

Sub-Module 3.5: DC Source of Electricity

1. A battery rated at 40 Ah will supply 200 mA for.

A. 200 hours.

B. 20 hours.

C. 5 hours.

Ref EASA Module 3 - Electrical Fundamentals/ DC Source of Electricity

2. A 20 cell battery with each cell having an internal resistance of 0.1 ohms is charged with 2 leads having a total resistance of 0.1 ohms. The battery is charged with a current of 5 amps. What is the charging voltage?

A. 0.5 volts.

B. 10.5 volts.

C. 0.005 volts.

Ref EASA Module 3 - Electrical Fundamentals/ DC Source of Electricity

3. Two 10V, 20 Ah batteries are connected in parallel and connected across a 10 ohm load. How long could they supply normal current before the voltage begins to decay?

A. 40 hours.

B. 20 hours.

C. 4 hours.

Ref EASA Module 3 - Electrical Fundamentals/ DC Source of Electricity

4. A zinc-carbon battery life depends upon.

A. The amount of zinc.

B. The purity of the carbon rod.

C. The amount of the electrolyte paste.

Ref EASA Module 3 - Electrical Fundamentals/ DC Source of Electricity

5. Two batteries 12V and 40 Ah each, are in series. What is the total capacity?.

A. 12V 80 Ah.

B. 24V 40 Ah.

C. 24V 80Ah.

Ref EASA Module 3 - Electrical Fundamentals/ DC Source of Electricity

6. If a battery has got low internal resistance, then the.

A. No load voltage will be the same as on load voltage.

B. On load voltage will be greater than no load voltage.

C. No load voltage will be greater than on load voltage.

Ref EASA Module 3 - Electrical Fundamentals/ DC Source of Electricity

7. Conventional current flow inside a battery is from.

A. Either anode to cathode or cathode to anode, depending on the active elements.

B. Cathode to anode.

C. Anode to cathode.

Ref EASA Module 3 - Electrical Fundamentals/ DC Source of Electricity

Sub-Module 3.13: AC Theory

1. Convention requires that in a symmetrical 3 phase system, the.

A. Red voltage is taken as the reference phase.

B. Yellow voltage phase leads the red phase by 120 degrees.

C. Red voltage phase leads the blue phase by 120 degrees.

Ref EASA Module 3 - Electrical Fundamentals/ AC Theory

2. One advantage of using AC electrical power in aircraft is.

A. Space and weight can be saved, since AC devices, especially motors, are smaller and simpler than DC devices.

B. AC electrical motors can be reversed while DC motors cannot.

C. The effective voltage is 1.41 times the maximum instantaneous voltage; therefore, less power input is required.

Ref EASA Module 3 - Electrical Fundamentals/ AC Theory

3. Frequency (Hz) is the number of cycles per.

A. Minute

B. Second

C. Revolution

Ref EASA Module 3 - Electrical Fundamentals/ AC Theory

4. An integrated step input at unequal time base produces.

A. a sine wave.

B. a square wave.

C. saw tooth wave.

Ref EASA Module 3 - Electrical Fundamentals/ AC Theory

5. An integrated step input at unequal time base produces.

A. a sine wave.

B. a square wave.

C. saw tooth wave.

Ref EASA Module 3 - Electrical Fundamentals/ AC Theory

6. A wave form having equal amplitude and time base is a.

A. Rectangular.

B. Saw tooth.

C. Square waveform

Ref EASA Module 3 - Electrical Fundamentals/ AC Theory

MODULE 5 - DIGITAL TECHNIQUES – ELECTRONIC INSTRUMENT SYSTEMS

Sub-Module 5.1: Electronic Instruments System

1. Typical displays on an EHSI are.

A. Engine indications.

B. VOR, Map, Plan and weather radar.

C. VOR, Plan, Map and Attitude.

Ref EASA Module 5 - Digital Techniques/ Electronic Instruments System

2. What is the fixed feature of an ADI?

A. The glideslope pointer.

B. The aircraft symbol.

C. The lateral deviation bar.

Ref EASA Module 5 - Digital Techniques/ Electronic Instruments System

3. Engine parameters are displayed on.

A. ECAM.

B. EHSI.

C. FMSCDU

Ref EASA Module 5 - Digital Techniques/ Electronic Instruments System

4. What is the fixed feature of an ADI?

A. The glideslope pointer.

B. The aircraft symbol.

C. The lateral deviation bar.

Ref EASA Module 5 - Digital Techniques/ Electronic Instruments System

5. What are the basic elements for 'Classic T' format?

A. Direction, altitude and height.

B. Airspeed, pitch and roll.

C. Airspeed, attitude, altitude and direction

Ref EASA Module 5 - Digital Techniques/ Electronic Instruments System

6. On an EFIS system the weather radar is displayed on.

A. The FMC CDU.

B. The EADI.

C. The EHSI

Ref EASA Module 5 - Digital Techniques/ Electronic Instruments System

7. EADI displays show.

A. Pitch, roll and waypoints.

B. Pitch and roll attitudes.

C. Heading and weather radar.

Ref EASA Module 5 - Digital Techniques/ Electronic Instruments System

8. Radio altitude is displayed on an EFIS system.

A. On the Electronic Horizontal Situation Indicator.

B. On the Electronic Attitude Direction Indicator.

C. On the RMI.

Ref EASA Module 5 - Digital Techniques/ Electronic Instruments System

9. On a modern 'glass cockpit' aircraft, engine information is displayed on.

A. FMS.

B. EFIS.

C. ECAM.

Ref EASA Module 5 - Digital Techniques/ Electronic Instruments System

10. What does EFIS mean?

A. Electronic Fire Indication Signal.

B. Electronic Flight Instrument System.

C. Electronic Flight Information System.

Ref EASA Module 5 - Digital Techniques/ Electronic Instruments System

11. What does EICAS mean?

A. Electronic indicator and control alerting system.

B. Engine indicating and Crew alerting system.

C. Electronic indicator and crew alerting system.

Ref EASA Module 5 - Digital Techniques/ Electronic Instruments System

12. What does ECAM mean?

A. Engine Centralised Aircraft Management System.

B. Engine Centralised Aircraft Monitoring System.

C. Electronic Centralised Aircraft Monitoring System.

Ref EASA Module 5 - Digital Techniques/ Electronic Instruments System

13. On an EADI, the Flight Director command bars show.

A. The path with respect to the horizon.

B. The required path with respect to the actual path.

C. The actual path with respect to the required path.

Ref EASA Module 5 - Digital Techniques/ Electronic Instruments System

14. What would you expect to see displayed on an EADI display?

A. Compass heading, selected heading and VOR.

B. Course information, weather radar, way point alert and bearing pointers.

C. Flight director command bars, slip indicator, rate to altitude& autoland.

Ref EASA Module 5 - Digital Techniques/ Electronic Instruments System

15. An EADI display of flight director commands are coloured.

A. cyan.

B. magenta.

C. red.

Ref EASA Module 5 - Digital Techniques/ Electronic Instruments System

16. The EFIS system consists of.

A.  EHSI, Mode control panel, EADI.

B.  EADI, EHSI, Symbol generators.

C.  Mode control Panel, RDMI, EHSI.

Ref EASA Module 5 - Digital Techniques/ Electronic Instruments System

Sub-Module 5.6: Basic Computer Structure

1. A BYTE is usually.

A. An 8 bit word.

B. A six bit word.

C. A 12 bit word.

Ref EASA Module 5 - Digital Techniques/ Basic Computer Structure

2. A computer using RAM would utilize.

A. Magnetic tape.

B. A compact disc.

C. An integrated circuit (chip).

Ref EASA Module 5 - Digital Techniques/ Basic Computer Structure

3. What is EPROM?

A. Enhanced programmable read only memory.

B. Erasable programmable read only memory.

C. Erasable programming read only module.

Ref EASA Module 5 - Digital Techniques/ Basic Computer Structure

4. How many bytes can be carried in a 32bit word?

A. 8 bytes.

B. 4 bytes.

C. 2 bytes.

Ref EASA Module 5 - Digital Techniques/ Basic Computer Structure

5. Data is usually stored in.

A. RAM.

B. ROM.

C. EEPROM.

Ref EASA Module 5 - Digital Techniques/ Basic Computer Structure

6. A computer requires to operate, a.

A. Address bus, a data bus only.

B. Data bus, a control bus, a address bus.

C. Data bus, a control bus only.

Ref EASA Module 5 - Digital Techniques/ Basic Computer Structure

Sub-Module 5.12: Electrostatic Sensitive Devices

1. What precautions are required when removing ESDS equipment?

A. Wear a wrist strap connected to an approved ground point.

B. No special precautions are required.

C. Keep one hand on the airframe.

Ref EASA Module 5 - Digital Techniques/ Electrostatic Sensitive Devices

2. Electrostatic discharge occurs when.

A. The ground lead on a wrist strap touches the skin.

B. Materials are rubbed together or pulled apart.

C. Materials of different potential are brought in close proximity to each other.

Ref EASA Module 5 - Digital Techniques/ Electrostatic Sensitive Devices

3. The resistance of the wrist strap used as part of anti static precautions should be.

A. Clipped into a receptacle with a 200 megaohm resistor connected to a common ground point.

B. 1 megohm - 200 megohm.

C. Clipped into a receptacle with a 1 megaohm resistor connected to a common ground point.

Ref EASA Module 5 - Digital Techniques/ Electrostatic Sensitive Devices

4. Which insulating materials should be kept out of the work area.

A. Vinyl, rubber, mica, copper

B. Vinyl, rubber, mica, nylon

C. Vinyl, rubber, mica, nylon, conductive footwear.

Ref EASA Module 5 - Digital Techniques/ Electrostatic Sensitive Devices

5. After attaching a wrist strap to your wrist (which is connected to aircraft ground), prior to removal of ESDS equipment, the resistance measured.

A. Between your skin and the pin ground must be >1 megohms.

B. Between the strap and the pin ground must be <1 megohms.

C. Between the strap and the pin ground must be >1 Megohms.

Ref EASA Module 5 - Digital Techniques/ Electrostatic Sensitive Devices

6. A wrist band must be worn when working with ESDS devices.

A. To dissipate and prevent static build-up on the operator

B. To prevent the operator from getting an electric shock.

C. To dissipate static charge on the device.

Ref EASA Module 5 - Digital Techniques/ Electrostatic Sensitive Devices

7. A humid atmosphere.

A. Reduces static charge.

B. Eliminates static charge.

C. Has no effect on the level of charge.

Ref EASA Module 5 - Digital Techniques/ Electrostatic Sensitive Devices

8. Static Safe Workstation must have

A. Floor and table-top mats both connected through a 1 mega-ohm resistor connected to a common ground point.

B. Floor and table-top mats both connected through a 20 mega-ohm resistor connected to a common ground point.

C. Floor and table-top mats do not require static protection, only personal require protection.

Ref EASA Module 5 - Digital Techniques/ Electrostatic Sensitive Devices

MODULE 6 - MATERIALS AND HARDWARE

Sub-Module 6.1: Aircraft Materials – Ferrous

1. What temperature would steel be tempered at?

A.  At the annealing temperature.

B.  Above the annealing temperature.

C.  Below the annealing temperature.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

2. Chromium added to plain carbon steel.

A.  Increases its resistance to corrosion.

B.  Turns it into a non-ferrous alloy.

C.  Makes the metal softer

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

3. Purpose of case hardening is to.

A.  Produce a hard case over a tough core.

B.  Reduce the carbon in the steel.

C.  Introduce carbon into the steel

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

4. Tempering entails.

A. Heating under the UCP and slow cooling.

B. Heating over the UCP and fast cooling.

C. Heating over the UCP and slow cooling.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

5. Nitriding is.

A. Tempering.

B. Anodizing.

C. Case hardening

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

6. A ferrous metal contains.

A.  Aluminium.

B.  Iron.

C.  Magnesium.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

7. With respect to ferrous metals which of the following is true?

A.  Iron is not any element of ferrous metals.

B.  Iron is a main element and most ferrous metal are magnetic.

C.  Iron is a main element and ferrous metals are not magnetic.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

8. The normalizing process on steel is required sometimes as it

A.  Provides a corrosion resistant layer that prevents oxidation.

B.  Allows the material a greater stress per unit area.

C.  Relieves internal stress suffered after engineering processes

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

9. The hardness of steel depends upon.

A.  Formation of pearlite into austenite.

B.  Formation of cementite. (iron carbide)

C.  The iron austenite grain structure

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

10. The difference between annealing and normalizing is.

A.  Both are heated above the UCT, cool slowly to anneal, cool in air to normalize

B.  Both are heated below the UCT, cool in air to anneal, cool slowly to normalize.

C.  Both are heated above the UCT, cool in air to anneal, cool slowly to normalize.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

11. Normalizing of steel is done to.

A.  Remove residual stress of the manufacturing process.

B.  Make steel softer.

C.  Restore the fatigue life of steel.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

12. When normalizing the material is.

A.  Quenched immediately.

B.  Left to cool in room temperature.

C.  Cooled slowly

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

13. Cast iron is.

A.  Very malleable.

B.  Tough.

C. Brittle.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

14. Case hardening can be carried out on.

A. Titanium.

B.  Any ferrous metal.

C.  Duralumin.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

15. Steel is tempered.

A. After hardening.

B. Before hardening.

C. To increase hardening

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

16. A low carbon steel would normally be case hardened using.

A.  The nitriding process.

B.  Flame or induction hardening.

C.  Pack or gas carburizing

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

17. Normalizing steels.

A. Increases toughness.

B. Increases the hardness.

C  Relieves the stresses

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

18. When metal is first heated slightly above its critical temperature and then cooled rapidly it is common that the metal will increase in.

A. Brittleness.

B.  Hardness

C.  Both of the above

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

19. Annealing steels.

A. Toughens the metal.

B. Makes the metal malleable.

C. Makes the metal brittle

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

20. Tempering of hardened steel is carried out to.

A. Retain surface hardness, but soften the core.

B. Retain core hardness, but soften the surface.

C. Significantly reduce the brittleness without suffering a major drop in its strength

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

21. Malleable materials are.

A.  Easy to forge.

B.  Easy to cast.

C.  Highly ductile.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

22. If a material has to undergo deep cold forming operation. The essential property would be.

A.  Ductility.

B.  Malleability.

C.  Elasticity.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

23. Ductility is the property of a metal which permits it

A. To be permanently drawn, bent, or twisted but is subject to breaking

B. To be permanently drawn, bent, or twisted

C. Not to be used for fuselage and wing skin

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

24. Non-heat treatable materials.

A. Cannot be hardened.

B. Can be hardened by strain hardening or cold working.

C. Can be hardened by annealing. Correct Answer is. can be hardened by strain hardening or cold working.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

25. The ease with which a material can be forged, rolled and extruded without fracture is an indication of a material's.

A. malleability.

B. ductility.

C. brittleness.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials – Ferrous

Sub-Module 6.2: Aircraft Materials - Non-Ferrous

1. Wrought alloys are used most widely in aircraft construction for

A. Stringers, bulkheads

B. Skin, rivets, and extruded sections

C. Stringers, bulkheads, skin, rivets, and extruded sections

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Non-Ferrous

2. The metal, which must not be heated in a salt bath, is.

A. Magnesium alloy.

B. Duralumin.

C. Rivets made of alclad

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Non-Ferrous

3. The critical process of heat treatment is.

A. Temperature, method of heating and cooling.

B. Temperature and method of heating only.

C. Method of heating only

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Non-Ferrous

4. What is the effect of precipitation heat treatment on aluminium?

A. It speeds up age hardening process.

B. It delays the age hardening process.

C. It softens the material to allow it to be worked

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Non-Ferrous

5. If caustic soda turns a material black what is it?

A. Aluminium alloy.

B. Alclad.

C. Aluminium.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Non-Ferrous

6. What chemical is used to identify aluminium alloys?

A. Caustic soda.

B. Copper sulphate.

C. Nitric acid

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Non-Ferrous

7. Precipitation treating makes the metal.

A. Precipitation treating makes the metal.

B. Less strong and hard.

C. Harder, stronger and less ductile

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Non-Ferrous

8. Clad aluminium alloy (alclad) has a pure aluminium coating of.

A. 0.002.

B. 1% of alloy thickness.

C. 5% of alloy thickness.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Non-Ferrous

9. Titanium can be identified by placing it on a grinding wheel and looking for.

A. Red Sparks.

B. Yellow Sparks.

C. White Sparks.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Non-Ferrous

10. Pure aluminium is.

A. Highly resistant to corrosion.

B. Not resistant to corrosion.

C. Reasonably resistant to corrosion.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Non-Ferrous

11. The main metal in monel is.

A. Aluminium.

B. Nickel.

C. Stainless steel.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Non-Ferrous

12. The time between removal from heat treatment furnace and quenching must be not more than.

A.   3 seconds.

B   10 seconds.

C.  7 seconds

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Non-Ferrous

13. Monel metal consists of approximately.

A.   66% Chromium and 33% Copper.

B.   66% Copper and 33% Nickel.

C.   66% Nickel and 33% Copper.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Non-Ferrous

14. The aluminum code number 1100 identifies what type of aluminum?

A. Aluminum alloy containing 11 percent copper.

B. Aluminum alloy containing zinc.

C. 99 percent commercially pure aluminum. Aluminum identified by the code number 1100 is 99 percent commercially pure aluminum.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Non-Ferrous

15. In the four-digit aluminium index system number 2024, the first digit indicates

A. The major alloying element.

B. The number of major alloying elements used in the Metal.

C. The percent of alloying metal added

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Non-Ferrous

16. A material containing approximately 66% nickel and 33% copper is known as.

A. Nimonic.

B. Monel metal.

C. Invar

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Non-Ferrous

17. The extrusion process

A. Involves the forcing of metal through an opening in a die

B. Of metals such as lead, tin, and aluminum may be extruded cold

C. Both A and B are correct

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Non-Ferrous

Sub-Module 6.3: Aircraft Materials - Composite & Non Metallics

1. Fibre weave strength is greatest in the direction of the.

A. Weft.

B. Warp.

C. Bias.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Composite & Non Metallics

2. Why are Electric motors not recommended for use with composite materials?

- Carbon dust wears the bearings.

- Carbon is conductive and can cause an electrical short circuit.

- They are not as strong as air driven tools.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Composite & Non Metallics

3. A mechanic has completed a bonded honeycomb repair using the potted compound repair technique. What non-destructive testing method can be used to determine the soundness of the repair after the repair has cured?.

A. Eddy current test.

B. Metallic ring test. ( Tap test)

C. Ultrasonic test.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Composite & Non Metallics

4. How do you reduce or remove electrostatic charges which may build up on composite surfaces?.

A.No special treatment is necessary because fibre glass is an ins

B. The surface is treated with a special conductive paint.

C. The surface is impregnated with copper strips.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Composite & Non Metallics

5. Useful method for the detection of internal delamination’s, voids, or inconsistencies in composite components not otherwise discernible using visual or tap methodology is

- X-ray

- Ultrasonic

- Eddy current

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Composite & Non Metallics

6. Storage temperature of prepreg, film adhesive, and foaming adhesives is

- Between -18°C to 0°C

- Near 4°C

- Below -18°C

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Composite & Non Metallics

7. What available NDE techniques are used to defect water ingression in radomes?

- Radiography, thermography, moisture meter

- Radiography, thermography, tap test, dye penetrant

- Radiography, thermography, eddy current, ultrasonic

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Composite & Non Metallics

8. Delamination, resin starved areas, resin rich areas, blisters, air bubbles, wrinkles, voids, thermal decomposition are included in

- Manufacturing defects

- In-service defects

- Manufacturing defects and In-service defects

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Composite & Non Metallics

9. Environmental degradation, impact damage, fatigue, cracks from local overload, de-bonding, delamination, fiber fracturing, erosion are included in

- Manufacturing defects

- In-service defects

- Manufacturing defects and In-service defects

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Composite & Non Metallics

10. Which types of damages cannot be found during visual inspections?

- Resin starvation, resin richness, wrinkles, ply bridging, discoloration (due to overheating, lightning strike, etc.)

- Impact damage by any cause, foreign matter, blisters, and dis-bondin

- Internal flaws in the composite, such as delamination’s, dis-bonds, and matrix crazing

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Composite & Non Metallics

11. Peel plies are used to

- Seal the vacuum bag to the part or tool

- Create a clean surface for bonding purposes.

- Creates a path for the air and volatiles to escape from the repair

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Composite & Non Metallics

12. Which vacuum bagging technique is frequently used for removable aircraft parts, such as flight controls, access panels, etc., and when a part's geometry and/or the repair location makes it very difficult to properly vacuum bag and seal the area in a vacuum?

- Single Side Vacuum Bagging

- Envelope Bagging.

- Impregnation Using a Vacuum Bag

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Composite & Non Metallics

13. Replacement of a wooden spar is.

A. Only permitted by the manufacturer.

B. A minor repair.

C. A major repair.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Composite & Non Metallics

14. A crack is found in a wooden spar, you.

A. Must replace the spar section.

B. Might be able to repair the spar.

C. Must replace the entire spar.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Composite & Non Metallics

15. Kevlar and Nomex are types of.

A. Glass fiber composite.

B. Aramid fiber.

C. Aluminium alloy.

Ref EASA Module 6 - Materials and Hardware/ Aircraft Materials - Composite & Non Metallics

Sub-Module 6.4: Corrosion

1. Intergranular corrosion is caused by.

A.  Improperly assembled components.

B.  Improper heat treatment.

C.  Dissimilar metal contact

Ref EASA Module 6 - Materials and Hardware/ Corrosion

2. After a product has been manufactured and all heat treatment has been carried out the stress remaining if any is termed as.

A. Residual stress.

B. Working stress.

C. Applied stress.

Ref EASA Module 6 - Materials and Hardware/ Corrosion

3. When buffing surface of Aluminium Alloy, what material are you removing?

A. Oxide layer.

B.  Aluminium.

C. Alloy

Ref EASA Module 6 - Materials and Hardware/ Corrosion

4. Galvanic corrosion caused by electrolytic action is the result of

A.   Contact between two unlike metals.

B.   Excessive anodization.

C.   The wrong quenching agent

Ref EASA Module 6 - Materials and Hardware/ Corrosion

5. Corrosion may be regarded as the destruction of metal by.

A.  Electrochemical action.

B.  Hydroelectric action.

C.  Electromechanical action

Ref EASA Module 6 - Materials and Hardware/ Corrosion

6. Which of these is a common cause of corrosion

A.  Water in fuel.

B.  Spilled battery acid

C.  Untreated metal.

Ref EASA Module 6 - Materials and Hardware/ Corrosion

7. What is stress corrosion?

A.  Corrosion in an area under cyclic loading.

B.  Corrosion due to fretting.

C.  Corrosion in an area under continuous loading.

Ref EASA Module 6 - Materials and Hardware/ Corrosion

8. Chemical attack is a direct result of.

A.  Intergranular corrosion.

B. Cathodic & anodic changes in the material.

C. Filiform corrosion.

Ref EASA Module 6 - Materials and Hardware/ Corrosion

9. Galvanic corrosion is caused by.

A.  Incorrect heat treatments or incorrect alloying.

B.  Cyclic stressing and a decrease in cross sectional area.

C.  The joining of 2 dissimilar metals.

Ref EASA Module 6 - Materials and Hardware/ Corrosion

10. Corrosion will spread more rapidly when metals are exposed to

A.  Cold climates.

B.  Higher temperature

C.  Dry climates

Ref EASA Module 6 - Materials and Hardware/ Corrosion

11. Corrosion control begins at the.

A.  Production stage.

B.  Manufacturing stage.

C.  Design stage.

Ref EASA Module 6 - Materials and Hardware/ Corrosion

12. When galvanic corrosion takes place, which part corrodes?

A.  Anode.

B.  Oxide film coating.

C.  Cathode

Ref EASA Module 6 - Materials and Hardware/ Corrosion

13. Corrosion caused by galvanic action is the result of.

A.  Contact between two unlike metals.

B.   Excessive anodization.

C.   Excessive etching

Ref EASA Module 6 - Materials and Hardware/ Corrosion

14. When dissimilar metals are brought together, or the same metal in different states, one metal forms the anode and the other the cathode. Which will suffer from corrosion?

A.  Both will corrode equally

B.  The cathode.

C.  The anode.

Ref EASA Module 6 - Materials and Hardware/ Corrosion

15. Black streaks back from a rivet hole signify.

- Galvanic corrosion.

- Intergranular corrosion.

- Fretting

Ref EASA Module 6 - Materials and Hardware/ Corrosion

16. The lifting or flaking of the metal at the surface due to delamination of grain boundaries caused by the pressure of corrosion is.

- Exfoliation.

- Electrolysis.

- Trans granulation

Ref EASA Module 6 - Materials and Hardware/ Corrosion

17. Aluminium is.

A.  Not resistant to corrosion.

B.  Highly resistant to corrosion.

C.  Reasonably resistant to corrosion

Ref EASA Module 6 - Materials and Hardware/ Corrosion

18. In the galvanic series, the most noble metal will, if joined to another metal.

A. Always be at the top of the table

B. Corrode before the less noble metal.

C. Allow the less noble metal to corrode first.

Ref EASA Module 6 - Materials and Hardware/ Corrosion

19. What is the indication of fretting corrosion on aluminium alloy?.

- Black powder.

- Brown powder

- White powder

Ref EASA Module 6 - Materials and Hardware/ Corrosion

20. What action is taken to protect integral tanks from corrosion due to micro-biological growth?.

- The inside of the tank is coated with yellow chromate.

- Rubber liners are installed in the tank.

- A biocidal additive is used in the fuel

Ref EASA Module 6 - Materials and Hardware/ Corrosion

21. What type of corrosion attacks grain boundaries of aluminium alloys which are improperly or inadequately heat treated?.

- Stress corrosion.

- Intergranular corrosion

- Surface corrosion.

Ref EASA Module 6 - Materials and Hardware/ Corrosion

22. Intergranular corrosion in structural aluminium alloy parts.

- Are not likely to occur in parts fabricated from heat-treated sheet aluminium.

- May be detected by the white, powdery deposit formed on the surface of the metal.

- Cannot always be detected by surface indications

Ref EASA Module 6 - Materials and Hardware/ Corrosion

23. Corrosion under the surface coating can be detected by

A. Frosted appearance

B. Roughening of the surface

C. Paint or plating is lifted off the surface in small blisters

Ref EASA Module 6 - Materials and Hardware/ Corrosion

24. Corrosion at the grain boundaries is called.

- Intergranular.

- Fretting.

- Filiform

Ref EASA Module 6 - Materials and Hardware/ Corrosion

25. Fretting corrosion occurs where?

- Two material are subject to very slight movement between the two.

- A deep scratch increases to become fretting corrosion.

- Pitting corrosion is allowed to become more pronounced

Ref EASA Module 6 - Materials and Hardware/ Corrosion

26. What are the signs of fretting corrosion?

- Black stain around a rivet

- Intergranular cracking.

- Flaking

Ref EASA Module 6 - Materials and Hardware/ Corrosion

27. Surface corrosion on stainless steel is identified by.

- Uniform red, brown, stain.

- White film.

- Black pitting

Ref EASA Module 6 - Materials and Hardware/ Corrosion

28. The usual manufacturer’s anti-corrosion process to be applied to Fe aircraft parts is.

- Cadmium plating.

- Anodizing.

- Metal spraying.

Ref EASA Module 6 - Materials and Hardware/ Corrosion

29. Corrosion on copper is identified by.

- Red/brown rust.

- Black powder.

- Green to blue copper salts on the corroded part

Ref EASA Module 6 - Materials and Hardware/ Corrosion

30. Fretting corrosion occurs when

- Water is trapped between moving parts.

- Two mating surfaces are subject to slight relative motion.

- Improperly heat treated.

Ref EASA Module 6 - Materials and Hardware/ Corrosion

31. Intergranular corrosion is also known as what?

- Stress corrosion.

- Exfoliation.

- Galvanic.

Ref EASA Module 6 - Materials and Hardware/ Corrosion

32. Corrosion of metals in a metal to metal joint or corrosion at the edge of a joint is

- Filiform corrosion.

- Fretting corrosion.

- Crevice corrosion.

Ref EASA Module 6 - Materials and Hardware/ Corrosion

33. Lead acid battery fluid has been found to be leaking on the surface of the aircraft structure. What substance would you use to neutralize the acid?

- Sulphur and Lime

- Hot distilled water.

- Sodium bicarbonate of soda.

Ref EASA Module 6 - Materials and Hardware/ Corrosion

34. A primary cause of intergranular corrosion is.

- Improper heat treatment.

- Improper application of primer.

- Dissimilar metal contact

Ref EASA Module 6 - Materials and Hardware/ Corrosion

Sub-Module 6.5: Fasteners

1. Aircraft bolts are almost always manufactured in

- Class 2, free fit

- Class 3, medium fit

- Class 4, close fit

Ref EASA Module 6 - Materials and Hardware/ Fasteners

2. Most commonly used thread form in aviation is

A. Unified or metric threads

B. Round thread.

C. Buttress.

Ref EASA Module 6 - Materials and Hardware/ Fasteners

3. Pitch of a screw thread is defined as.

A. Distance from the centre of one crest to the next.

B. Difference between major diameter and minor diameter.

C. Distance between the crest and the root of the thread.

Ref EASA Module 6 - Materials and Hardware/ Fasteners

4. A metric screw thread angle is.

- 7 °

- 57 °

- 60 °

Ref EASA Module 6 - Materials and Hardware/ Fasteners

5. Aircraft bolts with a cross or asterisk marked on the bolt head are

- Made of aluminum alloy.

- Close tolerance bolts.

- Standard steel bolts.

Ref EASA Module 6 - Materials and Hardware/ Fasteners

6. A particular component is attached to the aircraft structure by the use of an aircraft bolt and a castle tension nut combination. If the cotter pin hole does not align within the recommended torque range, the acceptable practice is to

A. The nut may not be over torque

B. Tighten below the torque range.

C. Change washers and try again.

Ref EASA Module 6 - Materials and Hardware/ Fasteners

7. A bolt with a single raised dash on the head is classified as an

- AN corrosion-resistant steel bolt.

- NAS standard aircraft bolt.

- NAS close tolerance bolt.

Ref EASA Module 6 - Materials and Hardware/ Fasteners

8. Where is an AN clevis bolt used in an airplane?

- For tension and shear load conditions.

- Where external tension loads are applied.

- Only for shear load applications.

Ref EASA Module 6 - Materials and Hardware/ Fasteners

9. A bolt with an X inside a triangle on the head is classified as an

- NAS standard aircraft bolt.

- NAS close tolerance bolt.

- AN corrosion-resistant steel bolt.

Ref EASA Module 6 - Materials and Hardware/ Fasteners

10. Aircraft bolts are usually manufactured with a

- Class 1 fit for the threads.

- Class 2 fit for the threads.

- Class 3 fit for the threads.

Ref EASA Module 6 - Materials and Hardware/ Fasteners

11. How is the locking feature of the fiber-type locknut Obtained?

- By the use of an unthreaded fiber locking insert.

- By a fiber insert held firmly in place at the base of the load carrying section.

- By making the threads in the fiber insert slightly smaller than those in the load carrying section.

Ref EASA Module 6 - Materials and Hardware/ Fasteners

12. Which type of pin is used with tie rod terminals and in secondary controls which are not subject to continuous operation?

- Flathead pin

- Taper pin

- Cotter pin.

Ref EASA Module 6 - Materials and Hardware/ Fasteners

13. Plain washers, both the AN960 and AN970,

- Are used under hex nuts

- Are used with machine screws or bolts where the self locking or castellated type nut is not appropriate

- Are used where a bolt is installed at an angle to a surface, or where perfect alignment with a surface is required.

Ref EASA Module 6 - Materials and Hardware/ Fasteners

14. Lock washers, both the AN935 and AN936,

- Are used under hex nuts

- Are used with machine screws or bolts where the self-locking or castellated type nut is not appropriate

- Are used where a bolt is installed at an angle to a surface, or where perfect alignment with a surface is required.

Ref EASA Module 6 - Materials and Hardware/ Fasteners

15. The ball socket and seat washers, AC950 and AC955,

- Are used under hex nuts

- Are used with machine screws or bolts where the selflocking or castellated type nut is not appropriate.

- Are used where a bolt is installed at an angle to a surface, or where perfect alignment with a surface is required.

Ref EASA Module 6 - Materials and Hardware/ Fasteners

16. What method is used on most modern aircraft to safety turn buckles?

A. The double wrap method

B.  Clip locking method

C. Single wrap spiral method

Ref EASA Module 6 - Materials and Hardware/ Fasteners

17. The AN-3 and the AN-73 series bolts are

- Interchangeable for all practical purposes

- Tension and shear strengths are different.

- Are non-drilled head bolts

Ref EASA Module 6 - Materials and Hardware/ Fasteners

18. What metal is suitable for riveting magnesium alloy?

- Monel metal

- 5606 aluminium alloy

- 2024 aluminium alloy

Ref EASA Module 6 - Materials and Hardware/ Fasteners

19. If  D or DD rivets are not broached in time , or removed from the freezer in time, they

- Must be re-heat treated before use

- Can be re-heat treated once then discarded

- Must be discarded

Ref EASA Module 6 - Materials and Hardware/ Fasteners

20. An aluminium alloy rivet identification is

- D embossed

- X embossed

- Embossed

Ref EASA Module 6 - Materials and Hardware/ Fasteners

21. D and DD rivets are refrigerated to

- Retard age hardening

- Increase age hardening

- Precipitation harden

Ref EASA Module 6 - Materials and Hardware/ Fasteners

22. What do you do to 2017 and 2024 rivets after heat treatment?

- Use immediately or refrigerate

- Leave for a minimum of 2 hours before using

- Refrigerate for a minimum of 2 hours before using

Ref EASA Module 6 - Materials and Hardware/ Fasteners

23. An American rivet with a cross on the head is

- 2117 aluminium alloy

- 5056t aluminium alloy

- 2124 aluminium alloy

Ref EASA Module 6 - Materials and Hardware/ Fasteners

24. A countersunk rivet of the ‘SP’ series has a head style of

- 90o

- 100o

- 20o

Ref EASA Module 6 - Materials and Hardware/ Fasteners

25. When a solid rivet is formed it only holds 75% of its shear strength, the other 25% comes from

- Heating the completed assembly

- Cooling completed assembly

- Leaving assembly for 4 days to age harden

Ref EASA Module 6 - Materials and Hardware/ Fasteners

26. One of the main advantages of the Hi-Lok type fasteners over earlier generations is that.

A. They can be installed with ordinary hand-tools.

B. They can be removed and reused again.

C. The squeezed on collar installation provides a more secure, tighter fit.

Ref EASA Module 6 - Materials and Hardware/ Fasteners

27. Deutsch rivet consists of

A. An aluminium sleeve and the hardened steel drive pin

B. A stainless steel sleeve and the hardened steel drive pin

C. The stainless steel sleeve and the aluminium drive pin

Ref EASA Module 6 - Materials and Hardware/ Fasteners

28. Which thread form is compatible with the unified or metric forms?

- Obsolete British Standard thread forms

- Whitworth thread forms.

- Neither A nor B is correct

Ref EASA Module 6 - Materials and Hardware/ Fasteners

29. Lock plates are used

- Where positive retention of a nut is required

- Are used where positive retention of a nut is required.

- Both A and B are correct

Ref EASA Module 6 - Materials and Hardware/ Fasteners

30. Jo-bolts

- Consists of three parts

- Are resistance to vibration, weight saving, and are fast to install by one person.

- Both A and B are correct

Ref EASA Module 6 - Materials and Hardware/ Fasteners

Sub-Module 6.6: Pipes and Unions

1. A fluid line marker with a skull & crossbones is.

  - Fluid line carries toilet waste.

  - Warning symbol.

  - Radioactive symbol.

Ref EASA Module 6 - Materials and Hardware/ Pipes and Unions

2. The international marking for a fire protection pipe line is a series

  - Circles.

  - Squares.

  - Diamonds.

Ref EASA Module 6 - Materials and Hardware/ Pipes and Unions

3. The end of the fitting that passes through the bulkhead is

- Shorter than the other end(s), which allows a locknut to be installed, securing the fitting to the bulkhead.

- Equal to the other end(s), which allows a locknut to be installed, securing the fitting to the bulkhead.

- Longer than the other end(s), which allows a locknut to be installed, securing the fitting to the bulkhead.

Ref EASA Module 6 - Materials and Hardware/ Pipes and Unions

4. Size of MS flareless tube fittings MS 21900 –4D is

- 4/8 inch

- 4/16 inch

- 4/32 inch

Ref EASA Module 6 - Materials and Hardware/ Pipes and Unions

5. MS flareless tube fittings MS 21900 –4D is made from

- Aluminum Alloy

- Steel

- Brass

Ref EASA Module 6 - Materials and Hardware/ Pipes and Unions

Sub-Module 6.8: Bearings

1. Ball and roller bearings are made from a combination of low carbon steel and a percentage of.

  - Chromium.

  - Nickel.

  - Nickel chrome

Ref EASA Module 6 - Materials and Hardware/ Bearings

2. Thrust bearings transmit.

  - Thrust loads, thus limiting axial movement.

  - Radial loads, thus limiting axial movement.

  - Thrust loads, thus limiting radial movement.

Ref EASA Module 6 - Materials and Hardware/ Bearings

Sub-Module 6.9: Transmissions

1. Backlash is a type of wear associated with.

- Gears.

- Rivets.

- Bearings.

Ref EASA Module 6 - Materials and Hardware/ Transmissions

2. A gear with teeth twisted at an angle from the gear center line is a

        - Helical Worm Gear

        - Rack and pinion.

        - Bevel gears

Ref EASA Module 6 - Materials and Hardware/ Transmissions

3. Rack and pinion gears

- Are used in changing rotary motion into linear motion or the reverse.

- Has multiple uses.

- Has less precision than most other options.

Ref EASA Module 6 - Materials and Hardware/ Transmissions

Sub-Module 6.10: Control Cables

1. Control cable pulleys should be lubricated

        - Every C check.

        - When stiffness is felt in the control column.

        - Not required bearings are sealed lubricated at the factory

Ref EASA Module 6 - Materials and Hardware/ Control Cables

2. If the turnbuckles are in a control system to

  - Enable quick disconnect of cables.

  - Enable cable tension to be adjusted.

  - Enable cable ends to be joined without swaging.

Ref EASA Module 6 - Materials and Hardware/ Control Cables

3. A 7x7 cable has seven strands each of.

A. Fourteen wires.

B. Seven wires.

C. One wire.

Ref EASA Module 6 - Materials and Hardware/ Control Cables

4. In a Teleflex flexible control system, the Teleflex cable consists of.

A. multi strand steel wires and is used primarily as a single one way device operated from a control lever.

B. a high tensile steel wire with a right or left hand helix wire wound on to it. The system can operate in two directions.

C. a flexible seven or nineteen strand steel cable used for the operation of manual flying controls.

Ref EASA Module 6 - Materials and Hardware/ Control Cables

5. A cable tension regulator will be installed in a flying control system to.

A. automatically compensate for low cable tension caused by worn cables.

B. compensate for rapid movement of the controls by taking up the slack.

C. allow for variations in temperature which will vary the cable tension.

Ref EASA Module 6 - Materials and Hardware/ Control Cables

6. How are changes in direction of a control cable accomplished?

A. Bell cranks.

B. Fairleads.

C. Pulleys.

Ref EASA Module 6 - Materials and Hardware/ Control Cables

Sub-Module 6.11: Electrical Cables and Connectors

1. A cable loom should be protected by conduit when fed through the.

  - Main equipment center.

  - Wheel wells

  - Cargo compartment ceiling

Ref EASA Module 6 - Materials and Hardware/ Electrical Cables and Connectors

2. Why are copper wires used in electrical systems?

  - They have high permeability.

  - They do not give off a magnetic field.

  - Copper has a higher conductivity

Ref EASA Module 6 - Materials and Hardware/ Electrical Cables and Connectors

3. The main advantage of Coaxial cables is

A. they are shielded against electrostatic and magnetic fields and they will not pick up any energy, or be influenced by other strong fields.

B. The outer sheathing is damage proof

C. They do not require secure cable clamping.

Ref EASA Module 6 - Materials and Hardware/ Electrical Cables and Connectors

MODULE 7 - MAINTENANCE PRACTICES

Sub-Module 7.1: Safety Precautions - Aircraft and Workshop

1. A dry powder extinguisher is colored.

A. Green.

B. Red.

C. Blue.

Ref EASA Module 7A - Maintenance Practices/ Safety Precautions - Aircraft and Workshop

2. Which type of extinguisher can be used for an electric fire?

A. Foam.

B. Water.

C. CO2.

Ref EASA Module 7A - Maintenance Practices/ Safety Precautions - Aircraft and Workshop

3. If welding in the hangar is necessary, no aircraft are to be:

A. within 5 meters of the welding operation

B. within 10 meters of the welding operation

C. within 15 meters of the welding operation

Ref EASA Module 7A - Maintenance Practices/ Safety Precautions - Aircraft and Workshop

4. The colour of CO2 type fire extinguisher is.

A. Red.

B. Black.

C. Green.

Ref EASA Module 7A - Maintenance Practices/ Safety Precautions - Aircraft and Workshop

5. Which type of extinguisher can be used for engine fire?

A. CO2.

B. BCF.

C. Water.

Ref EASA Module 7A - Maintenance Practices/ Safety Precautions - Aircraft and Workshop

6. Fire on landing gear brake should be extinguished with.

A. Dry powder extinguisher.

B. Carbon dioxide extinguisher.

C. Water extinguisher.

Ref EASA Module 7A - Maintenance Practices/ Safety Precautions - Aircraft and Workshop

7. The most appropriate fire extinguisher for an aircraft wheel and brake fire would be.

A. Carbon dioxide.

B. Dry powder.

C. Water.

Ref EASA Module 7A - Maintenance Practices/ Safety Precautions - Aircraft and Workshop

8. Neither oil nor grease should be used as a lubricant on couplings or pipelines carrying.

A. Oxygen.

B. Kerosene.

C. Nitrogen.

Ref EASA Module 7A - Maintenance Practices/ Safety Precautions - Aircraft and Workshop

9. All oxygen lines should be marked and should have

A. at least 5 cm of clearance from moving parts, electrical wiring, and all fluid lines.

B. at least 10 cm of clearance from moving parts, electrical wiring, and all fluid lines

C. at least 15 cm of clearance from moving parts, electrical wiring, and all fluid lines

Ref EASA Module 7A - Maintenance Practices/ Safety Precautions - Aircraft and Workshop

10. The colour of vaporizing liquids type fire extinguisher is.

A. Red.

B. Black.

C. Green.

Ref EASA Module 7A - Maintenance Practices/ Safety Precautions - Aircraft and Workshop

Sub-Module 7.2: Workshop Practices

1. When using a reamer.

A. Use no lubricant.

B. Use the same lubricant as was used on the drill bit.

C. Use lard oil.

Ref EASA Module 7A - Maintenance Practices/ Workshop Practices

2. The difference between high and low limits of a size for a dimension is known as the.

A. Deviation.

B. Tolerance.

C. Fit.

Ref EASA Module 7A - Maintenance Practices/ Workshop Practices

3. Vices should be wiped over frequently

A. with an oily rag.

B. with an dry rag

C. with a brush

Ref EASA Module 7A - Maintenance Practices/ Workshop Practices

4. Metal covered benches should have its surfaces cleaned

A. with a kerosene rag.

B. with a dry rag

C. with a soft brush

Ref EASA Module 7A - Maintenance Practices/ Workshop Practices

5. Plain wooden benches should be cleaned

A. with a soft brush

B. with a dry rag

C. with a stiff brush

Ref EASA Module 7A - Maintenance Practices/ Workshop Practices

6. Some materials are flammable must be stored

A. outdoors or in specially designed cabinets

B. in a vented room

C. in a room far from hangar.

Ref EASA Module 7A - Maintenance Practices/ Workshop Practices

7. Tool calibration for tool takes place per:

A. the standard specified by the equipment manufacturer, design organization

B. The maintenance organization

C. requirement of authority

Ref EASA Module 7A - Maintenance Practices/ Workshop Practices

Sub-Module 7.3: Tools

1. When using a moving coil as an ammeter the greatest amount of current flows through the.

A. Bushes.

B. Coil.

C. Shunt.

Ref EASA Module 7A - Maintenance Practices/ Tools

2. How are files classified?

A. By length, grade and material

B. By length, grade, cut and section

C. By length, grade, cut, section and material.

Ref EASA Module 7A - Maintenance Practices/ Tools

3. One megohm is equal to.

A. 1,000 ohms.

B. 1,000,000 ohms.

C. 100,000 ohms.

Ref EASA Module 7A - Maintenance Practices/ Tools

4. The test equipment normally used to carry out a continuity test on an electrical cable is.

A. A high tension circuit tester.

B. An ammeter.

C. A low reading ohmmeter.

Ref EASA Module 7A - Maintenance Practices/ Tools

5. A voltage drop across a component is measured by placing the meter in.

A. Parallel with the component.

B. Series with the component.

C. Series with the power source.

Ref EASA Module 7A - Maintenance Practices/ Tools

6. On barrel of micrometer, each of the sections between the tenths divisions (between 1, 2, 3, 4, and so forth) is divided into:

A. four parts of 0.025 inch each.

A. four parts of 0.05 inch each.

A. four parts of 0.02 inch each.

Ref EASA Module 7A - Maintenance Practices/ Tools

7. When torque loading, a wrench should be selected where the required value falls.

A. At the top end of the range.

B. In the middle of the range.

C. At the bottom end of the range

Ref EASA Module 7A - Maintenance Practices/ Tools

8. Torque loading is determined by multiplying the tangent force applied at the free end of the spanner.

A. By the dia. Of the bolt and the distance of its point of application.

B. By the distance moved by the point of application.

C. By its distance of application from the axis of the bolt.

Ref EASA Module 7A - Maintenance Practices/ Tools

9. When tightening a nut on a bolt the torque loading applied is

A. independent of whether the threads are wet or dry

B. inversely proportional to the force applied to the spanner

C. the tangential application of the force times the perpendicular distance from the point of application to the centre of the bolt.

Ref EASA Module 7A - Maintenance Practices/ Tools

10. How are taper reamers classified?

A. Taper and diameter of small end

B. Length and diameter of large end

C. Length, diameter of large end, and diameter of small end

Ref EASA Module 7A - Maintenance Practices/ Tools

11. The purpose of a taper tap is to.

A. Produce a fine thread.

B. Start a thread.

C. Form a tapered thread.

Ref EASA Module 7A - Maintenance Practices/ Tools

12. Hammers are classified by

A. shape of head and length of shaft

B. weight and length of shaft

C. weight and type of head

Ref EASA Module 7A - Maintenance Practices/ Tools

13. When measuring current in a circuit, the ammeter is placed.

A. In series with the circuit.

B. In series with the shunt.

C. In parallel with the circuit.

Ref EASA Module 7A - Maintenance Practices/ Tools

14. Scribers are used to

A. mark guide lines on material

B. produce an accurate finish

C. make centre pop marks for drilling

Ref EASA Module 7A - Maintenance Practices/ Tools

15. Which electrical measuring device needs a power source?

A. A voltmeter.

B. An ohmmeter.

C. An ammeter.

Ref EASA Module 7A - Maintenance Practices/ Tools

16. The main scale of a metric vernier caliper is calibrated in.

A. Millimetres.

B. Micro-meters.

C. Millimetres and half millimetres.

Ref EASA Module 7A - Maintenance Practices/ Tools

17. A multiplier resistance is used to extend the working range of the.

A. Ammeter.

B. Voltmeter.

C. Ohmmeter.

Ref EASA Module 7A - Maintenance Practices/ Tools

18. An AC voltmeter reads.

A. Peak value.

B. Peak to peak value.

C. Average value.

Ref EASA Module 7A - Maintenance Practices/ Tools

19. To measure ac with a moving coil, it.

A. Can be directly connected.

B. Is rectified and measures peak value.

C. Is rectified and measures average value.

Ref EASA Module 7A - Maintenance Practices/ Tools

20. When using a reamer, in which direction should it be turned?

A. Anti-clockwise when Cutting and removing.

B. Clockwise when cutting and anti-clockwise when removing.

C. Clockwise when cutting and removing.

Ref EASA Module 7A - Maintenance Practices/ Tools

21. When installing a blade in a hacksaw frame, mount the blade with:

A. the teeth pointing forward

B. the teeth pointing backward

C. both of direction are allowed

Ref EASA Module 7A - Maintenance Practices/ Tools

22. Crossfiling is a method used for:

A. filing the edges of metal parts that must fit tightly together.

B. filling the larger surfaces need to be smoothed and squared

C. filling the vertical holes

Ref EASA Module 7A - Maintenance Practices/ Tools

23. Triangular and three square files are used for:

A. draw filing and to some extent for filing soft metals

B. filing slots and key seats, and for surface filing

C. filing internal angles, clearing out corners, and filing taps and cutters

Ref EASA Module 7A - Maintenance Practices/ Tools

24. Burring tools are used to:

A. remove a burr from round hole

B. remove a burr from an edge of a sheet or to deburr a hole

C. remove a burr from the metal surface

Ref EASA Module 7A - Maintenance Practices/ Tools

25. Reamers are used to

A. smooth and enlarge holes to exact size.

B. make a new thread

C. remove a stuck bolt in the hole.

Ref EASA Module 7A - Maintenance Practices/ Tools

26. A hole that is to be reamed to exact size must be drilled about

A. 0.005 to 0.007 inch under size.

A. 0.002 to 0.006 inch under size.

C. 0.003 to 0.007 inch under size.

Ref EASA Module 7A - Maintenance Practices/ Tools

27. A tap is used to

A. cut threads on the inside of a hole

B. remove a stuck bolt in the hole

C. cut the external threads on round stock.

Ref EASA Module 7A - Maintenance Practices/ Tools

Sub-Module 7.5: Engineering Drawings, Diagrams and Standards

1. What is third angle projection?

A. each view represents the side of the object furthest from the adjacent view.

B. each view represents the side of the object nearest to it in the adjacent view.

C. each view is at an angle of 30 degrees to the plane of projection.

Ref EASA Module 7A - Maintenance Practices/ Engineering Drawings, Diagrams and Standards.

2. Which pictorial projection shows one face in true elevation and line of depth normally draw at 30° or 45° to the horizontal?

A. Oblique.

B. Perspective.

C. Isometric.

Ref EASA Module 7A - Maintenance Practices/ Engineering Drawings, Diagrams and Standards.

3. If a design amendment is made on a drawing.

A. a new issue number and date must be allocated to the drawing.

B. the old issue number is retained, with the amendment date added.

C. no change in issue number or date is necessary.

Ref EASA Module 7A - Maintenance Practices/ Engineering Drawings, Diagrams and Standards.

4. The scale of an engineering drawing is shown as 1:4. This indicates it is.

A. drawn to a quarter.

B. drawn to scale.

C. drawn four times larger.

Ref EASA Module 7A - Maintenance Practices/ Engineering Drawings, Diagrams and Standards.

5. An orthographic projection usually shows.

A. one, three-dimensional view of an object.

B. a pictorial view of the object.

C. three, two-dimensional views of an object.

Ref EASA Module 7A - Maintenance Practices/ Engineering Drawings, Diagrams and Standards.

6. 10:1 on an engineering drawing indicates.

A. the drawing is full size.

B. the drawing is one tenth full size.

C. the drawing is ten times full size.

Ref EASA Module 7A - Maintenance Practices/ Engineering Drawings, Diagrams and Standards.

7. Lines known as short dashes (thin) are used on drawings to indicate.

A. hidden detail.

B. visible outlines.

C. cutting revolved.

Ref EASA Module 7A - Maintenance Practices/ Engineering Drawings, Diagrams and Standards.

8. Drawing numbers are.

A. the same as serial numbers.

B. changed after each drawing amended after May 28, 1999.

C. unique to each drawing.

Ref EASA Module 7A - Maintenance Practices/ Engineering Drawings, Diagrams and Standards.

9. Break lines are used.

A. to show where components are expected to break.

B. in sectional drawing.

C. where it would be inconvenient (because of limited space) to draw long lengths of the same section.

Ref EASA Module 7A - Maintenance Practices/ Engineering Drawings, Diagrams and Standards.

10. An oblique projection.

A. is the same as an isometric projection.

B. has one view looking directly at one face with the lines representing depth drawn at 90°.

C. has one view looking directly at one face with the lines representing depth drawn at a constant angle.Ref EASA Module 7A - Maintenance Practices/ Engineering Drawings, Diagrams and Standards.

11. A drawing in which the subassemblies or parts are shown as brought together on the aircraft is called.

A. an installation drawing.

B. a detail drawing.

C. a sectional drawing.

Ref EASA Module 7A - Maintenance Practices/ Engineering Drawings, Diagrams and Standards.

12.  A line used to show an edge which is not visible is a.

A. break line.

B. phantom line.

C. hidden line.

Ref EASA Module 7A - Maintenance Practices/ Engineering Drawings, Diagrams and Standards.

13. One purpose for schematic diagrams is to show the.

A. size and shape of components within a system.

B. functional location of components within a system.

C. physical location of components within a system.

Ref EASA Module 7A - Maintenance Practices/ Engineering Drawings, Diagrams and Standards.

14. In a first angle orthographic projection the plan view is placed.

A. above the front elevation.

B. below the side elevation.

C. below the front elevation.

Ref EASA Module 7A - Maintenance Practices/ Engineering Drawings, Diagrams and Standards.

Sub-Module 7.6: Fits and Clearances

1. Which of the following is a 'Bilateral Tolerance’?

A. 2 inches -0.002.

B. 2 inches +0.002.

C. 2 inches ±0.002.

Ref EASA Module 7A - Maintenance Practices/ Fits and Clearances

2. Which of the following is checked when using a 'GO / NO -GO' gauge?

A. Clearance.

B. Tolerance.

C. Allowance.

Ref EASA Module 7A - Maintenance Practices/ Fits and Clearances

3. Which of the following shafts would you use to obtain a clearance fit in a bush 0.750 inch diameter?

A. 0.752 inch.

B. 0.748 inch.

C. 750 inch.

Ref EASA Module 7A - Maintenance Practices/ Fits and Clearances

4. What is the maximum bow allowed in a strut?

A. 1 in 200.

B. 1 in 500.

C. 1 in 600.

Ref EASA Module 7A - Maintenance Practices/ Fits and Clearances

5. A tolerance is.

A. a permitted difference between new and worn dimensions.

B. a permitted variation on a dimension to allow for inaccuracy of equipment.

C. a required difference in dimension between mating parts to obtain a certain class of fit.

Ref EASA Module 7A - Maintenance Practices/ Fits and Clearances

6. A transition fit is one in which the shaft is.

A. larger than the hole.

B. smaller than the hole.

C. the same size as the hole.

Ref EASA Module 7A - Maintenance Practices/ Fits and Clearances

7. If there is a positive allowance between the smallest possible hole and the largest possible shaft, the fit is known as.

A. a transition fit.

B. a clearance fit.

C. an interference fit.

Ref EASA Module 7A - Maintenance Practices/ Fits and Clearances

8. The British Standards System provides for:

A. 21 types of holes designated by small letters a, b, c, d, etc., and 21 types of shaft designated by capital letters: A, B, C, D, etc.,

B. 21 types of holes designated by capital letters A, B, C, D, etc., and 21 types of shaft designated by small letters: a, b, c, d, etc.,

C. 25 types of holes designated by capital letters A, B, C, D, etc., and 25 types of shaft designated by small letters: a, b, c, d, etc.,

Ref EASA Module 7A - Maintenance Practices/ Fits and Clearances

9. Which of the following is a “Unilateral Tolerance”?

A. 5.325 inches +0.002 /-0.002 inches

B. 5.325 inches +0.1/-0.002 inches

C. 5.325 inches +0.00/-0.002 inches

Ref EASA Module 7A - Maintenance Practices/ Fits and Clearances

10. Bow in a shaft can be determined in a workshop by utilizing

A. T blocks, a surface gauge and a dial indicator.

B. V blocks, a surface gauge and a dial indicator.

C. H blocks, a surface gauge and a dial indicator.

Ref EASA Module 7A - Maintenance Practices/ Fits and Clearances

Sub-Module 7.7: Electrical Wiring Interconnection System (EWIS)

1. A wire clamped vertically at one end and horizontally at the other end should have a bend radius of no less than.

A. 3 times the diameter of the wire.

B. 5 times the diameter of the wire.

C. 10 times the diameter of the wire.

Ref EASA Module 7A - Maintenance Practices/ Electrical Wiring Interconnection System (EWIS)

2. What gauge of pin would a yellow plastic insert/extract tool be used on?

A. 16 - 14.

B. 12 - 10.

C. 22 - 18.

Ref EASA Module 7A - Maintenance Practices/ Electrical Wiring Interconnection System (EWIS)

3. What gauge of pin would a red plastic insert/extract tool be used on?

A. 12 - 10.

B. 22 - 18.

C. 16 - 14.

Ref EASA Module 7A - Maintenance Practices/ Electrical Wiring Interconnection System (EWIS)

4. What gauge of pin would a blue plastic insert / extract tool be used on?

A. 12 - 10

B. 16 - 14

C. 22 - 18

Ref EASA Module 7A - Maintenance Practices/ Electrical Wiring Interconnection System (EWIS)

5. To find a high resistance or open circuit, carry out.

A. a milli-volt drop test.

B. a continuity check.

C. a bonding check.

Ref EASA Module 7A - Maintenance Practices/ Electrical Wiring Interconnection System (EWIS)

6. The maximum bonding resistance on an aircraft primary structure should normally not exceed

A. 0.003 ohm

B. 0.01 ohms.

C. 0.001 ohms.

Ref EASA Module 7A - Maintenance Practices/ Electrical Wiring Interconnection System (EWIS)

7. A hole is placed in the lowest point of electrical cable conduit.

A. to allow for pull-through of the cables.

B. to secure the conduit to a piece of aircraft structure.

C. to allow for drainage of moisture.

Ref EASA Module 7A - Maintenance Practices/ Electrical Wiring Interconnection System (EWIS)

8. Splices should not be used within

A. 30 cm of a termination device.

B. 3 inches of a termination device

C. 0.5 feet of a termination device

Ref EASA Module 7A - Maintenance Practices/ Electrical Wiring Interconnection System (EWIS)

9. Forward release electrical connectors are removed by the wire being.

A. pushed forwards.

B. twisted to the right.

C. pulled backwards.

Ref EASA Module 7A - Maintenance Practices/ Electrical Wiring Interconnection System (EWIS)

10. Why are copper wires used in electrical systems?

A. They have high permeability.

B. They do not give off a magnetic field.

C. They have a low resistance to current.

Ref EASA Module 7A - Maintenance Practices/ Electrical Wiring Interconnection System (EWIS)

11. When crimping wires, the wire should.

A. be flush with the crimp.

B. extend 0.8 mm beyond the crimp.

C. be beneath the crimp.

Ref EASA Module 7A - Maintenance Practices/ Electrical Wiring Interconnection System (EWIS)

12. A white/blue insertion-extraction tool would be used on a cable of what size

A. 10

B. 22

C. 16

Ref EASA Module 7A - Maintenance Practices/ Electrical Wiring Interconnection System (EWIS)

13. An in-line splice should be positioned on the.

A. terminal of the loom.

B. outside of the loom.

C. outer surface of the loom for easy inspection.

Ref EASA Module 7A - Maintenance Practices/ Electrical Wiring Interconnection System (EWIS)

14. A cable loom should be protected by conduit when fed through the.

A. main equipment centre.

B. wheel well door.

C. cargo compartment ceiling.

Ref EASA Module 7A - Maintenance Practices/ Electrical Wiring Interconnection System (EWIS)

15. The size of proper conduit for electrical wires must be.

A. 75% larger than the maximum diameter of wires.

B. 25% larger than the maximum diameter of wires.

C. 100% larger than the average diameter of wires.

Ref EASA Module 7A - Maintenance Practices/ Electrical Wiring Interconnection System (EWIS)

16. Blue metal extract tool would be used with contacts sized.

A. 16.

B. 12.

C. 22.

Ref EASA Module 7A - Maintenance Practices/ Electrical Wiring Interconnection System (EWIS)

17. Dielectric strength is:

A. the ability of the insulator to withstand current difference

B. the ability of the insulator to withstand potential difference

C. the ability of the insulator to withstand resistance difference

Ref EASA Module 7A - Maintenance Practices/ Electrical Wiring Interconnection System (EWIS)

18. Maximum dielectric strength values can be measured by:

A. raising the current of a test sample until the insulation breaks down

B. raising the frequency of a test sample until the insulation breaks down

C. raising the voltage of a test sample until the insulation breaks down

Ref EASA Module 7A - Maintenance Practices/ Electrical Wiring Interconnection System (EWIS)

19. In hightemperature, applications, the terminal temperature rating must be:

A. lower than the ambient temperature plus current related temperature rise.

B. greater than the ambient temperature plus current related temperature rise.

C. equal to the ambient temperature plus current related temperature rise.

Ref EASA Module 7A - Maintenance Practices/ Electrical Wiring Interconnection System (EWIS)

20. Hydraulic crimping tools are used for crimping:

A. terminal wire sizes 4, 6, 12 and 16

B. terminal wire sizes 8, 6, 4, and 2

C. terminal wire sizes 6, 8, 12 and 14

Ref EASA Module 7A - Maintenance Practices/ Electrical Wiring Interconnection System (EWIS)

21. Identification markings should be placed at each end of the wire and at:

A. 38 inchs maximum intervals along the length of the wire.

B. 38 cm maximum intervals along the length of the wire.

C. 3.8 feet maximum intervals along the length of the wire.

Ref EASA Module 7A - Maintenance Practices/ Electrical Wiring Interconnection System (EWIS)

Sub-Module 7.8: Riveting

1. When riveting, the distance from the edge to the rivet (land) should not be less than.

A. 1D

B. 2D

C. 4D

Ref EASA Module 7A - Maintenance Practices/ Riveting

2. The average rivet pitch usually ranges:

A. from 4 to 6 rivet diameters

B. from 6 to 8 rivet diameters

C. from 8 to 10 rivet diameters

Ref EASA Module 7A - Maintenance Practices/ Riveting

3. Transverse pitch is usually:

A. 65 percent of the rivet pitch

B. 75 percent of the rivet pitch

C. 85 percent of the rivet pitch

Ref EASA Module 7A - Maintenance Practices/ Riveting

4. The smallest allowable transverse pitch is

A. 2 1/2 rivet diameters.

B. 3 1/2 rivet diameters

C. 4 1/2 rivet diameters

Ref EASA Module 7A - Maintenance Practices/ Riveting

5. The minimum spacing between flush head rivets

A. shall not be less than 2 times the diameter of the rivet.

B. shall not be less than 4 times the diameter of the rivet.

C. shall not be less than 6 times the diameter of the rivet.

Ref EASA Module 7A - Maintenance Practices/ Riveting

6. The minimum edge distance for universal rivets is

A. 1 times the diameter of the rivet

B. 2 times the diameter of the rivet

C. 3 times the diameter of the rivet

Ref EASA Module 7A - Maintenance Practices/ Riveting

7. The countersunk head angle can vary

A. from 40° to 80°

B. from 60° to 120°

C. from 80° to 140°

Ref EASA Module 7A - Maintenance Practices/ Riveting

8. Rivets most commonly chosen in the assembly and repair of aircraft range:

A. from 1/32 inch to 1/8 inch in diameter.

B. from 3/16 inch to 3/16 inch in diameter

C. from 3/32 inch to 3/8 inch in diameter

Ref EASA Module 7A - Maintenance Practices/ Riveting

Sub-Module 7.9: Pipes and Hoses

1. Metal tubing is sized by:

A. outside diameter (o.d.)

B. inside diameter (i.d)

C. circumference of the tube

Ref EASA Module 7A - Maintenance Practices/ Pipes and Hoses

2. The wall thickness is typically printed on the tubing in:

A. Tens of an inch

B. Hundreds of an inch

C. Thousands of an inch

Ref EASA Module 7A - Maintenance Practices/ Pipes and Hoses

3. A number 10 piece of tubing with a wall thickness of 0.063" has an inside diameter of:

A. 0.399"

B. 0.499"

C. 0.599"

Ref EASA Module 7A - Maintenance Practices/ Pipes and Hoses

4. Tube forming consists of four processes:

A. Bending, flaring, beading and cutting

B. Beading, bending, cutting, and flaring.

C. Cutting, bending, flaring, and beading.

Ref EASA Module 7A - Maintenance Practices/ Pipes and Hoses

5. The assembly can be formed by hand bending during installation, if:

A. the tube diameter  is 1⁄2"

B. the tube diameter  is 1⁄4"

C. the tube diameter is 1⁄6"

Ref EASA Module 7A - Maintenance Practices/ Pipes and Hoses

Sub-Module 7.10: Springs

1. A wear check on a cylinder head valve spring should include.

A. length under load.

B. diameter and length.

C. length off-load.

Ref EASA Module 7A - Maintenance Practices/ Springs

2. Overheating is evidenced as:

A. Blistering of the surface protection

B. A change of color of the metal due to the loss of temper.

C. Blistering of the surface protection and/ or a change of color of the metal due to the loss of temper.

Ref EASA Module 7A - Maintenance Practices/ Springs

3. Springs are manufactured from.

A. high carbon alloy steel with high strength requirements.

B. low carbon alloy steel with high strength requirements.

C. high carbon alloy steel with low strength requirements.

Ref EASA Module 7A - Maintenance Practices/ Springs

Sub-Module 7.11: Bearings

1. Needle roller bearings.

A. are susceptible to brinelling.

B. can accept a small amount of misalignment.

C. are designed to carry axial loads.

Ref EASA Module 7A - Maintenance Practices/ Bearings

2. When rotating a ball bearing by hand, a regular click indicates.

A. damage to the balls.

B. intergranular corrosion in the outer ring.

C. a cracked ring.

Ref EASA Module 7A - Maintenance Practices/ Bearings

3. Brinelling of a wheel bearing could be caused by.

A. rotation of the outer race in the wheel housing.

B. heavy landing.

C. overheating of the brakes.

Ref EASA Module 7A - Maintenance Practices/ Bearings

4. Brinelling of a bearing is.

A. bluing of the bearing due to overheating.

B. shallow smooth depressions caused by the rollers being forced against the cup, due to overtorquing.

C. indentations in the race surface caused by continual static vibration.

Ref EASA Module 7A - Maintenance Practices/ Bearings

5. A tapered roller bearing is designed to take.

A. radial loads only.

B. both radial and axial loads.

C. axial loads only.

Ref EASA Module 7A - Maintenance Practices/ Bearings

Sub-Module 7.12: Transmissions

1. An aircraft control chain is connected using.

A. nuts and bolts.

B. quick release pins.

C. a split link and spring clip.

Ref EASA Module 7A - Maintenance Practices/ Transmissions

2. The initial lubricant on a new chain.

A. must be replaced with grade altitude grease.

B. should be removed and replaced with the approved oil.

C. should not be removed.

Ref EASA Module 7A - Maintenance Practices/ Transmissions

3. If a control chain can be lifted clear of a tooth, it should be.

A. rejected as unserviceable.

B. removed and an elongation check carried out.

C. cleaned, re-tensioned and inspected after a period of time.

Ref EASA Module 7A - Maintenance Practices/ Transmissions

4. A typical percentage limit of elongation could be:

A. 2% when the clean and dry chain is loaded with the correct tension.

B. 2% when the grease-applied chain is loaded with the correct tension.

C. 5% when the clean and dry chain is loaded

Ref EASA Module 7A - Maintenance Practices/ Transmissions

5. Backlash is a type of wear associated with.

A. gears.

B. rivets.

C. bearings.

Ref EASA Module 7A - Maintenance Practices/ Transmissions

6. If corrosion is found on a chain.

A. replace the chain.

B. clean off the corrosion and if acceptable re-fit the chain.

C. lubricate the chain.

Ref EASA Module 7A - Maintenance Practices/ Transmissions

7. Pushpull tubes are generally made in short lengths

A. to save the space

B. to prevent vibration and bending under compression loads.

C. to install/remove easily

Ref EASA Module 7A - Maintenance Practices/ Transmissions

Sub-Module 7.13: Control Cables

1. A control cable that has been contaminated with acid should be.

A. cleaned.

B. rejected.

C. cleaned, inspected, and reinspected after a period of time.

Ref EASA Module 7A - Maintenance Practices/ Control Cables

2. Check the swaged fitting with:

A. a "go no-go" gauge

B. a tensionmeter

C. a vernier caliper

Ref EASA Module 7A - Maintenance Practices/ Control Cables

3. How would you inspect a cable for fraying?

A. Run your fingers the full length of the cable.

B. Operate the controls and feel for stiffness.

C. Run a rag the full length of the cable.

Ref EASA Module 7A - Maintenance Practices/ Control Cables

4. The check for a cable pulling out of a swaged fitting is by.

A. seeing that the cable is still past the safety holes in the swage.

B. looking for a shiny surface on the cable near the fitting.

C. seeing that there are no broken wires near the fitting.

Ref EASA Module 7A - Maintenance Practices/ Control Cables

5. A cable is replaced.

A. when a shiny portion is found.

B. when each strand is worn to limits.

C. if a chemical spillage is suspected.

Ref EASA Module 7A - Maintenance Practices/ Control Cables

6. The best way to check control cables for broken wires is to.

A. run a rag along the cable in both directions.

B. examine them visually.

C. increase the tension and check with a magnifying glass.

Ref EASA Module 7A - Maintenance Practices/ Control Cables

7. Examine cable routing at fairleads to ensure that deflection angles are:

A. no greater than 2° maximum.

B. no greater than 3° maximum.

C. no greater than 4° maximum.

Ref EASA Module 7A - Maintenance Practices/ Control Cables

8. A cable should be replaced when individual wires are worn greater than.

A. 40%.

B. 60%.

C. 20%.

Ref EASA Module 7A - Maintenance Practices/ Control Cables

Sub-Module 7.17: Aircraft handling and storage

1. When mooring an aircraft what type of rope should be used?

A. Nylon.

B. Fibre, tied tight due to stretch when wet.

C. Fibre, with some slack due to shrinkage when wet.

Ref EASA Module 7A - Maintenance Practices/ Aircraft handling and storage

2. Removal of ice by the use of deicing fluid on the aircraft, before flight.

A. must be 1 hour before flight to enable fluid to be cleaned from aircraft.

B. will provide sufficient prevention of ice formation until take off.

C. may remove ice for a period of time depending on the airfield conditions.

Ref EASA Module 7A - Maintenance Practices/ Aircraft handling and storage

3. Deicing fluid should be applied:

A. within 60 minutes of flight.

B. within 2 hours of flight

C. in the whole of flight

Ref EASA Module 7A - Maintenance Practices/ Aircraft handling and storage

4. When Ground Power is connected to aircraft, the generators are.

A. paralleled to supply.

B. paralleled to supply for ground starting only.

C. never paralleled.

Ref EASA Module 7A - Maintenance Practices/ Aircraft handling and storage

5. If ice and snow is found on the wings of an aircraft. Before flight:

A. the snow should be removed but ice can remain because it has no appreciable affect on the airflow.

B. all snow and ice must be removed.

C. ice should be removed but snow can remain because the airflow will remove it.

Ref EASA Module 7A - Maintenance Practices/ Aircraft handling and storage

6. When refueling an aircraft from a tanker, why are the aircraft and tanker bonded together?

A. To discharge static electricity from the aircraft to the tanker.

B. To maintain the aircraft and tanker at the same electrical potential.

C. To enable the aircraft re-fuel pumps to be operated from the tanker electrical supply.

Ref EASA Module 7A - Maintenance Practices/ Aircraft handling and storage

7. Immediately after the engine starts, check the oil pressure indicator. If oil pressure does not show within:

A. 15 seconds, stop the engine and determine the trouble.

B. 30 seconds, stop the engine and determine the trouble

C. 45 seconds, stop the engine and determine the trouble

Ref EASA Module 7A - Maintenance Practices/ Aircraft handling and storage

8. Do not fuel an aircraft when there is lightning within:

A. 5 km.

B. 8 km

C. 15 km

Ref EASA Module 7A - Maintenance Practices/ Aircraft handling and storage

Sub-Module 7.18: Disassemble, Inspection, Repair and Assemble Techniques

1. Torque loading is carried out to provide.

A. as tight a joint as possible.

B. sufficient clamping without over-stressing.

C. flexibility.

Ref EASA Module 7A - Maintenance Practices/ Disassemble, Inspection, Repair and Assemble Techniques

2. A metallic stiff nut.

A. cannot be torque loaded.

B. cannot be used in areas in excess of 250oC.

C. is pre lubricated and does not need lubricating.

Ref EASA Module 7A - Maintenance Practices/ Disassemble, Inspection, Repair and Assemble Techniques

3. Removal of corrosion from aluminium clad alloy is best done.

A. mechanically by buffing.

B. chemically by trichloroethylene.

C. chemically by sulphuric acid solution.

Ref EASA Module 7A - Maintenance Practices/ Disassemble, Inspection, Repair and Assemble Techniques

4. The treatment for stress corrosion is.

A. not the same as fatigue corrosion.

B. the same as for surface corrosion or surface cracks in sheet metal.

C. always the replacement of the part.

Ref EASA Module 7A - Maintenance Practices/ Disassemble, Inspection, Repair and Assemble Techniques

5. Very light corrosion on aluminium alloy can be removed by.

A. using a solvent.

B. rubbing with wire wool.

C. using Alocrom 1200.

Ref EASA Module 7A - Maintenance Practices/ Disassemble, Inspection, Repair and Assemble Techniques

6. To remove corrosion on Ferrous metals use.

A. selenious acid rust remover.

B. sulphuric acid rust remover.

C. phosphoric acid rust remover.

Ref EASA Module 7A - Maintenance Practices/ Disassemble, Inspection, Repair and Assemble Techniques

7. How is damage classified on an aircraft skin?

A. Negligible, repairable, replacement.

B. Negligible, allowable, replacement.

C. Negligible, allowable, repairable.

Ref EASA Module 7A - Maintenance Practices/ Disassemble, Inspection, Repair and Assemble Techniques

8. A jig is device that allows:

A. the manufacture, repair components to a high dimensional accuracy.

B. as a reference when checking the C of G position.

C. to support the structure during repairs.

Ref EASA Module 7A - Maintenance Practices/ Disassemble, Inspection, Repair and Assemble Techniques

9. The majority of nuts, bolts and setscrews, on an aircraft, are subject to a standard torque value. This depends on:

A. their material, finish, lubrication, thread type and size

B. their material, shape, lubrication

C. their material, shape, size

Ref EASA Module 7A - Maintenance Practices/ Disassemble, Inspection, Repair and Assemble Techniques

Sub-Module 7.19: Abnormal Events

1. When inspecting an aircraft after a lightning strike, you should observe.

A. entry damage.

B. all signs of burning.

C. entry and exit damage.

Ref EASA Module 7A - Maintenance Practices/ Abnormal Events

2. To ensure protection against HIRF affecting audio and navigation aids.

A. ensure that all audio and navigation equipment is adequately screened.

B. inspect and check all bonding leads to ensure their serviceability and replace if defective.

C. ensure that the correct number of static wicks are fitted.

Ref EASA Module 7A - Maintenance Practices/ Abnormal Events

3. Which of the following is a preventive process against HIRF?

A. Monitoring HIRF on the communication system.

B. Visual inspections.

C. Periodically checking aircraft bonding.

Ref EASA Module 7A - Maintenance Practices/ Abnormal Events

4. Skin wrinkling on the lower surface of a wing is caused by.

A. hogging.

B. tension.

C. sagging.

Ref EASA Module 7A - Maintenance Practices/ Abnormal Events

5. The most easily detected sign of an excessive load having been imposed during a landing is:

A. Deformed gear

B. Wrinkled wing skin

C. Cracked frame

Ref EASA Module 7A - Maintenance Practices/ Abnormal Events

6. A lightning strike on an aircraft would show.

A. the entry and exit point.

B. the entry point only.

C. the exit point only.

Ref EASA Module 7A - Maintenance Practices/ Abnormal Events

7. HIRF interference occurs when.

A. in use on mid frequencies.

B. in use on low frequencies.

C. in use on all frequencies

Ref EASA Module 7A - Maintenance Practices/ Abnormal Events

8. What is the primary purpose of bonding of metallic parts of an aircraft?

A. To provide a return path for electrical two-wire system.

B. To prevent high potential differences between metallic parts from building up.

C. To prevent lightening strikes.

Ref EASA Module 7A - Maintenance Practices/ Abnormal Events

9. Which of the following could be a primary cause of HIRF protection failure?

A. Corrosion on bonding leads.

B. Broken or missing static wicks.

C. Unserviceable radio filters.

Ref EASA Module 7A - Maintenance Practices/ Abnormal Events

10. What is the reason for a primary bonding connection’s large cross-sectional area?

A. To carry the static discharge current to the conducting nose wheel.

B. To carry lightening discharge current should the need arise.

C. To maintain the airframe at the same potential throughout.

Ref EASA Module 7A - Maintenance Practices/ Abnormal Events

Sub-Module 7.20: Maintenance Procedures

1. ATA specification 100 is.

A. the procedures which must be complied with before an aircraft can be given a Certificate of Airworthiness in the Transport Category (Passenger).

B. the International standardization of maintenance manuals, illustrated parts catalogues, overhaul and repair manuals, service bulletins and letters.

C. the American FAA specification controlling the manufacture of aluminium and its alloys.

Ref EASA Module 7A - Maintenance Practices/ Maintenance Procedures

2. Maintenance Schedules are issued.

A. in a folder with the operators name on the cover.

B. with an approval certificate by the CAA.

C. by the operator with CAA approval.

Ref EASA Module 7A - Maintenance Practices/ Maintenance Procedures

3. After a mandatory inspection has been carried out by a Licensed Engineer, what is issued.

A. a Certificate of Maintenance Review.

B. Certificate of Release to Service.

C. a Flight Release Certificate.

Ref EASA Module 7A - Maintenance Practices/ Maintenance Procedures

4. A Certificate of Release to Service must be issued after.

A. a repair has been carried out in accordance with an approved repair scheme.

B. a re-fuel has been done.

C. engine runs.

Ref EASA Module 7A - Maintenance Practices/ Maintenance Procedures

5. The purpose of the CRS is.

A. to ensure that the log book entry is complete.

B. to turn a log book or job card entry into a legal document and to ensure that the signatory takes full responsibility for the work done.

C. to comply with article 15 of the ANO which states that an aircraft must not fly unless it is properly equipped for the intended flight.

Ref EASA Module 7A - Maintenance Practices/ Maintenance Procedures

6. What work has to be recorded and signed for?

A. Only work which entails a duplicate inspection.

B. Only work involving replacements.

C. All work carried out.

Ref EASA Module 7A - Maintenance Practices/ Maintenance Procedures

7. Rubber components should be stored.

A. in a cool dark area.

B. in warm and humid conditions.

C. in a well lit room.

Ref EASA Module 7A - Maintenance Practices/ Maintenance Procedures

8. Storage of components to prevent corrosion is helped.

A. by using silica gel.

B. by wrapping in grease proof paper.

C. by placing them in a plastic box.

Ref EASA Module 7A - Maintenance Practices/ Maintenance Procedures

9. Dye penetrant kits should be stored.

A. out of sunlight in a dry place.

B. in direct sunlight to keep it dry.

C. in a dark damp cupboard.

Ref EASA Module 7A - Maintenance Practices/ Maintenance Procedures

MODULE 8 - BASIC AERODYNAMICS

Sub-Module 8.1: Physics of the atmosphere

1. If a pilot changes the altimeter setting from 30.11 in.Hg to 29.96 in.Hg, what will be the approximate change in indication?

A. Altimeter will indicate 150 feet lower

B. Altimeter will indicate 150 feet higher

C. Altimeter will indicate 0.15 in.Hg higher .

Ref EASA Module 8 - Basic Aerodynamics/ Physics of the atmosphere

2. Which is the ratio of the water vapour actually present in the atmosphere to the amount that would be present if the air were saturated at the prevailing temperature and pressure?

A. Dew point

B. Relative humidity

C. Absolute humidity

Ref EASA Module 8 - Basic Aerodynamics/ Physics of the atmosphere

3. The amount of water vapour capacity in the air (humidity holding capacity of the air) is

A. greater on a hotter day and lower on a colder day

B. doesn't have a significant difference

C. greater on a colder day, and lower on a hotter day

Ref EASA Module 8 - Basic Aerodynamics/ Physics of the atmosphere

4. The temperature lapse rate below the tropopause is

A. 1°C per 1000 ft

B. 2°C per 1000 ft

C. 3°C per 1000 ft

Ref EASA Module 8 - Basic Aerodynamics/ Physics of the atmosphere

5. What is absolute humidity?

A. The actual amount of the water vapour in a mixture of air and water

B. The temperature to which humid air must be cooled at constant pressure to become saturated.

C. The ratio of the water vapour actually present in the atmosphere to the amount that would be present if the air were saturated at the prevailing temperature and pressure

Ref EASA Module 8 - Basic Aerodynamics/ Physics of the atmosphere

6. The state of the atmosphere is characterized by some key variables, namely

A. pressure, temperature, and humidity

B. pressure, temperature, humidity and density

C. pressure, temperature and density

Ref EASA Module 8 - Basic Aerodynamics/ Physics of the atmosphere

7. In which of the following conditions will air density be highest?

A. Hot humid day at sea level

B. Cold dry day at sea level

C. Hot dry day at 500 ft

Ref EASA Module 8 - Basic Aerodynamics/ Physics of the atmosphere

8. The temperature of the lower stratosphere

- remains constant

- increases with altitude

- decreases with altitude

Ref EASA Module 8 - Basic Aerodynamics/ Physics of the atmosphere

9. Which condition is the actual amount of water vapour in a mixture of air and water?

- Absolute humidity

- Dew point

- Relative humidity

Refer: EASA Module 8 - Basic Aerodynamics/ Physics of the atmosphere

10. At sea level, ISA atmospheric pressure is

- 10 Bar

- 14.7 PSI

- 14.7 kPa

Refer: EASA Module 8 - Basic Aerodynamics/ Physics of the atmosphere

11. With an increase in altitude under I.S.A. conditions the temperature in the troposphere

- increases

- decreases

- remains constant

Refer: EASA Module 8 - Basic Aerodynamics/ Physics of the atmosphere

Sub-Module 8.2: Aerodynamics

1. If the IAS is increased by a factor of 4, by what factor would the drag increase?

- 4

- 16

- 8

Ref EASA Module 8 - Basic Aerodynamics/ Aerodynamics

2. When airflow velocity over an upper cambered surface of an aerofoil decreases, what takes place?

- Pressure increases, lift increases

- Pressure increases, lift decreases

- Pressure decreases, lift increases

Ref EASA Module 8 - Basic Aerodynamics/ Aerodynamics

3. The difference between the mean camber line and the chord line of an aerofoil is

- neither are straight

- they both may be curved

- one is always straight and the other may be straight

Ref EASA Module 8 - Basic Aerodynamics/ Aerodynamics

4. Subsonic flow over a cambered aerofoil at 4° angle of attack will cause

- an increase in speed and drop in pressure over the upper surface and a decrease in speed and a rise in pressure over the lower surface

- a decrease in speed and drop in pressure over the upper surface and a decrease in speed and a drop in pressure over the lower surface

- an increase in speed and drop in pressure over the upper surface and an increase in speed and a drop in pressure over the lower surface

Ref EASA Module 8 - Basic Aerodynamics/ Aerodynamics

5. The vertical component of the aerodynamic resultant is defined as

- lift

- total aerodynamic force

- drag

Ref EASA Module 8 - Basic Aerodynamics/ Aerodynamics

6. Which statement is correct about the Cl and angle of attack?

- For a symmetric aerofoil, if angle of attack = 0, Cl =0

- For an asymmetric aerofoil, if angle of attack = 0, Cl =0

- For a symmetric aerofoil, if angle of attack = 0, Cl is not equal to 0

Ref EASA Module 8 - Basic Aerodynamics/ Aerodynamics

7. What determines the coefficient of lift?

- The aircraft designer

- Aerofoil shape and angle of attack

- The amount of camber

Ref EASA Module 8 - Basic Aerodynamics/ Aerodynamics

8. Which of the following describes the 'Empennage'?

- Nose section of an aircraft, including the cockpit

- Tail section of the aircraft, including fin, rudder, tail plane and elevators

- The wings, including the ailerons

Ref EASA Module 8 - Basic Aerodynamics/ Aerodynamics

9. The induced drag of an aircraft

- increases with increasing speed

- decreases with increasing speed

- increases if aspect ratio is increased

Ref EASA Module 8 - Basic Aerodynamics/ Aerodynamics

10. Bernoulli’s equation states that

- static pressure equals stagnation pressure

- dynamic pressure equals stagnation pressure

- the sum of static pressure and dynamic pressure is equal to stagnation pressure

Ref EASA Module 8 - Basic Aerodynamics/ Aerodynamics

11. Induced drag can be reduced by the use of

- fairings at junctions between fuselage and wings

- high aspect ratio wings

- streamlining

Ref EASA Module 8 - Basic Aerodynamics/ Aerodynamics

12. On a symmetrical aerofoil the camber line is

- equal to the chord

- less than the chord

- greater than the chord

Ref EASA Module 8 - Basic Aerodynamics/ Aerodynamics

13. Which of the following can reduce induced drag?

- Smooth surfaces

- Winglets

- Streamlining

Ref EASA Module 8 - Basic Aerodynamics/ Aerodynamics

14. Which of the following statements about boundary layers is correct?

- The turbulent boundary layer is thinner than the laminar boundary layer

- The turbulent boundary layer gives a lower skin friction than the laminar boundary layer

- The turbulent boundary layer has more kinetic energy than the laminar boundary layer

Ref EASA Module 8 - Basic Aerodynamics/ Aerodynamics

15. The boundary layer of a body in a moving airstream is

- a layer of air over the surface where the airspeed is changing from free stream velocity to zero velocity

- a thin layer of air over the surface where the air is stationary

- a layer of air which is moving at free stream speed

Ref EASA Module 8 - Basic Aerodynamics/ Aerodynamics

16. The angle of attack of an aerofoil section is the angle between the

- underside of the wing surface and the mean airflow

- chord line and the relative airflow

- chord line and the centre line of the fuselage

Ref EASA Module 8 - Basic Aerodynamics/ Aerodynamics

17. A High Aspect Ratio wing is a wing with

- short span, long chord

- long span, long chord

- long span, short chord

Ref EASA Module 8 - Basic Aerodynamics/ Aerodynamics

18. Downwash, on a wing section during flight, occurs

- at the trailing edge only

- at both the leading edge and the trailing edge

- at the leading edge only

Ref EASA Module 8 - Basic Aerodynamics/ Aerodynamics

19. Lift is a function of

- velocity, wing shape, wing area and CL

- velocity, wing area, CL and density

- velocity, density, wing shape and lift coefficient

Ref EASA Module 8 - Basic Aerodynamics/ Aerodynamics

20. The Mean Aerodynamic Chord (MAC) for a given wing of any planform is

- the wing area divided by the wing span

- the chord of a rectangular wing with same moment and lift

- the average chord of the actual aeroplane

Ref EASA Module 8 - Basic Aerodynamics/ Aerodynamics

Sub-Module 8.3: Theory of Flight

1. 'Load factor' has the following meaning:

- The ratio of a specified load to the mass of the aircraft the former being expressed in terms of aerodynamic and inertia forces

- The ratio of a specified load to the mass of the aircraft, the former being expressed in terms of aerodynamic forces, inertia forces and ground reactions

- The ratio of a specified load to the weight of the aircraft, the former being expressed in terms of aerodynamic forces, inertia forces and ground reactions

Ref EASA Module 8 - Basic Aerodynamics/ Theory of Flight

2.  Which of the following are lift increasing device?

- Flaps only

- Slots only

- Flaps and slots

Ref EASA Module 8 - Basic Aerodynamics/ Theory of Flight

3. A boundary layer fence on a swept wing will

- improve the high speed characteristics

- increase the critical Mach Number

- improve the low speed characteristics

Ref EASA Module 8 - Basic Aerodynamics/ Theory of Flight

4. A deployed slat will

- increase the camber of the aerofoil and increase the effective angle of attack, suction peak so that Clmax is reached at higher angle of attack

- increase the boundary layer energy and increase the suction peak on the fixed part of the wing, so that the stall is postponed to higher angle of attack

- increase the boundary layer energy, move the suction peak from the fixed part of the wing to the slat, the stall is postponed to higher angle of attack

Ref EASA Module 8 - Basic Aerodynamics/ Theory of Flight

5. The type of flaps which increase both lift and area of the wing is a

- slotted flap

- plain flap

- Fowler flap

Ref EASA Module 8 - Basic Aerodynamics/ Theory of Flight

6. What happens to lift when flaps are lowered?

- Decreases

- Remains same

- Increases

Ref EASA Module 8 - Basic Aerodynamics/ Theory of Flight

7. In a gliding flight

- lift is greater than weight

- lift is less than weight

- lift is zero

Ref EASA Module 8 - Basic Aerodynamics/ Theory of Flight

8. A slotted flap will increase the CLmax by

- increasing only the camber of the aerofoil

- increasing the camber of the aerofoil and improving the boundary layer

- increasing the critical angle of attack

Ref EASA Module 8 - Basic Aerodynamics/ Theory of Flight

9. A stall warning device must be set to operate

- at a speed just above stalling speed

- at a speed just below stalling speed

- at the stalling speed

Ref EASA Module 8 - Basic Aerodynamics/ Theory of Flight

10. A stick shaker is intended to come into operation

- prior to the actual moment of aircraft stall

- at the actual moment of aircraft stall

- immediately after the aircraft is stalled

Ref EASA Module 8 - Basic Aerodynamics/ Theory of Flight

11. About which axis of the aircraft does a rolling motion take place?

- Vertical axis

- Longitudinal axis

- Lateral axis

Ref EASA Module 8 - Basic Aerodynamics/ Theory of Flight

12. After take-off the slats (when installed) are always retracted later than the flaps. Why?

- Because SLATS EXTENDED gives a large decrease in stall speed with relatively less drag

- Because SLATS EXTENDED provides a better view from the cockpit than FLAPS EXTENDED

- Because FLAPS EXTENDED gives a large increase in stall speed with relatively less drag

Ref EASA Module 8 - Basic Aerodynamics/ Theory of Flight

13. Aircraft flying in the transonic range most often utilize

- high wings

- sweptback wings

- advanced super-critical airfoils

Ref EASA Module 8 - Basic Aerodynamics/ Theory of Flight

14. An aeroplane has the following flap settings: 0°, 15°, 30° and 45°. Slats can be selected too. Which of the above selections will produce the greatest negative influence on the CL/CD ratio?

- Flaps from 15° to 30°

- Flaps from 30° to 45°

- Flaps from 0° to 15°

Ref EASA Module 8 - Basic Aerodynamics/ Theory of Flight

15. An aircraft banks into a turn. No change is made to the airspeed or angle of attack. What will happen?

- The aircraft enters a side slip and begins to lose altitude

- The aircraft turns with no loss of height

- The aircraft yaws and slows down

Ref EASA Module 8 - Basic Aerodynamics/ Theory of Flight

16. An aircraft is flying into the wind. In a given amount of time, it will

- fly with the same IAS as in still air, but travel less distance

- fly with a higher IAS than in still air, and travel further

- fly with a higher IAS than in still air, and travel less

Ref EASA Module 8 - Basic Aerodynamics/ Theory of Flight

17. An aircraft wing is said to have stalled when the

- boundary layer is completely turbulent

- speed of the aircraft falls below the stalling speed

- airflow separates from the upper surface

Ref EASA Module 8 - Basic Aerodynamics/ Theory of Flight

18. An increase in angle of attack (below the stalling angle of attack) increases lift because

- induced drag is reduced

- the vertical component of weight is reduced

- the lift coefficient increases

Ref EASA Module 8 - Basic Aerodynamics/ Theory of Flight

19. An increase of wing loading will

- increase the stall speed

- decrease the minimum gliding angle

- decrease take off speeds

Ref EASA Module 8 - Basic Aerodynamics/ Theory of Flight

Sub-Module 8.4: Flight Stability and Dynamics

1. The lateral axis is

- a straight line through the CG parallel to a line joining the wingtips

- a straight line through the CG from nose to tail

- a straight line through the CG at right angles to the longitudinal and lateral axis

Ref EASA Module 8 - Basic Aerodynamics/ Flight Stability and Dynamics

2. The lateral axis is also called the

- pitch axis

- normal axis

- roll axis

Ref EASA Module 8 - Basic Aerodynamics/ Flight Stability and Dynamics

3. Dihedral wings combat instability in

- side-slip

- pitch

- yaw

Ref EASA Module 8 - Basic Aerodynamics/ Flight Stability and Dynamics

4. An aircraft with neutral stability has its attitude changed due to a gust of wind, how will this effect it?

- It will return to its original attitude

- It will go further out of control continuously

- It will remain in its new attitude

Ref EASA Module 8 - Basic Aerodynamics/ Flight Stability and Dynamics

5. The directional axis

- acts through the C of G, perpendicular to the longitudinal axis and lateral axis.

- is perpendicular to the longitudinal axis

- is perpendicular to the horizontal axis

Ref EASA Module 8 - Basic Aerodynamics/ Flight Stability and Dynamics

6. If an aircraft returns to a position of equilibrium it is said to be

- positively stable

- neutrally stable

- negatively stable

Ref EASA Module 8 - Basic Aerodynamics/ Flight Stability and Dynamics

7. Which of the following is used to counteract yaw?

- Fin area

- Lateral dihedral

- Longitudinal dihedral

Ref EASA Module 8 - Basic Aerodynamics/ Flight Stability and Dynamics

8. Dutch roll is an oscillatory motion that is an interaction between

- longitudinal and directional stability

- lateral and directional stability

- longitudinal and lateral stability

Ref EASA Module 8 - Basic Aerodynamics/ Flight Stability and Dynamics

9. Lateral stability is about the

- vertical axis

- longitudinal axis

- lateral axis

Ref EASA Module 8 - Basic Aerodynamics/ Flight Stability and Dynamics

10. Dutch Roll is

- a type of slow roll

- primarily a pitching instability

- a combined rolling and yawing motion

Ref EASA Module 8 - Basic Aerodynamics/ Flight Stability and Dynamics

11. Movement of an aircraft about its vertical axis

- is yawing

- is pitching

- is rolling

Ref EASA Module 8 - Basic Aerodynamics/ Flight Stability and Dynamics

12. The pendulum effect on a high wing aircraft

- increases lateral stability

- decreases lateral stability

- has no effect on lateral stability

Ref EASA Module 8 - Basic Aerodynamics/ Flight Stability and Dynamics

13. Longitudinal stability is provided by the

- horizontal stabilizer

- vertical stabilizer

- mainplane

Ref EASA Module 8 - Basic Aerodynamics/ Flight Stability and Dynamics

14. The fin gives stability about which axis?

- Lateral axis

- Vertical axis

- Longitudinal axis

Ref EASA Module 8 - Basic Aerodynamics/ Flight Stability and Dynamics

15. Dihedral improves

- longitudinal stability

- directional stability

- lateral stability

Ref EASA Module 8 - Basic Aerodynamics/ Flight Stability and Dynamics

16. An aircraft, which is longitudinally stable, will tend to return to level flight after a movement in which axis?

- Roll

- Yaw

- Pitch

Ref EASA Module 8 - Basic Aerodynamics/ Flight Stability and Dynamics

17. Stability of an aircraft is

- the ability of the aircraft to rotate about an axis

- the tendency of the aircraft to stall at low airspeed

- the tendency of the aircraft to return to its original trimmed position after having been displaced

Ref EASA Module 8 - Basic Aerodynamics/ Flight Stability and Dynamics

18. A high wing position gives

- the same lateral stability as a low wing

- more lateral stability than a low wing

- less lateral stability than a low wing

Ref EASA Module 8 - Basic Aerodynamics/ Flight Stability and Dynamics

19. The three axes concerned with stability of an aircraft have

- longitudinal axis nose to tail, lateral axis at furthest span point, vertical axis through centre of pressure

- vertical axis through C of G. Lateral axis - wing tip to wing tip. Longitudinal axis - nose to tail but not through C of G

- longitudinal, lateral and vertical axis all passing through aircraft centre of gravity

Ref EASA Module 8 - Basic Aerodynamics/ Flight Stability and Dynamics

20. Yawing is a rotation around

- the vertical axis obtained by the elevator

- the lateral axis obtained by the rudder

- the vertical axis obtained by the rudder

Ref EASA Module 8 - Basic Aerodynamics/ Flight Stability and Dynamics

MODULE 9A - HUMAN FACTORS

Sub-Module 9.1: General

- The company's safety policy should be defined in:

A. Safety Management System (SMS) manual

B. The Maintenance (MM) manual

C.The Maintenance Organization Exposition (MOE)

Ref EASA Module 9A - Human Factors/ General

- The memory of an individual can be aided by:

A. Detailed checklist

B. Memory checking

C. Mind logging

Ref EASA Module 9A - Human Factors/ General

- If you are given a task and you are not clear of the contents then you should:

A. Consult the appropriate/relevant approved data

B. Ask someone who has done it before

C. Consult type course notes

Ref EASA Module 9A - Human Factors/ General

- Aircraft Troubleshooting are:

A. Rule based.

B. Skill and Experience based

C. Knowledge based

Ref EASA Module 9A - Human Factors/ General

- The famous Murphy's Law says:

A.If something can go wrong, it will.

B.If something can go wrong, we can stop it

C.If something can go wrong, we can blame others

Ref EASA Module 9A - Human Factors/ General

- The SHEL model of human factors takes into account

A.Software, Hardware, Efficiency and Liveware

B.Software, Hardware, Environment and Liveware

C.Software, Hardware, Environment and Location

Ref EASA Module 9A - Human Factors/ General

- Learning of a routine by repeated practice is known as:

A.Episodic memory

B.Motor programming

C.Cognitive learning

Ref EASA Module 9A - Human Factors/ General

- The impact of human factors and human error is most important to

A.Technicians and engineers

B.Planners and managers

C.It is equally important to technicians, engineers, planners and Managers

Ref EASA Module 9A - Human Factors/ General

Sub-Module 9.2: Human Performance and Limitation

- 70 - 80% of the total focusing ability of the human eye is carried out by the:

A. Iris.

B. Cornea.

C. Lens.

Ref EASA Module 9A - Human Factors/ Human Performance and Limitation

- The part of the eye controls the amount of light that is allowed to enter into the eye is called:

A. The pupil.

B. The iris.

C. The cornea.

Ref EASA Module 9A - Human Factors/ Human Performance and Limitation

- The type of memory is most susceptible to interference from external influences is:

A.Long term.

B.Ultra-short term.

C.Short term

Ref EASA Module 9A - Human Factors/ Human Performance and Limitation

- The Extreme discomfort experienced by a maintenance engineer due to working in a confined space is known as:

A. Claustrophobia (Fear of confined spaces)

B. Acrophobia (Fear of heights)

C. Agoraphobia (Fear of leaving the place)

Ref EASA Module 9A - Human Factors/ Human Performance and Limitation

- The 'working memory' is consider as:

A.Long term memory.

B. Short term memory.

C. Ultra-short-term memory.

Ref EASA Module 9A - Human Factors/ Human Performance and Limitation

- The Long-term memory capacity is usually:

A. An unlimited memory.

B. Last for 4 - 8 years.

C. Last for 12 months.

Ref EASA Module 9A - Human Factors/ Human Performance and Limitation

- Human error can be caused by:

A. High body temperature.

B. Normal body temperature.

C.Low body temperature.

Ref EASA Module 9A - Human Factors/ Human Performance and Limitation

- If a maintenance engineer has a cold or flu, He/ She should:

A. Only absent himself from duty if his work contract includes sickness pay.

B. Only absent himself from duty if there are no staff shortages at his workplace or within his work team.

C. Absent himself from duty until fully recovered, regardless of other factors.

Ref EASA Module 9A - Human Factors/ Human Performance and Limitation

- When working with bright lights consideration should be given to.

A. Blurred image.

B. Shadows.

C. Glare.

Ref EASA Module 9A - Human Factors/ Human Performance and Limitation

- When working in different noise levels the Human Performance:

A. Has no effect on the number of errors or the speed of performance of an individual.

B. An individual can find noise levels annoying but still perform well indefinitely.

C. Is directly proportional to the number of errors and the speed of performance of an individual.

Ref EASA Module 9A - Human Factors/ Human Performance and Limitation

Sub-Module 9.3: Social Phsychology

- A regular behaviour of an individual is known as:

A. The norm.

B. Habit.

C. Culture.

Ref EASA Module 9A - Human Factors/ Social Phsychology

- An engineer who has developed his own way of performing a task:

A. Is in regular violation punishable by law.

B. Is improving maintenance efficiency.

C.Should be commended for his resourcefulness.

Ref EASA Module 9A - Human Factors/ Social Phsychology

- The difference between what a person 'can' do and what he 'will' do is largely determined by his:

A. Motivation.

B. Education and training.

C. Physical and mental health.

Ref EASA Module 9A - Human Factors/ Social Phsychology

- The Motivation can be defined as:

A. A reward or punishment designed to reduce errors.

B. A reward or punishment designed to increase work rate.

C. The thing that drives someone to do something all the times.

Ref EASA Module 9A - Human Factors/ Social Phsychology

- An effective group work is the one when:

A. Everyone contributes in some way.

B. Everyone discusses ideas and opinions.

C. Everyone has a job to do.

Ref EASA Module 9A - Human Factors/ Social Phsychology

- A Successful working teams have the following attributes:

A. All the Members participate in team activities but retain their own individual requirements.

B. All the members participate in team activities and discussions.

C. All the team members work in isolation and therefore provide highly individual solutions to the same problems.

Ref EASA Module 9A - Human Factors/ Social Phsychology

- Long shift work will.

A.Initially decrease your diagnostic and maintenance ability but eventually increase your diagnostic and maintenance ability as you get used to it.

B. Always decrease your diagnostic and maintenance ability.

C. Always increase your diagnostic and maintenance ability.

Ref EASA Module 9A - Human Factors/ Social Phsychology

- Group polarization can result in:

A.A group making a more cautious or extreme decision

B.A group making a better decision than the individual

C.Members of the group not talking to one another

Ref EASA Module 9A - Human Factors/ Social Phsychology

- In a discussion, the person who is most likely to be agreed with is.

A.The person who repeats the point most times.

B.The most argumentative person.

C.The person with the highest status.

Ref EASA Module 9A - Human Factors/ Social Phsychology

- The aim of human factors programs within an organization is.

A.to safeguard the health and safety of maintenance personnel by reducing accidents in the workplace.

B.To optimize the relationship between maintenance personnel and systems with a view to improving safety, efficiency and well-being.

C.To reduce costs by increasing efficiency, safety and quality and decreasing waste through human error.

Ref EASA Module 9A - Human Factors/ Social Phsychology

Sub-Module 9.4: Factors Affecting performances

- Consumption of alcohol before/during work will:

A. Increases mental and physical reaction times.(Slow Performance)

B. Has no affect upon mental and physical reaction times. (Normal Performance)

C.Decreases mental and physical reaction times. (Fast Performance)

Ref EASA Module 9A - Human Factors/ Factors Affecting performances

- The only permitted stimulant when working is:

A. Bromine.

B. Caffeine.

C.Amphetamine.

Ref EASA Module 9A - Human Factors/ Factors Affecting performances

- The symptoms of stress are:

A. Violence, sickness, absence from work, drug and alcohol abuse.

B. Indications of improved work performance.

C. Irritability, forgetfulness, sickness, absence from work, drug and alcohol abuse

Ref EASA Module 9A - Human Factors/ Factors Affecting performances

- When comparing noise levels on Human Performance:

A. Noise has no effect on the number of errors or the speed of performance of an individual.

B. An individual can find noise levels annoying but still perform well indefinitely.

C. Noise is directly proportional to the number of errors and the speed of performance of an individual.

Ref EASA Module 9A - Human Factors/ Factors Affecting performances

- The environmental noise level has a effect on maintenance team it:

A. Always improves concentration and quality of work.

B. Always decreases concentration and quality of work.

C.Has no effect on concentration of quality of work.

Ref EASA Module 9A - Human Factors/ Factors Affecting performances

- Human performance is limited by:

A. Physical factors only.

B. Mental factors only.

C.Always Both physical factors and mental factors

Ref EASA Module 9A - Human Factors/ Factors Affecting performances

- In an error chain, an accident can be prevented by breaking:

A. All links in the error chain.

B. More than one link in the error chain.

C. Only one link which is the main cause, in the error chain.

Ref EASA Module 9A - Human Factors/ Factors Affecting performances

- In an error chain, an accident can be prevented by breaking:

A. All links in the error chain.

B. More than one link in the error chain.

C. Only one link which is the main cause, in the error chain.

Ref EASA Module 9A - Human Factors/ Factors Affecting performances

Sub-Module 9.5: Physical Environment

- Environmental stresses are.

A.Caused by noise, fumes, heat and vibration.

B.Tolerated by everyone equally.

C.Not normally cumulative.

Ref EASA Module 9A - Human Factors/ Physical Environment

- Employers must provide their employees with personal ear protectors if the noise level reaches.

A.70 dB.

B.85 dB.

C.60 dB.

Ref EASA Module 9A - Human Factors/ Physical Environment

- The maximum allowable noise dose is.

A.85 dB.

B.any combination of noise and time which exceeds 90 dB TWA.

C.90 dB for 24 hours.

Ref EASA Module 9A - Human Factors/ Physical Environment

- Environmental capture' is a type of error possible when an engineer does the same job repeatedly.

A.But on different types of aircraft.

B.On the same type of aircraft.

C.In a short timescale.

Ref EASA Module 9A - Human Factors/ Physical Environment

- When working with bright lights consideration should be given to.

A.Blurred image.

B.Shadows.

C.Glare.

Ref EASA Module 9A - Human Factors/ Physical Environment

- Which of the following is true?

A.Noise affects the standard of work proportionately with the level of the noise.

B.Noise does not affect a person's standard of work.

C.Noise does affect the standard of work with certain people.

Ref EASA Module 9A - Human Factors/ Physical Environment

- Excess noise in a working environment can.

A.Raise resistance to other stresses.

B.Not affect performance.

C.Lower resistance to other stresses.

Ref EASA Module 9A - Human Factors/ Physical Environment

- Which of the following environments is best suited to aircraft maintenance?

A.Working outside, at night, in the rain.

B.Working outside, in the direct midday sun.

C.Working inside in a well lit, comfortable hangar.

Ref EASA Module 9A - Human Factors/ Physical Environment

Sub-Module 9.6: Tasks

- Itemized checklists should be followed by:

A. In any order, provided all steps are completed.

B. Item by item, in order, to cover each and every step diligently.

C. As per one memorized.

Ref EASA Module 9A - Human Factors/ Tasks

- A good Shift/work handover constitutes:

A.A written and verbal account of the work done.

B. A written documentation of the work done.

C. A verbal account of the work done.

Ref EASA Module 9A - Human Factors/ Tasks

- When you performing a given a task and you are unsure of the task then you should:

A.Consult the appropriate approved data.

B. Ask someone who has done it before.

C. Consult type training course materials.

Ref EASA Module 9A - Human Factors/ Tasks

- If you are given a task that you are unsure of the task then you should.

A. Consult the appropriate approved data, like AMM, IPC, TSM, etc.

B. Ask someone who has done it before.

C. Consultation your type course notes might guide you.

Ref EASA Module 9A - Human Factors/ Tasks

- When carrying out a risk assessment.

A. Safety items should be worn during assessment.

B. It is necessary to identify where equipment/procedures might fail.

C. Nothing, it is not maintenance job.

Ref EASA Module 9A - Human Factors/ Tasks

- Risk assessment and management is.

A. Reduction of risks to a tolerable standard and monitoring the situation.

B. The investigation of risks and totally removing them and the monitoring of new work practices.

C. Choosing a cheaper supplier or manufacturer for parts.

Ref EASA Module 9A - Human Factors/ Tasks

- Correct Recording / hand over/ logging of work performed allows:

A. Troubleshooting.

B. Traceability.

C. Good troubleshooting and traceability for the entire team.

Ref EASA Module 9A - Human Factors/ Tasks

- A good practice for a shift handover is for shifts to be specifically rostered so there is an overlap of:

A.5 - 10 minutes

B.20 - 30 minutes

C.2 - 3 hours

Ref EASA Module 9A - Human Factors/ Tasks

- When describing tasks, the term 'complex system' refers to a task:

A.Involving fault finding

B.Which may be broken down into many steps

C.Which involve decision making rather than practical work

Ref EASA Module 9A - Human Factors/ Tasks

- At lower light levels, the visual sensing is performed mainly by the:

A.Safety Cones

B.Safety rods

C.Safety Cones, Safety barriers and/or reflective tapes.

Ref EASA Module 9A - Human Factors/ Tasks

Sub-Module 9.7: Communication

- Technical communication between two engineers can go wrong if:

A.The recipient makes an assumption of the meaning of the information being transmitted

B.The sender uses a sketch to augment the information transmittal

C.The recipient uses body language to provide feedback to the sender

Ref EASA Module 9A - Human Factors/ Communication

- A good safe working environment written Communication provides:

A. Traceability.

B. Continuity.

C. Both traceability and continuity

Ref EASA Module 9A - Human Factors/ Communication

- The most effective form of communication is.

A. Verbal communication.

B. Written communication.

C. Explicit communication.

Ref EASA Module 9A - Human Factors/ Communication

- Most common ways of Communication can be:

A. Written.

B. Speaking.

C. Written, Speaking, Visual images and Body language.

Ref EASA Module 9A - Human Factors/ Communication

- Diffusion of responsibility may occur when:

A. An individual working alone

B. An individual working as a member of a team

C. with certain people whether they are working in a team or alone

Ref EASA Module 9A - Human Factors/ Communication

- List three examples of non-verbal communication.

A.Verbal, written, and textual

B.Graphic, symbolic and spoken

C.Graphic, symbolic and body language

Ref EASA Module 9A - Human Factors/ Communication

- What constitutes a good work handover?

A.A written and verbal account of the work done.

B.A written documentation of the work done.

C.A verbal account of the work done.

Ref EASA Module 9A - Human Factors/ Communication

- A good shift handover should include details of.

A.Tasks that have been completed; persons who carried out the tasks; tasks to be carried out and general company and technical information.

B.Tasks that have been completed; tasks in progress, their status, any problems encountered etc.; tasks to be carried out and tools required to carry out the tasks.

C.Tasks that have been completed; tasks in progress, their status, any problems encountered etc.; tasks to be carried out and general company and technical information.

Ref EASA Module 9A - Human Factors/ Communication

Sub-Module 9.8: Human error

- Working excessively long shifts and unsociable hours can lead to:

A. Decreased ability to detect defects during aircraft maintenance.

B. An increased immunity to stress.

C. Increased ability to detect defects during aircraft maintenance.

Ref EASA Module 9A - Human Factors/ Human Error

- Environmental stresses are mainly:

A. Caused by noise, fumes, heat and vibration.

B. Tolerated by everyone equally.

C. Not normally cumulative.

Ref EASA Module 9A - Human Factors/ Human Error

- An engineer who has developed his or her own method of performing a complex task.

A. Should be commended for his/her resourcefulness.

B. Is performing a rule-based behaviour.

C. Is violating on a regular basis and punishable.

Ref EASA Module 9A - Human Factors/ Human Error

- In an error chain, an accident can be prevented by breaking:

A. All links in the error chain.

B. More than one link in the error chain.

C. Only the link causes the error in the chain link.

Ref EASA Module 9A - Human Factors/ Human Error

- The Majority of Maintenance Errors are caused during:

A. Installation.

B. Servicing.

C. Removal

Ref EASA Module 9A - Human Factors/ Human Error

- The violations are considered in aviation as an:

A. Un-planned action.

B. Un-intentional action.

C. Deliberate action

Ref EASA Module 9A - Human Factors/ Human Error

- In Maintenance Repetitive tasks result in:

A. Stress.

B. Fatigue.

C. Complacency

Ref EASA Module 9A - Human Factors/ Human Error

- The incident where a 737 lost oil from both engines is a direct result of

A.Engine vibration

B.Human factors

C.Human error

Ref EASA Module 9A - Human Factors/ Human Error

Sub-Module 9.9: Hazards in the workplace

- Tunnel Vision is an example of:

A.Lack of Awareness.

B. Fatigue.

C. Stress.

Ref EASA Module 9A - Human Factors/ Hazards in the workplace

- Fatigue can be caused by:

A. Physical strain.

B. Mental strain.

C. Both physical and mental strain.

Ref EASA Module 9A - Human Factors/ Hazards in the workplace

- In Human Factor the Complacency is also known as:

A. Over-confidence.

B. Under-confidence.

C. Dissatisfaction

Ref EASA Module 9A - Human Factors/ Hazards in the workplace

- Preparing for emergencies in work place requires:

A. That you have Two things in place: planning, attitude.

B. That you have Three things in place: planning, attitude, supplies.

C. That you have Four things in place: planning, attitude, supplies, and communications.

Ref EASA Module 9A - Human Factors/ Hazards in the workplace

- When someone is working in an enclosed space (such a fuel tank/LG bay/THS) another person should be outside the space in constant communication to:

A.Ensure compliance with the maintenance manual

B.Provide instructions to the tradesman

C.For safety reasons

Ref EASA Module 9A - Human Factors/ Hazards in the workplace

- When a person moves from a poorly lit area to a well-lit area, what is the minimum time they should allow for the eyes to adapt:

A.7 minutes

B.7 seconds

C.30 seconds

Ref EASA Module 9A - Human Factors/ Hazards in the workplace

- In an organization which practices human factors there is:

A.No blame cultures

B.No blame unless there is a deliberate violation

C.Punishment for all violations

Ref EASA Module 9A - Human Factors/ Hazards in the workplace

- Engineers often work on raised platforms, ladders etc. What dangers can this contribute to:

A.Staging may be made of wood

B.Ladder may slip and man falls from heights

C.Two workers may be carrying out maintenance on the same lift

Ref EASA Module 9A - Human Factors/ Hazards in the workplace

- During maintenance “Attention Getters"(like cone, safety barriers/Remove before flight etc..) are used for;

A.To get the operators full attention

B.To gain the operators attention whilst allowing them to continue with the task in hand

C.To make the operator focus his/her attention fully on the task in hand

Ref EASA Module 9A - Human Factors/ Hazards in the workplace

- In the 'slips, lapses and mistakes' definition of errors, forgetting to replace an engine cowling would be considered as:

A.Slip

B.Lapse

C.Mistake

Ref EASA Module 9A - Human Factors/ Hazards in the workplace

MODULE 11A - TURBINE AEROPLANE AERODYNAMICS, STRUCTURES AND SYSTEMS

Sub-Module 11.1: Theory of flight

1. Wing spoilers, when used asymmetrically, are associated with.

A. Ailerons

B. Rudder

C. Elevators

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Theory of flight

2. If an aircraft is yawing to the left, where would you position the trim tab on the rudder?

A. To the centre

B. To the right

C. To the left

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Theory of flight

3. If an aircraft is flying with a left wing low, where would you move the left aileron trim tab?

A. Down

B. Up

C. Moving the aileron trim tab will not correct the situation

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Theory of flight

4. When a leading edge flap is fully extended, what is the slot in the wing for?

A. To allow the flap to retract into it when it retracts

B. To re-energise the boundary layer

C. To increase the lift

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Theory of flight

5. An aircraft left wing is flying low. The aileron trimmer control to the left aileron trim tab in the cockpit would be.

A. Moved up causing the left aileron to move up.

B. Moved up causing the left aileron to move down.

C. Moved down causing the left aileron to move down.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Theory of flight

6. The purpose of a slot in a wing is to.

A. Speed up the airflow and increase lift.

B. Act as venturi, accelerate the air and re-energise boundary layer.

C. Provide housing for the slat.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Theory of flight

7. Large flap deployment.

A. Has no effect on spanwise flow.

B. Causes increased spanwise flow towards tips on wing upper surface.

C. Causes increased spanwise flow towards tips on wing lower surface.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Theory of flight

8. To ensure that a wing stalls at the root first, stall wedges are.

A. Installed at the wing trailing edge at the wing root.

B. Installed at the wing trailing edge at the wing root.

C. Installed on the wing leading edge at the wing root.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Theory of flight

9. Ailerons control the aircraft in the.

A. Longitudinal plane.

B. Directional plane.

C. Lateral plane.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Theory of flight

10. An anti-balance tab is used.

A. For trimming the aircraft.

B. To give more feel to the controls.

C. To relieve stick loads.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Theory of flight

11. If the aircraft is flying nose heavy, which direction would you move the elevator trim tab?.

A. Up to move elevator up.

B. Down to move elevator up.

C. Up to move elevator down.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Theory of flight

Sub-Module 11.2: Airframe Structures – General Concepts

1. Zone 320 under the ATA system is.

A. Horizontal stabiliser.

B. Vertical stabiliser.

C. Central fuselage.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Airframe Structures – General Concepts

2.  How is damage to the nose cone of an aircraft prevented during a lightning strike?

A. Earthing Strap.

B. Bonding Strip.

C. Special paint.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Airframe Structures – General Concepts

3. Tension is the stress of.

A. Elongating or stretch.

B. Crush or compression.

C. Twisting.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Airframe Structures – General Concepts

4. A Fuselage Station is a.

A. Lateral point on aircraft wing.

B. Lateral point on aircraft fuse.

C. Longitudinal point on the aircraft fuselage.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Airframe Structures – General Concepts

5. ATA Zone 100 is.

A. Upper fuselage.

B. Left-hand wing.

C. Lower fuselage.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Airframe Structures – General Concepts

6. Which of the following is an example of a failsafe structure?

A. Longeron

B. Spar

C. Stringer

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Airframe Structures – General Concepts

7. In semi-monocoque construction, compression loads are taken by.

A. Stringers

B. Bulkheads

C. Frames

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Airframe Structures – General Concepts

8. A structural member intended to resist compression is a.

A. Tie

B. Strut

C. Web

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Airframe Structures – General Concepts

9. Stringers are used in which of the following types of aircraft fuselage construction?

A. Semi-monocoque.

B. Truss type.

C. Monocoque.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Airframe Structures – General Concepts

10. What material can be chromated as a protection against corrosion?

A. Aluminum alloys.

B. Magnesium alloys.

C. Ferrous alloys.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Airframe Structures – General Concepts

11. Sealant or levelling compound is installed during structure repair:

A. According to AMM and SRM chapter 51.

B. According to SB instructions.

C. According to separate manufacturer's documentations such as BAC.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Airframe Structures – General Concepts

Sub-Module 11.3: Airframe Structures – Aeroplanes

1. In pressurized aircraft, undercarriage bays.

A. Are unpressurised

B. May be pressurized or unpressurised.

C. Are pressurized

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Airframe Structures – Aeroplanes

2. The main longitudinal members in a fuselage are called.

A. Frames

B. Longerons

C. Spars

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Airframe Structures – Aeroplanes

3. Windshields are heated to.

A. Maintain strength and toughness only.

B. Prevent misting and icing only.

C. Demist, ant-ice and maintain strength and toughness

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Airframe Structures – Aeroplanes

4. If heat is applied to the vinyl interlayer of an aircraft windscreen, its ability to withstand impact will.

A. Increase

B. Decrease

C. Not change

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Airframe Structures – Aeroplanes

5. A plug type window is fitted.

A. From the outside

B. From either side

C. From the inside

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Airframe Structures – Aeroplanes

6. Passenger cabin seat rails form part of the.

A. Frames

B. Stringers

C. Longerons

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Airframe Structures – Aeroplanes

7. Wing bending and shear loads are taken by.

A. Spar cap

B. Skin

C. Main spar

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Airframe Structures – Aeroplanes

8. On a mono-spar wing, what gives the wing its profile contour?

A. Milled stringers.

B. The position of the spars.

C. Ribs.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Airframe Structures – Aeroplanes

Sub-Module 11.4: Air Conditioning system and Cabin Pressurization (ATA 21)

1. In a Bleed Air air-conditioning system, the warm air supply is provided by.

A. The compressor of the gas turbine engine

B. The gas turbine exhaust.

C. The engine exhaust heat.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

2. In most pressurization systems, the amount of compressed air delivered to the cabin is.

A. Reasonably constant irrespective of altitude

B. Constant at any particular altitude but varies for different altitudes.

C. Variable, depending on the amount selected by the cabin rate of change selector.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

3. The purpose of a mass flow controller is to.

A. Ensure that a constant mass of air is delivered to cabin at all times

B. Allow pilot to select the desired cabin altitude.

C. Ensure the cabin altitude remains constant during cruise at all altitudes.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

4. A refrigerant is used in which of the following?

A. Air cycle machine

B. Pneumatic pump

C. Vapour cycle

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

5. A water separator is located.

A. Upstream of the turbine.

B. Downstream of heat exchanger.

C. Downstream of turbine.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

6. Air conditioning systems.

A. Decrease the temperature of air.

B. Increase and decrease the temperature of air.

C. Increase the temperature of air.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

7. An air cycle machine turbine.

A. Drives compressor to increase temperature.

B. Drives compressor to pressurise aircraft.

C. Drives compressor to decrease temperature.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

8. An air-to-air heat exchanger is provided to.

A. Reduce the air supply temperature.

B. Provide an emergency ram air supply.

C. Increase the air supply temperature.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

9. Temperature control of cabin air is achieved by.

A. Controlling the water vapour in the supply.

B. Varying the ambient airflow to the heat exchanger.

C. Regulating the amount of air by-passing the cooling system.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

10. In pressurized aircraft, temperature conditioning is mainly achieved by.

A. Varying cabin pressure.

B. Extracting heat from the pressurising air.

C. Adding heat to the pressurising air.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

11. A cold air unit produces a drop in temperature by.

A. Reducing pressure and driving the units compressor.

B. Driving the units compressor.

C. Reducing pressure.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

12. Heating for pressure cabins is obtained from.

A. Heating for pressure cabins is obtained from.

B. Air supply heated by the pressurising process.

C. Driving the units compressor.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

13. A cold air unit produces a drop in temperature by.

A. Directing compressed air into a heat exchanger.

B. Air supply to the cabin.

C. Expanding hot air across a turbine which is driving a compressor.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

14. In a turbo fan cold air system, the heat exchanger cooling air is.

A. Ram air from ambient conditions

B. Air bled directly from engine or through blower

C. Bled from cabin air supply duct

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

15. Conditioned air is.

A. Oxygen added

B. Moisture removed

C. Temperature and pressure adjusted

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

16. An aircraft cabin is air conditioned and pressurized in order to.

A. Maintain human efficiency and comfort during flights at high altitudes.

B. Ensure that the air density within the cabin is maintained at a lower figure than outside the cabin in order to prevent moisture precipitation during rapid decompression.

C. Ensure that the pressure within the fuselage is always less than the ambient pressure, thus increasing the fatigue life of the fuselage.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

17. In the case of a vapour cycle cooling, system heat is removed from the charge air by.

A. Reducing the pressure of a vapour.

B. Changing a vapour into a liquid.

C. Changing a liquid into a vapour.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

18. The heat exchanger in a turbo-fan system is cooled by.

A. Engine bleed air or blower air.

B. Air bled from the main cabin supply duct.

C. Ambient ram air.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

19. The effective temperature of a cabin is given by.

A. Temperature, humidity, thermal inertia and heat load.

B. Temperature only.

C. Temperature and humidity.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

20. When dissipating heat in a vapour cycle system

A. Vapour converts to a liquid

B. The liquid sublimates

C. Liquid converts to a vapour

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

21. As an aircraft descends from cruising altitude (34,000ft), the cabin altitude must?

A. Decrease

B. Increase

C. Stay the same

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

22. The basic system of cabin pressurisation is to arrange a constant.

A. Inlet and outlet

B. Inlet and vary the outlet

C. Outlet and vary the inlet

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

23. On touch-down of aircraft

A. The outflow valve will be shut

B. The cabin pressure will be zero

C. The outflow valve will be fully open

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

24. In the flightdeck of a pressurised aircraft, there is a gauge that shows?

A. Cabin pressure altitude

B. Aircraft altitude

C. Cabin differential pressure

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

25. Cabin differential pressure is the pressure difference between ?

A. 8,000ft and standard barometric pressure

B. Sea level air pressure and indicated dynamic pressure in the cabin

C. The pressure inside the aircraft and the ambient air pressure

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

26. Cabin pressure is maintained by

A. Controlling the supply of air to the cabin

B. Controlling the output of the compressor

C. Controlling the amount of air discharged from the cabin

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Air Conditioning system and Cabin Pressurization

Sub-Module 11.5: Instrument system (ATA 31)/ Avionic Systems (ATA 22, 23, 34)

A. Avionic Systems (ATA 22, 23, 34)

1. What are the main areas of the autopilot?

A. Error, correction, demand, resolved.

B. Error, correction, follow up, demand.

C. Error, correction, follow up, command

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Avionic Systems

2. Autopilot, when on approach to landing, how many axis are used?

A. 2

B. 4

C. 3

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Avionic Systems

3. A Yaw Damper is:

A. An elevator augmentor

B. A rudder damper designed to avoid the "Dutch roll"

C. A roll trim tab

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Avionic Systems

4. An Automatic Flight Control System receives inputs from the following ground based transmitters.

A. DME, ILS, ADF

B. VOR, ILS

C. RA, ADF, ILS

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Avionic Systems

5. A yaw damper receives error signals from:

A. the turn coordinator rate gyro

B. Vertical gyros

C. Directional gyros

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Avionic Systems

6. Pilot's instinctive cut-out buttons are positioned.

A. on right side of control wheel

B. on side of control wheel furthest from throttles

C. on left side of control wheel

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Avionic Systems

7. Inputs to the rudder channels initially originate from:

A. Compass gyro and gyro for AH

B. AH gyro and turn and slip gyro

C. Compass gyro and turn and slip gyro

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Avionic Systems

8. An aircraft is "homing" to a radio beacon whilst maintaining a relative bearing of zero. If the magnetic heading decreases, the aircraft is experiencing:

A. right drift

B. left drift

C. zero drift

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Avionic Systems

9. Which of the following is the ICAO allocated frequency band for ADF receivers?

A. 255 - 455 kHz

B. 200 - 1750 kHz

C. 300 - 3000 kHz

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Avionic Systems

10. In order to obtain an ADF bearing the:

A. signal must be received by both the sense and loop aerials

B. sense aerial must be tuned separately

C. mode selector should be switched to “loop”

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Avionic Systems

11. The ADF reception loop is always used so that the electromotive force (EMF)

A. induced is zero

B. induced is maximum

C. is zero

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Avionic Systems

12. A loop aerial, as used in the ADF, will

A. Always rotate at constant speed before locking on to a signal from an NDB

B. Will provide the most accurate bearing when it is aligned in the direction resulting in highest signal strength to the ADF receiver

C. Receive a minimum or null signal from a transmitter when the plane of the loop is at right angles to the direction of the transmitter

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Avionic Systems

13. Of the bearing indicators available for use on ADFs, the most sophisticated one is:

A. The Relative Bearing Indicator

B. The Radio Magnetic Indicator

C. The Deviation Indicator

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Avionic Systems

14. SATCOM is used to.

A. Monitor aircraft

B. Relay transmissions to and from the aircraft

C. Determine the position of the aircraft

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Avionic Systems

15. 121.5 MHz is what frequency?

A. VOR

B. VHF

C. ILS

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Avionic Systems

16. Which frequency is used to achieve line of sight radio communication?

A. HF

B. VHF

C. VHF and UHF

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Avionic Systems

17. Which is the frequency separation between consecutive frequencies in the VHF band

A. 75 KHz

B. 50 KHz

C. 25 KHz

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Avionic Systems

18. VHF frequency is.

A. 108 – 136.975 MHz

B. 108 - 112 MHz.

C. 108 - 118 MHz.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Avionic Systems

19. A radio frequency of 16 MHz would be used for ?

A. weather radar.

B. marker beacons.

C. HF communications

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Avionic Systems

20. Service interphone system allows

A. communication between the flight deck and some areas on the aircraft

B. communication to other aircraft

C. Communication to ATC

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Avionic Systems

B. Instrument system (ATA 31)

1. Which of these barometric instruments uses a restrictor to compute its output?

A. Machmeter

B. VSI

C. ASI

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Instrument Systems

2. In a Air Data Computer (ADC) system, Mach information is derived from.

A. pitot and static pressures

B. indicated airspeed and altitude

C. indicated air speed and total air temperature

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Instrument Systems

3. In a Air Data Computer, altitude is produced from.

A. static air pressure

B. pitot air pressure and total air temperature

C. pitot air pressure

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Instrument Systems

4. In a Air Data Computer (ADC), Indicated Air Speed (IAS) is produced from.

A. static pressure only

B. pitot pressure only

C. pitot and static air pressure

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Instrument Systems

5. A machmeter works.

A. always

B. always except on the ground

C. above 10,000 ft

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Instrument Systems

6. In Airways flying, what is the barometric scale of the altimeter set to?

A. QFE

B. QNH

C. 1013.25

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Instrument Systems

7. The pressure measured at the forward facing orifice of a pitot tube is the:

A. static pressure

B. total pressure

C. total pressure plus static pressure

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Instrument Systems

8. A pitot tube covered by ice which blocks the ram air inlet will affect the following instrument (s):

A. altimeter only.

B. airspeed indicator only

C. vertical speed indicator only

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Instrument Systems

9. In a standard atmosphere and at the sea level, the calibrated airspeed (CAS) is:

A. higher than the true airspeed (TAS).

B. independent of the true airspeed (TAS).

C. equal to the true airspeed (TAS)

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Instrument Systems

10. What corrections must be applied to indicated airspeed to produce true airspeed?

A. Correction for heading and altitude

B. Correction for wind and temperature

C. Correction for altitude and temperature

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Instrument Systems

11. Total air pressure consists of static pressure plus:

A. Differential pressure.

B. Total pressure.

C. Dynamic pressure

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Instrument Systems

Sub-Module 11.6: Electrical Power (ATA 24)

1. A lead-acid battery is considered to be fully charged when the.

A. cells begin to gas freely

B. SG and voltage remain constant for specified period

C. SG reaches 1.180

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

2. The preferred method of battery charging a Ni-Cad battery is constant.

A. power

B. voltage

C. current

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

3. To parallel an alternating current generator with one or more other generators, phase A must be.

A. 90 degrees out-of-phase and ABC

B. in-phase with other generators and ABC

C. in-phase with other generators and CBA only

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

4. A fuse-type current limiter

A. limits current flow to a load

B. can be used as a radio suppressor

C. opens a circuit after a time/current condition has been exceeded.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

5. Voltage is controlled by.

A. the constant speed drive

B. the swashplate

C. the voltage regulator

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

6. In an AC generation system, the frequency and phase rotation.

A. must be out of phase prior to paralleling.

B. is of no consequence after paralleling.

C. must be synchronised prior to paralleling.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

7. The output frequency of a hydraulically driven standby generator is controlled by.

A. an IDG

B. a hydraulically actuated swash plate

C. a CSD

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

8. An AC generator used with a CSD.

A. does not need a voltage controller because voltage is kept constant.

B. needs a voltage controller to maintain voltage with increasing load.

C. does not need a voltage controller because current is kept constant.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

9. One of the main functions of the airline battery is to:

A. Provide electric power for heating

B. Be an emergency source of electric power

C. Provide DC power for certain equipment

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

10. Circuit breakers protecting circuits may be:

A. used only in AC circuits.

B. used only in DC circuits.

C. used in AC and DC circuits

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

11. The output voltage of DC generators used in aircraft is normally regulated by:

A. Varying the RPM of the generator.

B. Controlling the current in the armature (anker) windings.

C. Controlling the current in the field windings

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

12. Assume a constant speed DC generator providing a constant output voltage. If the electrical load increases, the voltage regulator will:

A. change the direction of the excitation current.

B. maintain the intensity of the excitation current constant.

C. increase the intensity of the excitation current

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

13. The primary purpose of the reverse current relay is to:

A. Prevent the generator from delivering current to the generator.

B. Prevent the battery from delivering current to the generator.

C. Prevent the generator from delivering too much current

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

14. The so-called "hot buses" or "direct buses" are:

A. directly connected to the battery

B. kept in operating conditions by an electrical resistance in the case of energy failure.

C. automatically connected to the battery if generators have failed

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

15. A bus-bar is:

A. a distribution point for electrical power

B. a device permitting operation of two or more switches together

C. the stator of a moving coil instrument

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

16. A static inverter is a:

A. device for reversing the polarity of the static charge

B. transistorized unit used to convert DC into AC

C. static discharger

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

17. If voltage and frequency of the generator drop to zero in flight, it would be an indication that the.

A. CSD driveshaft had sheared

B. phase sequence detection circuit has operated

C. bus tie interlock is inoperative

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

18. Assuming all systems are operating normally, as aircraft electrical load increases, generator output voltage will

A. increases and amperage output increases.

B. remain constant and amperage output increases.

C. decrease and amperage output increases.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

19. What is the small pin on the DC ground power connector?

A. Positive to battery relay

B. Positive to external power relay

C. Ground or earth

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

20. How is RAT driven?

A. Airstream

B. Compressor

C. Fan

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

21. The most widely used electrical frequency in aircraft is:

A. 200 Hz

B. 50 Hz

C. 400 Hz

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

22. When an overvolt is present on an AC generator system, the tripping circuit.

A. trips after a time delay.

B. trips after a time delay inversely proportional to overvolt amplitude.

C. trips at a set level.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

23. In an AC distribution system, the purpose of the synchronising bus-bar is to.

A. supply essential services.

B. provide interconnection between the generator load busbars.

C. provide a means of monitoring the load.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

24. The moving part in an AC generator is usually referred to as the:

A. oscillator

B. stator

C. rotor

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

25. The function of a constant speed drive (CSD) in an AC generating system is to :

A. vary the engine rpm (within limits) to compensate for various AC loads.

B. vary generator rpm in order to compensate for various AC loads.

C. drive the generator at a constant speed.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

26. AC generator output indications are measured in.

A. KVA & KW

B. KW

C. KW & KVAR

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

27. In the event of complete AC generator failure in an AC split bus-bar generation system, how is power maintained to the essential AC bus-bar?

A. By power fed from static inverter only

B. By power fed from a hot battery bus-bar only

C. By power fed from a battery supplied static inverter

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

28. A 3 phase AC generator has 3 separate stator windings spaced at :

A. 90°

B. 60°

C. 120°

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

29. Assuming a CSD fault is indicated, the CSD should be disconnected:

A. on the ground only.

B. at flight idle engine rpm.

C. during engine operation only

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

30. A Constant Speed Drive aims at ensuring

A. equal AC voltage from all generators.

B. that the CSD remains at a constant RPM not withstanding the generator RPM

C. that the electric generator produces a constant frequency.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Electrical Power

Sub-Module 11.7: Equipment and Furnishings (ATA 25)

1. The specifications for emergency equipment requirements on any given aircraft are found in EASA issued airworthiness codes

A. CS-23

B. CS-25

C. CS-33

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Equipment and Furnishings

2. Some items regularly considered emergency equipment are:

A. Life Rafts, Flashlights, Fire Axes …

B. Life Vests, First Aid Kits, Water Boilers …

C. Fire Axes, Ovens, Life Vests …

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Equipment and Furnishings

3. If a seat belt is not functional and cannot be changed before flight

A. The seat must be placarded and can use

B. The seat can use without placard

C. The seat must be placarded and not used

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Equipment and Furnishings

4. Seat belts and harnesses are constructed from

A. Fiberglass Webbing

B. Nylon Webbing

C. Cotton Webbing

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Equipment and Furnishings

5. Cabin layout is a result of considerations for

A. Comfort for passengers and flight attendants

B. Safety requirements and practical functional arrangements to serve the passengers

C. Comfort for passengers and flight crews

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Equipment and Furnishings

6. Galleys themselves are preconstructed

A. One assembly

B. Modular units

C. One unit

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Equipment and Furnishings

7. Cargo compartments are lined with

A. Any material of panels

B. Fire resistant sidewall and ceiling panels

C. Fire resistant for ceiling panels only

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Equipment and Furnishings

Sub-Module 11.8: Fire Protection (ATA 26)

1. On pulling the fire handle, the aural warning.

A. goes off, the light goes off, hydraulics, bleed air and generator goes off.

B. stays on, the light extinguishes, the generator goes off line.

C. goes off, the light remains on, the generator goes off line.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Fire Protection

2. When a fire test on a Systron Donor fire detector system is carried out.

A. a signal passes through the unbroken centre conductor to produce the warning.

B. it checks the integrity of the contacts.

C. the signal passes through the centre and back through the outer to give warning.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Fire Protection

3. A fire detection loop must be capable of.

A. using master warning visual and aural signals.

B. detecting fire/overheat condition within 5 seconds and extinguishing such that after 30 seconds the condition is no longer present.

C. using master warning visual signals only.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Fire Protection

4. A toilet is fitted with thermal protection.

A. to protect against freezing at altitude.

B. to protect against the dangers of smoke and fire in the event of an overheat.

C. to protect against continuous flushing.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Fire Protection

5. What external warning is there for an APU fire?

A. Aural

B. Both aural and a red light

C. Red light

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Fire Protection

6. In a gas firewire system, what causes the firewire caption to come on when heat is applied?

A. Continuity

B. Pressure

C. Capacitance increase

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Fire Protection

7. A cargo compartment which will contain a fire itself and ventilate is a class.

A. A

B. D

C. C

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Fire Protection

8. What test is carried out to ensure serviceability of a squib?

A. Bottles are fired.

B. Safety ohmmeter is used to check voltage.

C. A continuity check.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Fire Protection

9. Smoke detectors consist of a.

A. lamp detecting 2% smoke

B. photoelectric cell

C. semiconductor loop

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Fire Protection

10. What are the minimum number of fire bottles in a cabin with 201- 300 people?

A. 3 BCF.

B. 4 Fire Extinguishers

C. 4 Methyl Bromide

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Fire Protection

11. How can fire extinguisher be identified as being used?

A. Check the tell-tale wire

B. Weighed

C. Check the pressure

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Fire Protection

12. The indication of the fire detection systems is performed by a:

A. warning bell

B. warning light

C. warning light and a warning bell

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Fire Protection

Sub-Module 11.9: Flight Controls (ATA 27)

1. In normal flight, if the control wheel is moved to the left, then the aileron on the right wing will.

A. Decrease the angle of attack of the right hand wing.

B. Increase the angle of attack of the right hand wing.

C. Keep the angle of attack of the right wing constant.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Flight Controls

2. Aileron input is fed into the yaw damper system to.

A. Prevent nose pitching down.

B. Prevent nose pitching up.

C. Prevent adverse yaw in a turn.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Flight Controls

3. In an automatic flight control system, when may the yaw damper be applied?

A. During manual control only.

B. During either manual or automatic control.

C. During automatic control only.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Flight Controls

4. The aircraft is controlled about the lateral axis by the.

A. Ailerons

B. Rudder

C. Elevator

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Flight Controls

5. The aircraft is controlled about the vertical axis by the.

A. Elevator

B. Rudder

C. Ailerons

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Flight Controls

6. A stick shaker is a device which.

A. Helps extricate an aircraft from soft ground.

B. Gives a short period of extra lift to assist take off.

C. Vibrates the control column near stalling speed.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Flight Controls

7. The aircraft is controlled about the longitudinal axis by the.

A. Ailerons

B. Elevator

C. Rudder

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Flight Controls

8. Which of the following is not a primary flying control?

A. Tailplane

B. Elevator

C. Rudder

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Flight Controls

9. A tab which assists the pilot to move a flying control by moving automatically in the opposite direction to the control surface is called a.

A. Trim tab

B. Servo tab

C. Balance tab

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Flight Controls

10. When spoilers are used asymmetrically, they combine with.

A. Ailerons

B. Elevators

C. Rudder

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Flight Controls

11. What is used to correct any tendency of the aircraft to move towards an undesirable flight attitude?

A. Trim tabs

B. Spring tabs

C. Balance tabs

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Flight Controls

Sub-Module 11.10: Fuel Systems (ATA 28)

1. A fuel crossfeed valve is usually in.

A. The closed position

B. The open position

C. Its last position

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Fuel Systems

2. As the aircraft maneuvers, some probes are in more fuel than others due to the attitude of the aircraft. The indication

A. Remains steady, because the total capacitance transmitted by all of the probes remains the same

B. Remains steady for center tank and difference of wing tanks

C. Remains steady for wing tanks and steady for center tank

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Fuel Systems

3. Fuel tanks interconnected must have.

A. Vents to allow overfueling.

B. Protection against overfill for tanks and fuel system

C. Tank where fuel is fed from both tanks to engine inlet.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Fuel Systems

4. Cross feed valves permit fuel transfer from.

A. Tank to tank.

B. Left tank to right tank.

C. Any tank to any engine

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Fuel Systems

5. In a light fixed wing aircraft, the fuel warning light indicates.

A. Pump failure.

B. The tank does not pressurise.

C. Ice blockage, filter popped or pump fail.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Fuel Systems

6. How is fuel supplied to a turbine engine?

A. By a fuel boost pump.

B. By suction from the engine driven fuel pump.

C. By a gravity feed pump.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Fuel Systems

7. A fuel tank suction valve will open when.

A. The booster pump fails.

B. Both the engine driven and booster pump fail.

C. The engine driven pump fails.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Fuel Systems

8. To prevent water condensation in a fuel tank.

A. The tank should be filled as full as possible

B. The tank should be filled with the minimum fuel possible.

C. The filler cap should be left open.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Fuel Systems

9. When considering the basic categories of fuel tank construction, which of the following is most prone to leaks?

A. Rigid

B. Integral

C. Flexible

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Fuel Systems

10. A leak from a fuel tank which manifests as fuel dripping from the tank is classified as a.

A. Stain

B. Seep

C. Run

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Fuel Systems

11. The advantages of integral fuel tanks are that they.

A. Are cheaper to manufacture, more durable and easier to maintain.

B. Use the maximum volume for fuel and have the minimum amount of weight.

C. Are easier to design and construct and use the maximum volume of fuel.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Fuel Systems

Sub-Module 11.11: Hydraulic Power (ATA 29)

1. Accumulators as fitted to aircraft hydraulic systems.

A. Are only ever used in an emergency

B. Store fluid under pressure

C. Provide additional fluid if leaks occur

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Hydraulic Power

2. A check flow valve in a hydraulic system prevents.

A. Pump cavitation

B. Reverse flow

C. Overpressure

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Hydraulic Power

3. A constant delivery pump has its output varied with.

A. Regulators

B. Gears

C. Servo pressure

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Hydraulic Power

4. A hydraulic closed system is one where.

A. Pressure is maintained at the selector valves at all times.

B. There is no pressure at the selector valves when no functions are required.

C. Most of the major components of the system are included in a self contained unit.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Hydraulic Power

5. What constitutes a hydraulic power pack?

A. Variable swashplate pump.

B. Reservoir, pump, selector valve, filter.

C. Hydraulic oil accumulator.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Hydraulic Power

6. If an accumulator loses air pressure, there will be.

A. Rapid depressurisation of system.

B. Pump cavitation.

C. Rapid fluctuations of instruments.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Hydraulic Power

7. One reason for fitting an accumulator in a hydraulic system is to.

A. Minimize the possibility of pump cavitation.

B. Absorb pressure surges

C. Relieve excess pressure.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Hydraulic Power

8. A thermal relief valve is fitted to a hydraulic system to.

A. Prevent a leak back of pressure.

B. Prevent excess temperature.

C. Relieve excess pressure

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Hydraulic Power

9. A component in a hydraulic system which ensures instantaneous action when a service is selected is.

A. Accumulator.

B. Engine driven pump.

C. Selector.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Hydraulic Power

10. A shuttle valve is used for.

A. Preventing fluid loss from a leaking jack.

B. Maintaining fluid press when the emergency system fails.

C. Change over from main to auxiliary system in the case of failure.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Hydraulic Power

11. The purpose of a mechanical sequence valve is to ensure the operation of.

A. Landing gear and doors

B. Safety switches

C. Brake anti-skid units

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Hydraulic Power

12. A hydraulic accumulator is charged with initial air pressure of 1000 PSI. When the hydraulic system pressure of 3000 PSI is reached, the air pressure is.

A. 4000 PSI

B. 3000 PSI

C. 1000 PSI

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Hydraulic Power

13. Hydraulic reservoirs are pressurized to.

A. Provide a reserve of stored energy.

B. Maintain a constant fluid level.

C. Minimize the possibility of pump cavitation.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Hydraulic Power

14. In aircraft hydraulic systems, it is permissible to use.

A. Only the specified fluid.

B. Any fluid of the same specific gravity.

C. Any hydraulic fluid available.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Hydraulic Power

15. To enable a hydraulic system to operate two components requiring different pressures.

A. A pressure reducing valve is used.

B. A pressure regulating valve is used.

C. A pressure relief valve is used.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Hydraulic Power

Sub-Module 11.12: Ice & Rain Protection system (ATA 30)

1. On large transport aircraft, the windshield wiper system is.

A. One system for both sides but with the same power source

B. Independent on each side but with the same power source

C. Independent on each side with different power sources.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Ice & Rain Protection system

2. On large transport aircraft, the windshield wiper system is.

A. One system for both sides but with the same power source.

B. Independent on each side but with the same power source.

C. Independent on each side with different power sources.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Ice & Rain Protection system

3. A deicer boot is completely bonded to the leading edge to.

A. Provide smoother airflow over leading edge

B. Prevent electrical static build up

C. Provide more efficient deicer cycles

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Ice & Rain Protection system

4. Air for anti-icing of the wings is obtained from.

A. Air conditioning ducting

B. Engine compressors

C. A combustion heater

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Ice & Rain Protection system

5. The usual material for pipelines in a fluid deice system.

A. Stainless steel

B. Monel

C. Nylon

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Ice & Rain Protection system

6. When operating a windscreen wiper on the ground, make sure to.

A. Use slow wiper only.

B. Use water as lubricant when operating.

C. Place soft cloth between blade and window.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Ice & Rain Protection system

7. There are two types of ice are encountered during flight:

A. Clear ice and rime ice

B. Block ice and clear ice

C. Crushed ice and rime ice

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Ice & Rain Protection system

8. When ice collect on the ice detector probe, the established

A. Temperature decreases

B. Frequency of vibration decreases

C. Frequency of vibration increases

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Ice & Rain Protection system

9. The most common source of the heated air is

A. The turbine engine compressor bleed air

B. The APU compressor bleed air

C. Air conditioning system

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Ice & Rain Protection system

10. Windshield heating provides.

A. Thermal expansion for a tighter fit.

B. Impact resistance enhancement.

C. Increases strength to resist cabin pressure.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Ice & Rain Protection system

11. Adjustment of a windshield wiper system consists of adjusting

A. The wiper blade tension, the angle at which the blade sweeps across the windshield, and proper parking of the wiper blades.

B. The wiper blade tension, the angle at which the blade sweeps across the windshield, and speed of the wiper blades

C. The angle at which the blade sweeps across the windshield, speed of the wiper blades and the angle of wiper arms

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Ice & Rain Protection system

Sub-Module 11.13: Landing Gear (ATA 32)

1. Spongy brakes are usually a result of.

A. Internal leakage

B. Air in the system

C. External leakage

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Landing Gear

2. Tubeless tyres are stored.

A. At 15 to 20 P.S.I

B. Horizontally maximum of four high with smallest diameter tyre on top.

C. Vertically

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Landing Gear

3. A restrictor valve.

A. Speed up the flow in one direction

B. May be used to slow down undercarriage extension

C. Restrict the extent of travel of the undercarriage

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Landing Gear

4. The function of the oil in a shock absorber is to.

A. Ensure the separator does not bottom.

B. Damp the rebound.

C. Absorb the landing shock.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Landing Gear

5. Fuse plugs in aircraft tyres are.

A. For over temperature protection.

B. For overpressure protection.

C. To deflate the tyre before removal.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Landing Gear

6. A red dot / triangle mark on aircraft tyres means.

A. Military reference.

B. The light part of the tyre.

C. Breather points.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Landing Gear

7. When fitting a tyre, the red dot should be positioned.

A. On the other side of the wheel opposite to the charging valve.

B. Opposite the charging valve.

C. Adjacent to the charging valve.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Landing Gear

8. The cam plate in a nose undercarriage is.

A. Part of the shimmy damper.

B. To align for nose wheel on nose undercarriage retraction.

C. For alignment of the nose wheel steering on nose undercarriage extension.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Landing Gear

9. A fusible plug fitted to an aircraft wheel prevents.

A. Over inflation of tyres during servicing.

B. Brakes seizure, if brake stators and rotors weld together under excessive temperatures.

C. Tyre bursts due to high temperatures created by excessive braking.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Landing Gear

10. If the shuttle valve was stuck in the normal position.

A. Neither system will be available.

B. Emergency braking will not be available.

C. Normal braking will not be available.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Landing Gear

11. The fusible plugs installed in some aircraft wheel will.

A. Facilitate servicing of the wheel assembly.

B. Melt at a specified elevated temperature.

C. Eliminate the need to check air pressure

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Landing Gear

12. Excessive wear in the shoulder area of an aircraft tyre is an indication of.

A. Under-inflation

B. Over-inflation

C. Excessive toe-in

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Landing Gear

13. When the landing gear is locked up, the cockpit indicator shows.

A. Red light

B. No indication

C. Green light

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Landing Gear

14. On a multi-brake unit with automatic adjusters, brake wear is checked by.

A. Using a go-no go gauge to measure the gap between the cylinder and the thrust plate.

B. Application of the brakes and checking indicator pin protrusion.

C. Measuring the protrusion of the indicator pins with the brakes released.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Landing Gear

Sub-Module 11.14: Lights (ATA 33)

1. With aircraft lights - which of the following is true?

A. Starboard light green, port light red, tail light white.

B. Starboard light red, port light green, tail light Red.

C. Starboard light red, port light green, tail light White.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Lights

2.  A fluorescent tube contains.

A. Orange coatings, rare gases and mercury vapour.

B. Phosphor coatings, rare gases and mercury vapour.

C. Iodine coatings and rare gases.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Lights

3. The visible angle of a white tail navigation light is.

A. 11°

B. 140°

C. 110°

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Lights

4. Wing navigation lights must be visible through which angle?

A. 110°

B. 125°

C. 180°

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Lights

5. Upper and lower strobe lights are coloured.

A. Green

B. Red

C. White

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Lights

6. How are passenger reading lights normally tested?

A. Each one is switched on individually at passenger panel.

B. By using a READ LIGHT TEST switch at any passenger panel.

C. By using a READ LIGHT TEST switch on the cabin attendant panel.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Lights

7. In what position should the selector switch be for a standby and emergency lighting system during flight

A. Armed

B. OFF

C. ON

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Lights

8. Service lights include.

A. refuelling lights, engine scanning lights, logo lights.

B. avionics bay lights, engine scanning lights, baggage compartment lights.

C. baggage compartment lights, avionics bay lights, refuelling lights.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Lights

9. The wing inspection lights are usually controlled through a relay by:

A. an on/off toggle switch in the cockpit

B. A push-button in cabin

C. A DIM/BRIGHT control knob

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Lights

10. A high-intensity white flash is produced from a:

A. strobe light

B. fluorescent tube

C. landing light

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Lights

11. Anti-collision lights can be provided by:

A. rotating beacon or strobe lights

B. fluorescent tubes

C. retractable assembly.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Lights

Sub-Module 11.15: Oxygen (ATA 35)

1. Oxygen cylinders are considered empty when the pressure inside drops below

A. 0 psi

B. 50 psi

C. 65 psi

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Oxygen

2. What pressure are oxygen cylinders usually pressurised to?

A. 300 PSI

B. 1800 PSI

C. 3000 PSI

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Oxygen

3. When charging an oxygen bottle with gaseous oxygen, the oxygen is.

A. Passed into the bottle slowly to keep the temperature at approximately ambient.

B. Passed into the bottle at any rate as this does not affect temperature.

C. Passed into the bottle as quickly as possible to keep the temperature down.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Oxygen

4. How do oxygen cylinders show over pressure?

A. Green rupturing disc

B. Audible warning

C. Red rupturing disc

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Oxygen

5. A chemical oxygen generator has a supply duration of at least.

A. 15 minutes

B. 30 minutes

C. 5 minutes

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Oxygen

Sub-Module 11.16: Pneumatic/ Vacuum (ATA 36)

1. High pressure pneumatic source is a.

A. Reciprocating pump.

B. Centrifugal Compressor.

C. Butterfly pump.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Pneumatic – Vacuum

2. Two compressors driven by separate engines use.

A. Interconnected to share loads.

B. NRVs to prevent compressors driving each other.

C. PRVs.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Pneumatic – Vacuum

3. When high pressure pneumatic air or nitrogen is used to move hydraulic fluid to an actuator, it is called

A. Pneudraulics

B. Leading- up pressure

C. Combined pressure

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Pneumatic – Vacuum

4. A normal working temperature in the pneumatic manifold of close to

A. 140oC

B. 165oC

C. 200oC

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Pneumatic – Vacuum

5. A pressure regulator maintains system pressure with a

A. Shut-off valve

B. Relief valve

C. Isolation valve

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Pneumatic – Vacuum

6. ______are used to lower the system pressure to that require by a particular component or sub system.

A. Pressure reducing valves

B. Pressure relief valves

C. Nonreturn valves

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Pneumatic – Vacuum

7. Pneumatic manifold ____ is a key parameter monitored on the flight deck

A. Pressure

B. Temperature

C. Humidity

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Pneumatic – Vacuum

8. On a high pressure pneumatic system, if the drain plug for oil and water is left open for long periods of time, the system would.

A. Lose pressure from the compressor side only.

B. Lose pneumatic pressure partially.

C. Lose all pressure.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Pneumatic – Vacuum

9. Wing anti-ice and engine starter air use of pneumatic manifold air

A. Without temperature adjustment

B. With temperature adjustment

C. With temperature and pressure adjustment

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Pneumatic – Vacuum

10. A pump that increasing both the velocity and the pressure of the air, this is type of

A. Centrifugal impeller pump

B. Piston pump

C. Vane pump

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Pneumatic – Vacuum

11. An “X” on an isolation valve symbol or the APU shutoff valve symbol shows the valve has

A. Closed

B. Opened but the pressure is abnormal

C. Failed

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Pneumatic – Vacuum

Sub-Module 11.17: Water/ Waste (ATA 38)

1. Waste water drain masts.

A. Are not heated.

B. Are heated using low amperage in the Air and on ground.

C. Are heated using low amperage with the aircraft on ground only.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Water - Waste

2. Drinking water pipes are prevented from freezing by.

A. Installation of neoprene foam insulation.

B. Wrapping the pipes with heated tapes or ribbons.

C. Placing the pipes adjacent to hot water piping.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Water - Waste

Sub-Module 11.18: On board maintenance Systems (ATA 45)

1. Onboard maintenance systems monitor the aircraft for faults, record and store the fault data, and provide information about these faults to.

A. flight crews and maintenance personnel

B. flight crews

C. maintenance personnel

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ On board maintenance Systems

2. The data collected by onboard maintenance systems can be accessed:

A. in flight

B. on the ground

C. both in flight and on the ground

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ On board maintenance Systems

3. Outputs from the CMC are provided in the form of:

A. visual displays, printed text,

B. visual displays, digital data that may be downloaded.

C. visual displays, printed text, and digital data that may be downloaded.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ On board maintenance Systems

4. It is common for a standard onboard maintenance system to have:

A. two CMCs installed

B. only one CMC installed

C. three CMCs installed

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ On board maintenance Systems

5. Class 1 faults are:

A. the most serious, and involve something listed in the MEL.

B. the system is still functioning normally, although it may not have full redundancy.

C. minor discrepancies within a monitored system. They do not affect the operation of the aircraft.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ On board maintenance Systems

6. Class 2 faults are:

A. the most serious, and involve something listed in the MEL.

B. the system is still functioning normally, although it may not have full redundancy.

C. minor discrepancies within a monitored system. They do not affect the operation of the aircraft.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ On board maintenance Systems

7. Class 3 faults are:

A. the most serious, and involve something listed in the MEL.

B. the system is still functioning normally, although it may not have full redundancy.

C. minor discrepancies within a monitored system. They do not affect the operation of the aircraft.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ On board maintenance Systems

8. The information gathered by central maintenance computers can be relayed to the ground through:

A.  the ACARS system

B. VHF system

C. SATCOM system

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ On board maintenance Systems

9. The individual system BITE circuits are connected to the CMC by:

A. a digital control bus

B. a digital data bus

C. a digital address bus

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ On board maintenance Systems

Sub-Module 11.19: Intergrated Modular Avionics (ATA 42)

1. An Integrated modular avionics is a

A. Design methodology

B. An avionics system per se.

C. Specific functions of the individual avionics systems

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Intergrated Modular Avionics

2. An IMA aircraft

A. Has non-redundant power supplies

B. Might have three redundant power supplies

C. One power failure affects each individual system

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Intergrated Modular Avionics

3. VHF communication frequency tuning can use a

A. ARINC 629 data bus

B. AFDX data bus

C. ARINC 429 data bus

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Intergrated Modular Avionics

4. IMA systems consist of a core system

A. This processing circuitry is not shared by the various avionics systems

B. The core system contains data processing circuitry that processes many different kinds of information.

C. The core system uses multiple computer processors for many different purposes

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Intergrated Modular Avionics

Sub-Module 11.20: Cabin Systems (ATA 44)

1. Some of the features that can be included in cabin systems are:

A. Passenger Address, Cabin Interphone, Audio and Video Entertainment

B. Telephone, arrival airport weather.

C. Flight Attendant Calling, cockpit and cabin crew information

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Cabin Systems

2. The Inflight entertainment system provides the passenger with

A. Audio and video on demand.

B. Audio via cabin speakers and video on a main screen

C. Does not include music channels.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Cabin Systems

3.  Cabin monitoring systems are used to monitor

A. Status of smoke and fire detectors in lavatories, Cabin temperature, passenger warning lights

B. Status of smoke and fire detectors in lavatories, Cabin altitude, Status of galley equipment, such as water tanks.

C. Cabin lighting and not cabin entry and exit door status (open or closed)

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Cabin Systems

4. A cabin intercommunication data system is used by

A. Passengers to operate reading lights

B.Flight attendants, pilots, and passengers.

C. flight attendants and pilots only

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Cabin Systems

5. Some of the features that can be included in cabin systems are

A. Cabin Interphone ,Flight Attendant Calling, Passenger seat adjustment

B. Cabin Interphone, Flight Attendant ,Calling Moving Map Displays

C. Cabin Interphone, Flight Attendant Calling, Personal air temperature control

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Cabin Systems

6. Typical user interfaces for the Cabin System are

A. Flight attendant panels, Cockpit handsets, Call buttons

B. Flight attendant panels, Cabin handsets, Cabin layout

C. Flight attendant panels, Reading lights, Gasper air outlets

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Cabin Systems

Sub-Module 11.21: Information systems (ATA 46)

1. Aircraft information systems can be used to store and retrieve

A. Airline policy manual and training manuals.

B. Flight deck information systems, maintenance information systems, and passenger cabin information systems

C. Stored information is only retained for 25 hours

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Information systems

2. Initial Aircraft information systems included

A. High capacity flash drives,

B. Compact disks

C. Manuals, drawings, charts, microfilm or microfiche

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Information systems

3. Aircraft information systems architecture selected by Airbus

A. Is a networked real-time servers and routers

B. Is unsecure both from the point of view of computer security and operational availability

C. Does not include the data related to the flight

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Information systems

4. The use of the digital electronics allows flight attendants to carry

A. A flash drive to insert in the onboard screen

B. A weight saving digital Library

C. Small device, such as a tablet

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Information systems

5. Advantages electronic information system

A. Reduced weight and bulk

B. Reduced weight, reduced bulk, and ease of update

C. Easy to update and reduced weight.

Ref EASA Module 11A - Turbine Aeroplane Aerodynamics/ Information systems

MODULE 15 - GAS TURBINE ENGINE

Sub-Module 15.1: Fundamentals

1. In what section of a turbojet engine is the jet nozzle located?

A. Turbine.

B. Exhaust.

C. Combustion.

Ref EASA Module 15 - Gas Turbine Engine/ Fundamentals

2. On a twin rotor fan engine, the high pressure turbine drives

A. the fan

B. the N1 gas generator

C. the high pressure compressor and accessory gearbox

Ref EASA Module 15 - Gas Turbine Engine/ Fundamentals

3. Which statements are true regarding aircraft engine propulsion?

A. Turbojet and turbofan engines impart a relatively large amount of acceleration to a smaller mass of air.

B. In modern turboprop engines, nearly 50 percent of the exhaust gas energy is extracted by turbines to drive the propeller and compressor with the rest providing exhaust thrust.

C. An engine driven propeller imparts a relatively small amount of acceleration to a large mass of air.

Ref EASA Module 15 - Gas Turbine Engine/ Fundamentals

4. If the temperature of the mass of air is increased while its volume is kept constant, then pressure will

A. decrease

B. stay the same

C. increase

Ref EASA Module 15 - Gas Turbine Engine/ Fundamentals

5. There is no peak pressure in a jet engine because of

A. continuous flow of air

B. constant pressure combustion

C. compression in stages

Ref EASA Module 15 - Gas Turbine Engine/ Fundamentals

6. A free turbine engine with two compressor and three turbine is

A. three spool

B. single spool

C. two spool

Ref EASA Module 15 - Gas Turbine Engine/ Fundamentals

7. An example of a divergent duct is the

A. propelling nozzle

B. passage between the nozzle guide vanes

C. jet pipe at the exit from the turbine

Ref EASA Module 15 - Gas Turbine Engine/ Fundamentals

8. In a twin spool compressor system, the first stage turbine drives the

A. N1 and N2 compressors.

B. N1 compressor.

C. N2 compressor.

Ref EASA Module 15 - Gas Turbine Engine/ Fundamentals

9. In what section of a turbojet engine is the jet nozzle located?

- Turbine.

- Exhaust.

- Combustion.

Ref EASA Module 15 - Gas Turbine Engine/ Fundamentals

10. On a twin rotor fan engine, the high pressure turbine drives

- the fan

- the N1 gas generator

- the high pressure compressor and accessory gearbox

Ref EASA Module 15 - Gas Turbine Engine/ Fundamentals

11. When the volume of fixed mass of gas is reduced to a quarter of its original volume, the pressure will be

- reduced by quarter

- increased by 4 times

- increased by 8 times

Ref EASA Module 15 - Gas Turbine Engine/ Fundamentals

12. Which of the following is the better combination?

- Centrifugal compressor, axial flow turbine, can-type combustion chamber

- Centrifugal compressor, can-type combustion chamber, centrifugal type turbine

- Axial flow single rotor compressor, annular-type combustion chamber, two-stage split turbine

Ref EASA Module 15 - Gas Turbine Engine/ Fundamentals

13. The principle of jet propulsion is

- every action has a equal and opposite reaction

- the interaction of fluids and gases

- the calorific value of fuel burnt is equal to aircraft thrust

Ref EASA Module 15 - Gas Turbine Engine/ Fundamentals

14. The velocity of subsonic air as it flows through a convergent nozzle

- decreases

- remains constant

- increases

Ref EASA Module 15 - Gas Turbine Engine/ Fundamentals

15. The 'core engine' or 'gas generator' is made up of the following components:

- Inlet, compressor, combustion chamber, turbine, exhaust

- Compressor, turbine, exhaust, propelling nozzle

- Turbine, combustion chamber, compressor

Ref EASA Module 15 - Gas Turbine Engine/ Fundamentals

16. Newton's First Law of Motion, generally termed the Law of Inertia, states:

- Every body persists in its state of rest, or of motion in a straight line, unless acted upon by some outside force.

- Force is proportional to the product of mass and acceleration.

- To every action there is an equal and opposite reaction.

Ref EASA Module 15 - Gas Turbine Engine/ Fundamentals

17. The basic gas turbine engine is divided into two main sections: the cold section and the hot section.

- The hot section includes the combustor, turbine, and exhaust

- The hot section includes the combustor, diffuser, and exhaust

- The cold section includes the engine inlet, compressor, and turbine sections.

Ref EASA Module 15 - Gas Turbine Engine/ Fundamentals

18. A modern gas turbine engine accelerates

- a large mass of air at a relatively low velocity

- a small mass of air at a relatively high velocity

- a large mass of air at a relatively high velocity

Ref EASA Module 15 - Gas Turbine Engine/ Fundamentals

19. In a ducted fan engine, the fan is driven by the

- accessory gearbox

- turbine

- air passing over the compressor

Ref EASA Module 15 - Gas Turbine Engine/ Fundamentals

20. The working cycle of gas turbine engine is the

- Diesel cycle

- Brayton cycle

- Otto cycle

Ref EASA Module 15 - Gas Turbine Engine/ Fundamentals

Sub-Module 15.3: Inlet

1. The intake of a gas turbine engine is designed to

A. Protect compressor from FOD

B. Provide streamlined fairing for aircraft

C. Provide turbulent free air

Ref EASA Module 15 - Gas Turbine Engine/ Inlet

2. Anti icing of jet engine air inlets is commonly accomplished by

A. electrical heating elements located within the engine air inlet cowling.

B. engine bleed air ducted through the critical areas.

C. electrical heating elements inside the inlet guide vanes.

Ref EASA Module 15 - Gas Turbine Engine/ Inlet

3. The inlet door on a variable geometry intake is open at

A. subsonic speeds

B. idle speed

C. supersonic speeds

Ref EASA Module 15 - Gas Turbine Engine/ Inlet

4. Inlet guide vanes are anti-iced with

A. engine bleed air

B. rubber boots

C. thermal blankets

Ref EASA Module 15 - Gas Turbine Engine/ Inlet

5. Engine anti-ice is taken from

A. HP compressor

B. Turbine

C. LP compressor

Ref EASA Module 15 - Gas Turbine Engine/ Inlet

6. Engine compressor stall may be induced during rapid attitude changes if the aircraft has

A. engine intakes mounted close to the fuselage

B. long engine intakes

C. engine pods mounted under the wings

Ref EASA Module 15 - Gas Turbine Engine/ Inlet

7. What is true for a bellmouth intake?

A. Velocity increases and pressure decreases

B. Pressure and velocity decrease

C. Pressure increases and velocity decreases

Ref EASA Module 15 - Gas Turbine Engine/ Inlet

8. The purpose of a bellmouth compressor inlet is to

A. maximize the aerodynamic efficiency of the inlet.

B. provide an increased ram air effect at low airspeeds.

C. provide an increased pressure drop in the inlet.

Ref EASA Module 15 - Gas Turbine Engine/ Inlet

9. Electrical anti-ice

- heats oil which is distributed around engine

- heats air which is distributed around engine

- heats elements, placed under mats around engine

Ref EASA Module 15 - Gas Turbine Engine/ Inlet

10. In subsonic multi-engine aircraft, a normal inlet duct will

- decrease and then increase in size, front to rear, along length of the duct

- increase and then decrease in size, front to rear, along length of the duct

- increase in size, front to rear, along length of the duct

Ref EASA Module 15 - Gas Turbine Engine/ Inlet

Sub-Module 15.4: Compressors

1. Compressor stalls may be eliminated by using which of the following systems?

A. Compressor bleed-air system and variable vane system

B. Inlet guide vane and stator vane system

C. Front and rear compressor system

Ref EASA Module 15 - Gas Turbine Engine/ Compressors

2. The variable stator vanes are controlled by

A. The ECU

B. The HMU

C. The VBV gear motor

Ref EASA Module 15 - Gas Turbine Engine/ Compressors

3. The ring of fixed blades at the intake of an axial flow compressor are called

A.first stage stator blades

B. first stage diffuser blades

C. inlet guide vanes

Ref EASA Module 15 - Gas Turbine Engine/ Compressors

4. What is a compressor stage?

A. One Nozzle Guide Vane and one rotor

B. One rotor plus one stator

C. One compressor rotor and one nozzle guide vane

Ref EASA Module 15 - Gas Turbine Engine/ Compressors

5. Stall will occur in the high pressure stages of an axial flow compressor because

A. air mass flow is increased when the compressor is turning at high speed

B. air mass flow is reduced when the compressor is turning at high speed

C. air mass flow is reduced when the compressor is turning at low speed

Ref EASA Module 15 - Gas Turbine Engine/ Compressors

6. The two types of centrifugal compressor impellers are

A. impeller and diffuser.

B. rotor and stator.

C. single entry and double entry.

Ref EASA Module 15 - Gas Turbine Engine/ Compressors

7. What purpose do the diffuser vanes of a centrifugal compressor serve?

A. To convert pressure energy into kinetic energy.

B. To convert kinetic energy into pressure energy.

C. To increase the air velocity.

Ref EASA Module 15 - Gas Turbine Engine/ Compressors

8. Compressor blades are reduced in size from

A. front to rear to maintain compressor efficiency

B. HP to LP to maintain pressure

C. LP to HP to maintain axial velocity

Ref EASA Module 15 - Gas Turbine Engine/ Compressors

9. A bypass engine LP compressor

- supplies more air than is required for combustion

- supplies less air than is required for combustion

- supplies only the required quantity for combustion

Ref EASA Module 15 - Gas Turbine Engine/ Compressors

10. Variable inlet guide vanes help to prevent

- compressor runaway

- compressor stalling

- ice build up on compressor blades

Ref EASA Module 15 - Gas Turbine Engine/ Compressors

Sub-Module 15.5: Combustion Section

1. Combustor air that is not used to support combustion

A. will film cool the liner and dilute combustion chamber exit temperature

B. is by-pass air

C. is considered as the total air flow

Ref EASA Module 15 - Gas Turbine Engine/ Combustion Section

2. Air through the combustion chamber retains a constant

A. Temperature

B. Velocity

C. Pressure

Ref EASA Module 15 - Gas Turbine Engine/ Combustion Section

3. In which type of turbine engine combustion chamber is the case and liner removed and installed as one unit during routine maintenance?

A. Annular

B. Can

C. Can annular

Ref EASA Module 15 - Gas Turbine Engine/ Combustion Section

4. The types of combustion chamber used in gas turbine engine are

A. tubo-annular, can-annular and multiple-can

B. multiple-can, annular and variable

C. Can, Annular and Can annular

Ref EASA Module 15 - Gas Turbine Engine/ Combustion Section

5.  The air passing through the combustion chamber of a jet engine is

A. speeded up and heated by the action of the turbines

B. entirely combined with fuel and burned

C. used to support combustion and to cool the engine

Ref EASA Module 15 - Gas Turbine Engine/ Combustion Section

6. The main purpose of combustion is to

A. increase gas velocity

B. increase gas mass flow

C. increase gas pressure

Ref EASA Module 15 - Gas Turbine Engine/ Combustion Section

7. An annular combustion chamber is preferred over the can design combustion chamber because

A. it is easier to construct and has even pressure throughout the chamber

B. it has multi spray nozzle, less chances of flame out and easy maintenance

C. it gives a smaller engine diameter for the same cross sectional area of airflow and has longer life

Ref EASA Module 15 - Gas Turbine Engine/ Combustion Section

8. Why is it necessary to have a combustion chamber drain?

A. To prevent pressure build-up in the combustion chamber

B. To allow unburnt fuel to drain away

C. To allow fuel to return to LP when hp cock is closed.

Ref EASA Module 15 - Gas Turbine Engine/ Combustion Section

9. How are combustion liner walls cooled in a gas turbine engine?

- By the pattern of holes and louvers cut in the diffuser section.

- By bleed air vented from the engine air inlet.

- By secondary air flowing through the combustion chamber

Ref EASA Module 15 - Gas Turbine Engine/ Combustion Section

10. Fuel entering the combustion chamber from an atomizer spray nozzle enters as

- a fuel/air mixture

- fuel in air pulses

- fuel continuously

Ref EASA Module 15 - Gas Turbine Engine/ Combustion Section

Sub-Module 15.6: Turbine Section

1. A nozzle guide vane is

A. hollow in construction to allow for thermal expansion

B. hollow in construction to allow for flow of cooling air

C. solid in construction to support the guide vane

Ref EASA Module 15 - Gas Turbine Engine/ Turbine Section

2. Forces driving the turbine are due to

A. momentum and directional acceleration of gases

B. expansion of gases

C. aerodynamic lift imposing impulse on blades

Ref EASA Module 15 - Gas Turbine Engine/ Turbine Section

3. Turbine blades can be cooled by

A. film and transpiration

B. convection and thermal barrier

C. film and radiation

Ref EASA Module 15 - Gas Turbine Engine/ Turbine Section

4. Gas pressure through the turbine section will generally

A. decrease

B. remain the same

C. increase

Ref EASA Module 15 - Gas Turbine Engine/ Turbine Section

5. What is meant by a shrouded turbine?

A. The turbine wheel is enclosed by a protective shroud to contain the blades in case of failure.

B. The turbine wheel has a shroud or duct which provides cooling air to the turbine blades.

C. The turbine blades are shaped so that their ends form a band or shroud.

Ref EASA Module 15 - Gas Turbine Engine/ Turbine Section

6. Why do some turbine engines have more than one turbine wheel attached to a single shaft?

A.To facilitate balancing of the turbine assembly.

B. To help stabilize the pressure between the compressor and the turbine.

C. To extract more power from the exhaust gases than a single wheel can absorb.

Ref EASA Module 15 - Gas Turbine Engine/ Turbine Section

7. Which of the following is most likely to occur in the turbine section of a gas turbine engine?

A. Pitting

B. Cracking

C. Galling

Ref EASA Module 15 - Gas Turbine Engine/ Turbine Section

8. Jet engine turbine blades removed for detailed inspection must be reinstalled in

A. a specified slot 90° away in the direction of rotation.

B. a specified slot 180° away.

C. the same slot.

Ref EASA Module 15 - Gas Turbine Engine/ Turbine Section

9. As the hot gases flow through an impulse turbine blading, the velocity

- will decrease

- will increase

- remains constant

Ref EASA Module 15 - Gas Turbine Engine/ Turbine Section

10. Air used to cool HP turbines is tapped from the

- last few stages of the compressor

- mid-stages of the compressor

- first few stages of the compressor

Ref EASA Module 15 - Gas Turbine Engine/ Turbine Section

Sub-Module 15.7: Exhaust

1. An exhaust cone placed aft of the turbine in a jet engine will cause the pressure in the first part of the exhaust duct to

A. increase and the velocity to increase.

B. increase and the velocity to decrease.

C. decrease and the velocity to increase.

Ref EASA Module 15 - Gas Turbine Engine/ Exhaust

2. The function of the exhaust cone assembly of a turbine engine is to

A. straighten and collect the exhaust gases into a solid exhaust jet.

B. swirl and collect the exhaust gases into a single exhaust jet.

C. collect the exhaust gases and act as a noise suppressor.

Ref EASA Module 15 - Gas Turbine Engine/ Exhaust

3. On front fan engines, to obtain thrust reversal, the

A. hot and cold streams are reversed

B. hot stream is reversed

C. cold stream is reversed

Ref EASA Module 15 - Gas Turbine Engine/ Exhaust

4. The exhaust section is designed to

A. impart a high exit velocity to the exhaust gases.

B. decrease temperature, therefore decreasing pressure.

C. increase temperature, therefore increasing velocity.

Ref EASA Module 15 - Gas Turbine Engine/ Exhaust

5. The struts on the exhaust cone

A. straighten the gas flow only.

B. support the exhaust cone only.

C. support the exhaust cone and straighten the gas flow.

Ref EASA Module 15 - Gas Turbine Engine/ Exhaust

6. What is the proper operating sequence when using thrust reversers to slow an aircraft after landing?

A. Retard thrust levers to ground idle, select reverse thrust, then advance power levers as required, and then retard thrust reverser levers to ground idle.

B. Advance thrust levers up to takeoff position as conditions require, select thrust reverse, de-select thrust reverser, retard thrust levers to ground idle.

C. Select thrust reverse, advance thrust reverser levers no higher than 75% N1, and retard thrust reverser levers to idle at approximately normal taxi speed.

Ref EASA Module 15 - Gas Turbine Engine/ Exhaust

7. Thrust reversal on a high bypass engine is achieved by

A. bucket type doors

B. blocker doors

C. clamshell configuration

Ref EASA Module 15 - Gas Turbine Engine/ Exhaust

8. As the air flows out at the outflow of a choked nozzle

A. velocity increases and pressure decreases

B. velocity and pressure decrease

C. velocity decreases and pressure increases

Ref EASA Module 15 - Gas Turbine Engine/ Exhaust

9. Normal gas turbine engine's exhaust duct is

- convergent/divergent

- convergent

- divergent

Ref EASA Module 15 - Gas Turbine Engine/ Exhaust

10. Thrust reversers may be operated at

- high Power

- any power

- low Power.

Ref EASA Module 15 - Gas Turbine Engine/ Exhaust

Sub-Module 15.9: Lubricants and Fuels

1. Oil used in a gas turbine engine is usually

A. mineral

B. natural

C. synthetic

Ref EASA Module 15 - Gas Turbine Engine/ Lubricants and Fuels

2. Ignition of fuel depends upon

A. Volatility

B. Both volatility and atomisation

C. Atomisation

Ref EASA Module 15 - Gas Turbine Engine/ Lubricants and Fuels

3. When using Prist or Biopor

A. it is left and burnt with the fuel

B. it is flushed out immediately

C. it is diluted with water to a 3-1 mix

Ref EASA Module 15 - Gas Turbine Engine/ Lubricants and Fuels

4. What action is taken to protect integral fuel tanks from corrosion due to micro biological contamination?

A. Rubber liners are installed in the tank

B. An Anti-microbial additive is added to the fuel

C. The inside of the tank is coated with yellow chromate

Ref EASA Module 15 - Gas Turbine Engine/ Lubricants and Fuels

5. The viscosity of a liquid is a measure of its

A. weight, or density.

B. resistance to flow.

C. rate of change of internal friction with change in temperature.

Ref EASA Module 15 - Gas Turbine Engine/ Lubricants and Fuels

6. In addition to lubricating (reducing friction between moving parts), engine oil performs what functions?

A. Cools, seals, prevents corrosion

B. Cools, seals, prevents corrosion, cushions shock loads

C. Cools and seals

Ref EASA Module 15 - Gas Turbine Engine/ Lubricants and Fuels

7. The types of water present in aviation fuel are

A. dissolved and entrained both harmful

B. dissolved that is harmless and entrained that is harmful

C. dissolved that is harmful and entrained that is harmless

Ref EASA Module 15 - Gas Turbine Engine/ Lubricants and Fuels

8. Compared to reciprocating engine oils, the types of oils used in turbine engines?

A. have less tendency to produce lacquer or coke.

B. are required to carry and disperse a higher level of combustion by-products.

C. may permit a somewhat higher level of carbon formation in the engine.

Ref EASA Module 15 - Gas Turbine Engine/ Lubricants and Fuels

Sub-Module 15.10: Lubrication Systems

1. What is the purpose of the last chance oil filters?

A. To assure a clean supply of oil to the lubrication system.

B. To filter the oil immediately before it enters the main bearings.

C. To prevent damage to the oil spray nozzle.

Ref EASA Module 15 - Gas Turbine Engine/ Lubrication Systems

2. What is the primary purpose of the oil to fuel heat exchanger?

A. De aerate the oil

B. Cool the fuel

C. Cool the oil

Ref EASA Module 15 - Gas Turbine Engine/ Lubrication Systems

3. Oil flow through a filter is

A. for a pressure filter, inside to out

B. for a pressure filter, outside to in

C. for a scavenge filter, inside to out.

Ref EASA Module 15 - Gas Turbine Engine/ Lubrication Systems

4. What will result if an oil filter element becomes completely blocked?

A. Oil will flow at the normal rate through the system

B. Oil will flow at a reduced rate through the system

C. Oil flow to the engine will stop.

Ref EASA Module 15 - Gas Turbine Engine/ Lubrication Systems

5. A turbine engine dry sump lubrication system of the self contained, high pressure design

A. has no heat exchanger.

B. stores oil in the engine crankcase.

C. consists of pressure, breather, and scavenge subsystems.

Ref EASA Module 15 - Gas Turbine Engine/ Lubrication Systems

6. How is engine oil usually cooled?

A. By a fuel/oil cooler

B. By ram air

C. By bleed air

Ref EASA Module 15 - Gas Turbine Engine/ Lubrication Systems

7. How are the teeth of the gears in the accessory section of an engine normally lubricated?

A. By splashed or sprayed oil.

B. By submerging the load bearing portions in oil.

C. By surrounding the load bearing portions with baffles or housings within which oil pressure can be maintained.

Ref EASA Module 15 - Gas Turbine Engine/ Lubrication Systems

8. The sump in a dry sump oil system

A. provides lubrication for the main bearings

B. houses all the engine oil

C. is used as a collecting point only

Ref EASA Module 15 - Gas Turbine Engine/ Lubrication Systems

Sub-Module 15.11: Fuel Systems

1. Why is it necessary to control fuel supply to the engine during rapid acceleration?

A. To control maximum RPM

B. To prevent excessively high EGT and possible compressor surge

C. To prevent compressor stall above cruise RPM

Ref EASA Module 15 - Gas Turbine Engine/ Fuel Systems

2. A fuel heater prevents

A. entrained water in fuel freezing

B. pipelines freezing

C. LP fuel filter icing

Ref EASA Module 15 - Gas Turbine Engine/ Fuel Systems

3. The purpose of the LP fuel pump is to

A. ensure the fuel flow governor gets enough fuel

B. ensure the HP fuel pump does not cavitate

C. pump fuel from the aircraft fuel tanks to the engine

Ref EASA Module 15 - Gas Turbine Engine/ Fuel Systems

4. Fuel boost pumps are cooled using

A. fuel

B. ram air

C. Fuel pumps do not require cooling

Ref EASA Module 15 - Gas Turbine Engine/ Fuel Systems

5. What power supplies the EEC during normal operation?

A. Main aircraft aircraft bus

B. A dedicated generator

C. Aircraft batteries

Ref EASA Module 15 - Gas Turbine Engine/ Fuel Systems

6. One advantage of FADEC over the normal hydro-mechanical fuel system is that it

A. reduces fuel consumption

B. improves engine performance and reduces pilot work-load

C. improves the thrust

Ref EASA Module 15 - Gas Turbine Engine/ Fuel Systems

7. Where is the engine fuel shutoff valve usually located?

A. Downstream of the engine driven fuel pump.

B. Adjacent to the fuel pump.

C. Aft of the firewall.

Ref EASA Module 15 - Gas Turbine Engine/ Fuel Systems

8. Which valve ensures the adequate fuel flow?

A. The fuel metering valve

B. The burner staging valve

C. The HP fuel SOV

Ref EASA Module 15 - Gas Turbine Engine/ Fuel Systems

Sub-Module 15.12: Air Systems

1. Engine anti-ice is taken from the

A. Turbine

B. HP compressor

C. LP compressor

Ref EASA Module 15 - Gas Turbine Engine/ Air Systems

2. In an axial flow turbine engine, compressor bleed air is sometimes used to aid in cooling the

A. turbine, vanes, blades, and bearings.

B. inlet guide vanes.

C. fuel.

Ref EASA Module 15 - Gas Turbine Engine/ Air Systems

3. Air used for film cooling of the HP nozzle guide vanes (NGVs) is

A. ducted into the by-pass duct

B. passes into the gas stream

C. ducted overboard

Ref EASA Module 15 - Gas Turbine Engine/ Air Systems

Sub-Module 15.13: Starting and Ignition Systems

1. On light up, the gas temperature will

A. rise rapidly, then fall as RPM increases to idle.

B. rise rapidly.

C. rise slowly.

Ref EASA Module 15 - Gas Turbine Engine/ Starting and Ignition Systems

2. During engine starting, self sustaining speed is the RPM at which

A. light-up occurs

B. the starter can be disengaged

C. steady idling is attained.

Ref EASA Module 15 - Gas Turbine Engine/ Starting and Ignition Systems

3. Generally, when removing a turbine engine igniter plug, in order to eliminate the possibility of the technician receiving a lethal shock, the ignition switch is turned off and

A. disconnected from the power supply circuit.

B. the transformer exciter input lead is disconnected and the centre electrode grounded to the engine after disconnecting the igniter lead from the plug and waiting the prescribed time.

C. the igniter lead is disconnected from the plug and the centre electrode grounded to the engine after disconnecting the transformer-exciter input lead and waiting the prescribed time.

Ref EASA Module 15 - Gas Turbine Engine/ Starting and Ignition Systems

4. The primary advantage of pneumatic (air turbine) starters over comparable electric starters for turbine engines is

A. a decreased fire hazard

B. high power-to-weight ratio

C. reduction gearing not required

Ref EASA Module 15 - Gas Turbine Engine/ Starting and Ignition Systems

5. What is the first engine instrument indication of a successful start of a turbine engine?

A. A rise in the engine fuel flow.

B. A rise in oil pressure.

C. A rise in the exhaust gas temperature.

Ref EASA Module 15 - Gas Turbine Engine/ Starting and Ignition Systems

6. Is it safe to touch the output from one ignition exciter 10 minutes after switching off the system?

A. Yes, even if there is voltage at the output, without input power this is not dangerous

B. Yes, I can be sure that there is no longer any voltage at the output

C. No, even after hours there can be a dangerous voltage at the output

Ref EASA Module 15 - Gas Turbine Engine/ Starting and Ignition Systems

7. A gas turbine engine normally has provisions for what total number of igniter plugs in the combustion chamber?

A. Two

B. One

C. Three

Ref EASA Module 15 - Gas Turbine Engine/ Starting and Ignition Systems

8. Self sustaining speed is

A. take off velocity

B. V1 speed

C. the RPM at which the engine continues without external assistance

Ref EASA Module 15 - Gas Turbine Engine/ Starting and Ignition Systems

9. A hung start will be indicated by

A. RPM lower than than self sustaining speed

B. engine fails to light up

C. EGT exceeding limit

Ref EASA Module 15 - Gas Turbine Engine/ Starting and Ignition Systems

10. How does the ignition system of a gas turbine engine differ from that of a reciprocating engine?

A. Magneto to engine timing is not critical.

B. One igniter plug is used in each combustion chamber.

C. A high energy spark is required for ignition.

Ref EASA Module 15 - Gas Turbine Engine/ Starting and Ignition Systems

11. The speed at which an engine will accelerate under its own power is known as

A. ground idle

B. self-sustaining speed

C. light up speed

Ref EASA Module 15 - Gas Turbine Engine/ Starting and Ignition Systems

Sub-Module 15.14: Engine Indication Systems

1. How are EGT thermocouples connected?

A. Series

B. Parallel

C. Parallel / Series

Ref EASA Module 15 - Gas Turbine Engine/ Engine Indication Systems

2. Torque pressure is usually read from a

A. direct reading pressure gauge

B. tension gauge

C. torque meter

Ref EASA Module 15 - Gas Turbine Engine/ Engine Indication Systems

3. Fuel flow indication is taken from

A.after the HP pump

B. after the LP pump

C. after either HP Pump or LP Pump

Ref EASA Module 15 - Gas Turbine Engine/ Engine Indication Systems

4. Where is Turbine Outlet Temperature (TOT) measured?

A. Downstream of the turbine

B. Upstream of the turbine

C. In the combustion chamber

Ref EASA Module 15 - Gas Turbine Engine/ Engine Indication Systems

5. In what units are gas turbine engine tachometers calibrated?

A. Actual engine RPM.

B. Percent of engine RPM.

C. Percent of engine pressure ratio.

Ref EASA Module 15 - Gas Turbine Engine/ Engine Indication Systems

6. The speed of the motor in the tacho-indicator depends on

A. current

B. voltage

C. frequency

Ref EASA Module 15 - Gas Turbine Engine/ Engine Indication Systems

7. Fuel flow is measured in

A. lbs/hr or kg/hr

B. litres (volume)

C. lbs/min or kg/min

Ref EASA Module 15 - Gas Turbine Engine/ Engine Indication Systems

8. Engine pressure ratio is the total pressure ratio between the

- front of the engine inlet and the aft end of the compressor.

- aft end of the compressor and the aft end of the turbine.

- front of the compressor and the rear of the turbine.

Ref EASA Module 15 - Gas Turbine Engine/ Engine Indication Systems

Sub-Module 15.16: Turboprop Engines

1. What type of reduction gear is used on most turboprop modern engines?

A. Epicyclic reduction gear

B. Helical cut parallel spur gears

C. Straight cut parallel spur gears

Ref EASA Module 15 - Gas Turbine Engine/ Turboprop Engines

2. A free-turbine turbo-prop engine has

A. 2 turbine stages driving one shaft

B. 3 turbine stages driving one shaft

C. 2 or more turbine stages driving two shafts

Ref EASA Module 15 - Gas Turbine Engine/ Turboprop Engines

3. Modern high performance turbo-prop aeroplanes use a

A. fixed pitch propeller

B. constant speed propeller

C. ground adjustable propellers

Ref EASA Module 15 - Gas Turbine Engine/ Turboprop Engines

4. A turbo-propeller engine utilises a

A. either centrifugal compressor or axial flow compressor

B. axial flow compressor

C. centrifugal compressor

Ref EASA Module 15 - Gas Turbine Engine/ Turboprop Engines

5. A free turbine is usually found on a

A. turbo-fan

B. turbo-jet

C. turbo-prop

Ref EASA Module 15 - Gas Turbine Engine/ Turboprop Engines

6. A free turbine is

A. connected directly to the power output shaft

B. not directly connected to the power output shaft

C. connected directly to the propeller and compressor

Ref EASA Module 15 - Gas Turbine Engine/ Turboprop Engines

7. A free-turbine engine has how many turbines?

A. 3

B. 1

C. 2

Ref EASA Module 15 - Gas Turbine Engine/ Turboprop Engines

Sub-Module 15.17: Turboshaft Engine

1. On a twin spool turbo-shaft engine, the free turbine is connected to the

A. HP gearbox

B. LP gearbox

C. output gearbox

Ref EASA Module 15 - Gas Turbine Engine/ Turboshaft Engine

2. In the majority of helicopters, the thrust generated by the gas generator is absorbed by the

A. HP turbine

B. Free power turbine

C. LP turbine

Ref EASA Module 15 - Gas Turbine Engine/ Turboshaft Engine

3. A turbo-shaft engine has

A. a power shaft which is not connected to the compressor

B. neither of the other answers

C. a mechanical connection between compressor and the power turbine

Ref EASA Module 15 - Gas Turbine Engine/ Turboshaft Engine

4. The output of a turboshaft engine

A. is torque

B. is thrust

C. drives a fan

Ref EASA Module 15 - Gas Turbine Engine/ Turboshaft Engine

5. In relation to a turbo-shaft engine, what is the power turbine?

A. It is a turbine designed to drive the compressor and the load

B. It is a turbine wheel dedicated to drive the load

C. It is another name for a compressor turbine

Ref EASA Module 15 - Gas Turbine Engine/ Turboshaft Engine

6. What is the purpose of a free-wheeling unit as fitted to a helicopter reduction gearbox?

A. It will not allow the rotor to back-drive the engine

B. It will prevent the engine from overspeeding the rotor

C. It de-couples the engine from the rotor on engine start-up

Ref EASA Module 15 - Gas Turbine Engine/ Turboshaft Engine

Sub-Module 15.18: Auxiliary Power Unit (APUs)

1. An APU has

A. variable speed and is self contained

B. automatically controlled thrust and is self contained

C. constant speed and is self contained

Ref EASA Module 15 - Gas Turbine Engine/ Auxiliary Power Unit (APUs)

2. What are the two most important signals when monitoring an APU?

A. EGT and RPM

B. EGT and Oil Pressure

C. Oil Pressure and Inlet Pressure

Ref EASA Module 15 - Gas Turbine Engine/ Auxiliary Power Unit (APUs)

3. An APU is usually rotated during start by

A. a pneumatic starter.

B. a turbine impingement system.

C. an electric starter.

Ref EASA Module 15 - Gas Turbine Engine/ Auxiliary Power Unit (APUs)

4. Auxiliary power units provide

A. hydraulic and pneumatic power

B. pneumatic and electrical power

C. hydraulic and electrical power

Ref EASA Module 15 - Gas Turbine Engine/ Auxiliary Power Unit (APUs)

5. When is the APU at its greatest load?

A. With bleeds open and with generator loads on line

B. With generator loads on line

C. With bleeds closed

Ref EASA Module 15 - Gas Turbine Engine/ Auxiliary Power Unit (APUs)

6. An APU power lever is located

A. An APU is fully automatic and does not require a power lever

B. behind the throttles

C. at the Flight Engineer Station

Ref EASA Module 15 - Gas Turbine Engine/ Auxiliary Power Unit (APUs)

7. What initiates APU shutdown?

A. Low oil pressure, low oil quantity, high oil temperature

B. Overspeed, fire detection, low oil quantity

C. Fire detection, low oil pressure, high oil temperature, overspeed

Ref EASA Module 15 - Gas Turbine Engine/ Auxiliary Power Unit (APUs)

8. From where does the APU receive a fire signal?

A. It is dependent on the engine fire system

B. It has its own system

C. It is dependent on the airframe system

Ref EASA Module 15 - Gas Turbine Engine/ Auxiliary Power Unit (APUs)

9. When necessary, APU engine cooling before shutdown may be accomplished by

A. unloading the generator(s).

B. closing the bleed air valve.

C. opening the bleed air valve.

Ref EASA Module 15 - Gas Turbine Engine/ Auxiliary Power Unit (APUs)

10. An APU load compressor

A. loads the APU when electrical loads are required

B. supplies air to the gas generator

C. supplies air to the aircraft

Ref EASA Module 15 - Gas Turbine Engine/ Auxiliary Power Unit (APUs)

Sub-Module 15.19: Powerplant Installation

1. Forward engine mounts take which loads?

A. Thrust, vertical and side loads

B. Centrifugal, thrust and axial

C. Thrust, vertical and impact

Ref EASA Module 15 - Gas Turbine Engine/ Powerplant Installation

2. Rubber anti-vibration pads are fitted to engine

A. components to prevent fatigue

B. pylons to prevent vibration through the airframe

C. cradles to prevent damage during transportation

Ref EASA Module 15 - Gas Turbine Engine/ Powerplant Installation

3. A powerplant consists of

A. a basic engine plus E.C.U

B. a basic engine plus thrust reverser, exhaust system and gear box with accessories

C. the complete engine as it would be found on aircraft including all connections, controls, cowlings, intake etc

Ref EASA Module 15 - Gas Turbine Engine/ Powerplant Installation

4. Vibration mounts are used for

A. preventing engine vibration loads being transmitted to the airframe structure

B. stopping vibrations entering the engines

C. damping out vibration stresses on engine when being transported on an engine stand

Ref EASA Module 15 - Gas Turbine Engine/ Powerplant Installation

5. Acoustic blankets are installed to

A. increase thermal efficiency

B. aid the streamlining of the engine

C. reduce noise levels

Ref EASA Module 15 - Gas Turbine Engine/ Powerplant Installation

6. Where are the lifting points on a high bypass turbine engine?

A. On the fan and turbine casing

B. On the fan and compressor casing

C. On the fan, turbine and compressor casing

Ref EASA Module 15 - Gas Turbine Engine/ Powerplant Installation

Sub-Module 15.20: Fire protection System

1. The fire extinguisher used on an aircraft engine is

A. Halogenated Hydrocarbons

B. Sodium Chlorate

C. Inert Carbon

Ref EASA Module 15 - Gas Turbine Engine/ Fire protection System

2. 3 ways to test serviceability of a fixed fire bottle in situ are

A. weigh, pressure, pop up indicator

B. pressure, pop up indicator, blow-out disc

C. weigh, pressure, blow-out disc

Ref EASA Module 15 - Gas Turbine Engine/ Fire protection System

3. In a two shot fire extinguishing system

A. extinguishers distributed once to either engine compartment

B. extinguisher distributed twice to each engine compartment

C. one squib can be fired, if that fails then the 2nd squib can be fired

Ref EASA Module 15 - Gas Turbine Engine/ Fire protection System

4. For fire detection and extinguishing purposes, aircraft powerplant areas are divided into fire zones based on

A. the volume and smoothness of the airflow through engine compartments.

B. hot and cold sections of the engine.

C. engine type and size.

Ref EASA Module 15 - Gas Turbine Engine/ Fire protection System

5. The advantage of the two shot fire bottle system is

A. both bottles can be used in either of the engines

B. one bottle can be used twice

C. one bottle can be discharged after certain time delay from the other bottle

Ref EASA Module 15 - Gas Turbine Engine/ Fire protection System

6. Gas turbine engine fire bottles contain which of the following as extinguishing agents?

A. Halogenated Hydrocarbon

B. Carbon Dioxide

C. Water

Ref EASA Module 15 - Gas Turbine Engine/ Fire protection System

7. The explosive cartridge in the discharge valve of a fire extinguisher container is

A. not a life dated unit.

B. a life dated unit.

C. mechanically fired.

Ref EASA Module 15 - Gas Turbine Engine/ Fire protection System

8. What is the function of a fire detection system?

A. To activate a warning device in the event of a powerplant fire.

B. To discharge the powerplant fire extinguishing system at the origin of the fire.

C. To identify the location of a powerplant fire.

Ref EASA Module 15 - Gas Turbine Engine/ Fire protection System

9. In the event of an engine fire whilst carrying out ground runs (under cowl fire with cowls fitted and cold) the appropriate action would be?

A. Move throttle lever to idle, close HP cock, pull fire handle, discharging bottles

B. Pull fire handle, move throttle lever to idle, close HP cock, discharge bottles

C. Pull fire handle, discharge bottles, move throttle lever to idle, close HP cock

Ref EASA Module 15 - Gas Turbine Engine/ Fire protection System

10. What is the operating principle of the spot detector sensor in a fire detection system?

A. A bimetallic thermos switch that closes when heated to a high temperature

B. Resistant core material that prevents current flow at normal temperatures

C. A conventional thermocouple that produces a current flow.

Ref EASA Module 15 - Gas Turbine Engine/ Fire protection System

Sub-Module 15.21: Engine monitoring and Ground Operation

1. A magnetic chip detector detects

A. Ferrous particles only

B. Particles held in suspension

C. Particles which are too small for the naked eye

Ref EASA Module 15 - Gas Turbine Engine/ Engine monitoring and Ground Operation

2. Normal gas turbine engine starting sequence is

A. Starter, ignition, fuel

B. Fuel, starter, ignition

Ref EASA Module 15 - Gas Turbine Engine/ Engine monitoring and Ground Operation

3. When running down an engine

A. It should be done as slowly as possible to assist thermal stress

B. It should be done as quickly as possible to stop excess of fuel gathering

C. It should be done as slowly as possible to reduce thermal stress

Ref EASA Module 15 - Gas Turbine Engine/ Engine monitoring and Ground Operation

4. When accelerating from 'light-up' to ground idling speed, the EGT will

A. Increase above idle value then decrease to normal

B. Decrease below idle value then increase to normal

C. Remain constant

Ref EASA Module 15 - Gas Turbine Engine/ Engine monitoring and Ground Operation

5. A hung start or false start is one in which?

A. 'light up' occurs, but the RPM does not increase

B. There is no 'light up'

C. The engine does not rotate

Ref EASA Module 15 - Gas Turbine Engine/ Engine monitoring and Ground Operation

6. What is the first engine instrument indication of a successful start of a turbine engine?

A. A rise in the engine fuel flow.

B. A rise in the exhaust gas temperature.

C. A rise in oil pressure.

Ref EASA Module 15 - Gas Turbine Engine/ Engine monitoring and Ground Operation

7. During starting of an engine, light-up is indicated by the

A. Fuel-flow indicating system

B. Exhaust gas temperature

C. Tachometer-indicating system

Ref EASA Module 15 - Gas Turbine Engine/ Engine monitoring and Ground Operation

8. The most satisfactory extinguishing agent for an intake fire is

A. Methyl bromide

B. Carbon dioxide

C. Dry chemical

Ref EASA Module 15 - Gas Turbine Engine/ Engine monitoring and Ground Operation

MODULE 17A - PROPELLER

Sub-Module 17.1: Fundamentals

1. The propeller is 'feathered' when the blades are at

A. 20° to plane of rotation.

B. 0° to plane of rotation.

C. 90° to plane of rotation.

Ref EASA Module 17A – Propeller/ Fundamentals

2. The plane of rotation is defined as

A. the plane in which thrust force acts

B. the angle at which the blade strikes the airflow

C. the plane in which the propeller rotates

Ref EASA Module 17A – Propeller/ Fundamentals

3. Relative airflow is the airflow due to the

A. Aircraft forward speed only

B. Propeller rotation only

C. Combination of propeller rotation and aircraft forward speed

Ref EASA Module 17A – Propeller/ Fundamentals

4. When in the windmill position ATM

A. is not related to CTM

B. opposes CTM

C. assists CTM

Ref EASA Module 17A – Propeller/ Fundamentals

5. The thrust face of a propeller blade is the

A. Blade face or flat side

B. Root to which the gear segment is fitted

C. Blade back or curved side

Ref EASA Module 17A – Propeller/ Fundamentals

6. A left handed propeller is one that?

A. is fitted to an engine on the left side of the aircraft

B. rotates clockwise when viewed from the rear

C. rotates clockwise when viewed from the front

Ref EASA Module 17A – Propeller/ Fundamentals

7. Given that blade angle is measured from the centre of the hub, the blade angle at the blade shank is considered to be

- highest

- Master Station value

- lowest

Ref EASA Module 17A – Propeller/ Fundamentals

8. Thrust bending force on a propeller blade

- can be used in propeller design to reduce some operation stress

- tends to bend the propeller blade forward at the tip

- intensifies the centrifugal forces to some degrees

Ref EASA Module 17A – Propeller/ Fundamentals

9. A right hand propeller

- rotates clockwise when viewed from the front

- is always fitted to the starboard engine

- rotates clockwise when viewed from the rear

Ref EASA Module 17A – Propeller/ Fundamentals

10. The maximum airspeed of the aerofoil section of a propeller blade is at the

- root

- master station

- tip

Ref EASA Module 17A – Propeller/ Fundamentals

Sub-Module 17.2: Propeller Construction

1. What is the minimum longitudinal clearance?

A. 1 inch

B. 2 inches

C. 1/2 inch

Ref EASA Module 17A – Propeller/ Propeller Construction

2. The term 'Blade stations' refer to distances from the

A. blade root to blade tip

B. hub centre to blade tip

C. blade shank to blade tip

Ref EASA Module 17A – Propeller/ Propeller Construction

3. A reverse pitch propeller is

A. A propeller which varies from coarse to fine pitch automatically

B. Where the blade pitch has been increased - to increase the forward thrust

C. Where the blade pitch has been reduced to a negative pitch to reduce the aircraft's speed

Ref EASA Module 17A – Propeller/ Propeller Construction

4. The master element station is usually situated

A. at 0.7 blade span away from blade root

B. at 0.7 blade span away from centre of hub

A. at 0.7 blade span away from blade tip

Ref EASA Module 17A – Propeller/ Propeller Construction

5. Splined shafts on propeller installation are used normally on

A. large engines

B. lightweight engines

C. small engines

Ref EASA Module 17A – Propeller/ Propeller Construction

6. Propeller blade stations are measured from

- the root

- the master station

- the centre of the hub

Ref EASA Module 17A – Propeller/ Propeller Construction

7. What is the purpose of small holes at the tip of wooden propellers?

A. Drainage

B. Pivot points used during manufacture

C. Balancing

Ref EASA Module 17A – Propeller/ Propeller Construction

8. Nickel sheaths are used for

- aerodynamics

- erosion prevention

- strength

Ref EASA Module 17A – Propeller/ Propeller Construction

9. Metal at the tip and along the leading edge of a wooden propeller is

- for protection

- for balancing

- for Anti-icing

Ref EASA Module 17A – Propeller/ Propeller Construction

10. The variable pitch change mechanism is housed inside the

- propeller shaft

- propeller hub

- propeller blades

Ref EASA Module 17A – Propeller/ Propeller Construction

11. The blade face is the surface of the propeller blade that

- corresponds to the upper surface of an aerofoil or cambered side

- the airflow hits directly, which will be either the flat or curved side, depending on whether the propeller is windmilling or being driven by the engine

- corresponds to the lower surface of an aerofoil or flat side

Ref EASA Module 17A – Propeller/ Propeller Construction

12. If a flanged propeller shaft has dowel pins

- check carefully for front cone bottoming against the pins

- the propeller can be installed in only one position

- install the propeller so that the blades are positioned for hand propping

Ref EASA Module 17A – Propeller/ Propeller Construction

Sub-Module 17.3: Propeller Pitch Control

1. Reduction gearing allows the

A. Blade tips to rotate slower than the root of the propeller blade.

B. Blade tips to operate below the speed of sound.

C. Blade tips to operate above the speed of sound.

Ref EASA Module 17A – Propeller/ Propeller Pitch Control

2. Blade cuffs are fitted to the root of the blades

A. To increase flow of cooling air into the engine nacelle.

B. To increase thrust

C. To increase the strength of the blade

Ref EASA Module 17A – Propeller/ Propeller Pitch Control

3. The term spinner is applied to

A. A propeller tip vortex

B. An acrobatic manoeuvre

C. A streamline covering over the propeller hub

Ref EASA Module 17A – Propeller/ Propeller Pitch Control

4. The advantage of the beta range is it allows

A. high power settings with higher than normal pitch settings when in flight

B. low fine pitch settings with high power

C low power settings with higher than normal pitch setting for ground manoeuvres

Ref EASA Module 17A – Propeller/ Propeller Pitch Control

5. Counterweights are used to

A. balance the blades

B. counteract the CTM of the blades

C. counter the ATM of the blades

Ref EASA Module 17A – Propeller/ Propeller Pitch Control

6. In a propeller with counterweights, what is used to make it move to fine pitch?

A. Centrifugal force acting on the counterweight

B. ATM

C. Governor oil pressure

Ref EASA Module 17A – Propeller/ Propeller Pitch Control

7. If the speeder spring pressure of a CSU is increased the blade will

A. will not move

B. coarsen off

C. fine off

Ref EASA Module 17A – Propeller/ Propeller Pitch Control

8. The gear type pump in a C.S.U. or P.C.U

- assist the governor valve to move

- lubricates the entire propeller system

- boosts engine oil system pressure

Ref EASA Module 17A – Propeller/ Propeller Pitch Control

9. Possible blade angles are

- from max. reverse to feather position

- from max. reverse to take-off position

- from ground idle to feather position

Ref EASA Module 17A – Propeller/ Propeller Pitch Control

10. If governor flywheel overcomes the speeder spring, it indicates

- overspeed

- underspeed

- onspeed

Ref EASA Module 17A – Propeller/ Propeller Pitch Control

11. If underspeed occurs, the propeller blades move towards

- feathering

- coarse pitch

- fine pitch

Ref EASA Module 17A – Propeller/ Propeller Pitch Control

12. When the flyweights fly outwards in a PCU, this is known as

- overspeed

- underspeed

- onspeed

Ref EASA Module 17A – Propeller/ Propeller Pitch Control

Sub-Module 17.5: Propeller Ice Protection

1. Propeller de-icing for varying rates of icing can be varied by

A. increasing cyclic timing

B. increasing propeller RPM

C. increasing the AC or DC voltage

Ref EASA Module 17A – Propeller/ Propeller Ice Protection

2. Ice formation on the propellers, when the aircraft is in flight, will

A. decrease available engine power

B. decrease thrust and cause excessive vibration

C. increase aircraft stall speed and increase noise

Ref EASA Module 17A – Propeller/ Propeller Ice Protection

3. Propeller slip rings

A. conduct electrical power for de-icing

B. convey oil for PCU to pitch change mechanism

C. minimise slip

Ref EASA Module 17A – Propeller/ Propeller Ice Protection

4. On an electrical de-icing system, the brush block is used to transfer power to

A. the de-icing boot

B. the slip ring

C. the timer

Ref EASA Module 17A – Propeller/ Propeller Ice Protection

5. Propeller anti-icing may be achieved by

A. using commercial de-icing fluid sprayed on the propeller

B. using iso-propyl alcohol de-icing fluid sprayed on the blade

C. a using hot air from the compressor

Ref EASA Module 17A – Propeller/ Propeller Ice Protection

6. If an element in the propeller overshoe becomes open circuit

A. the overshoe is removed and element replaced

B. the element can be replaced in situ

C. the entire overshoe must be replaced

Ref EASA Module 17A – Propeller/ Propeller Ice Protection

7. Ice is removed from blades by

A. Rubber boots inflated in sequence using compressor hot air

B. Inboard and outboard boot sections heated in sequence by action of the timer

C. A continuously heated electrical boot

Ref EASA Module 17A – Propeller/ Propeller Ice Protection

8. Propeller blades are electrically de-iced by

- generating eddy currents in the blades

- electric heating elements wound around the blade root

- electric heating elements bound in rubber overshoes on the leading edge of the blades

Ref EASA Module 17A – Propeller/ Propeller Ice Protection

9. Blade dual heating elements are positioned

- inboard and outboard, but both near root

- upper and lower surfaces of blades

- one at the tip and one near the root, the centre blade portion is cleared by slipstreaming

Ref EASA Module 17A – Propeller/ Propeller Ice Protection

Sub-Module 17.6: Propeller Maintenance

1. Composite propellers may have minor repairs carried out by

A. Any approved 3rd party maintenance organization

B. The operator

C. Any approved composite repair facility

Ref EASA Module 17A – Propeller/ Propeller Maintenance

2. How is blending carried out on an aluminium propeller blade?

A. By using a scraper

B. By using a coarse file and peening

C. By using a needle file and finish with emery cloth

Ref EASA Module 17A – Propeller/ Propeller Maintenance

3. Propeller blade tracking is the process of determining

A. That the blade angles are within specified tolerance of each other

B. The plane of rotation of the propeller with respect to the aircraft longitudinal axis

C. The positions of the tips of the propeller blades relative to each other

Ref EASA Module 17A – Propeller/ Propeller Maintenance

4. Blade tracking is adjusted by

A. by fitting shims to the propeller shaft hub

B. adding lead wool to the blade shank

C. re-seating the propeller on the front and rear cones

Ref EASA Module 17A – Propeller/ Propeller Maintenance

5. Thrust and camber faces of a propeller should be blended out to

A. 5 times the depth of damage

B. 20 times to the depth of damage

C. 10 times to the depth of damage

Ref EASA Module 17A – Propeller/ Propeller Maintenance

6. Which of the following determines oil and grease specifications for lubrication of propellers?

A. Airframe manufacturers

B. Engine Manufacturers

C. Propeller manufacturers

Ref EASA Module 17A – Propeller/ Propeller Maintenance

7. The application of more protective coating on one blade than another when refinishing a wood propeller

A. has little or no effect on operating characteristics

B. should never be done

C. may be necessary to achieve final balancing

Ref EASA Module 17A – Propeller/ Propeller Maintenance

8. A metal propeller may be statically balanced by

A. removing metal from the opposite blade

B. adding or removing lead wool to the hollow blade roots

C. adding varnish to the lighter blade

Ref EASA Module 17A – Propeller/ Propeller Maintenance

9. Leading edges should be blended out to

A. 5 times the depth of damage

B. 30 times the depth of damage

C. 10 times the depth of damage

Ref EASA Module 17A – Propeller/ Propeller Maintenance

10. Which of the following is the most dangerous place to have a crack on a propeller?

- Trailing edge

- Leading edge

- Tip

Ref EASA Module 17A – Propeller/ Propeller Maintenance

11. Dynamic balance is confirmed by use of

- knife edges and mandrel

- a tracking check

- a vibration analyzer

Ref EASA Module 17A – Propeller/ Propeller Maintenance

12. When preparing a propeller blade for inspection it should be cleaned with

- steel wool

- methyl ethyl ketone

- mild soap and water

Ref EASA Module 17A – Propeller/ Propeller Maintenance

Sub-Module 17.7: Propeller Storage and Preservation

1. A wooden propeller should be stored in a

A. Room with high humidity to stop it drying and cracking

B. Dry and well ventilated area

C. Warm but light storeroom

Ref EASA Module 17A – Propeller/ Propeller Storage and Preservation

2. Short term temporary storage of a propeller is a period of up to

A.30 days

B.60 days

C.90 days

Ref EASA Module 17A – Propeller/ Propeller Storage and Preservation

3. If you find defects on the blades, spinner or backplate before it goes in the store, what do you do?

A. There may be no attention given to the components

B. Put it in store and will be seen and rectified on build-up

C. The defects must be rectified before the propeller is accepted in storage

Ref EASA Module 17A – Propeller/ Propeller Storage and Preservation

4. When storing wooden propellers

A. store in a dry well ventilated and illuminated location

B. wrap in greaseproof paper

C. store in a horizontal position

Ref EASA Module 17A – Propeller/ Propeller Storage and Preservation

5. The maximum storage periods for installed propellers are detailed in the

A. propeller overhaul and repair manual

B. Airworthiness Notices

C. Aircraft Maintenance Manual

Ref EASA Module 17A – Propeller/ Propeller Storage and Preservation

Total: 1074 questions